MICRO-STAR INTERNATIONAL CO., LTD.

7F, NO. 4, LANE 497, CHUNG CHENG RD.HSIN TIEN CITY, TAIPEI HSIEN, TAIWAN, R.O.C.TEL:  886-2-218-2292 (REP)FAX:  886-2-218-5552, 218-3361, 218-6049µ §

MS-4138  VL-BUS  GREEN  MAINBOARD

TEST  REPORT

Date:  August  5,  1994

MAINBOARD	MS-4138

CHIPSET	SiS  85C471

I.		SOFTWARE COMPATIBILITY TEST

A.  DOS

| MS-DOS | VER 5.0 | VER 6.0 | VER 6.2 | REMARKS |
| --- | --- | --- | --- | --- |
| INSTALL | OK | OK | OK |  |
| BOOT | OK | OK | OK |  |
| FORMAT A: | OK | OK | OK |  |
| BOOT FROM A | OK | OK | OK |  |
| COPY | OK | OK | OK |  |
| COMP | OK | OK | OK |  |
| XCOPY | OK | OK | OK |  |
| DISKCOPY | OK | OK | OK |  |
| DISKCOMP | OK | OK | OK |  |
| M.C.R. DIR | OK | OK | OK |  |
| EMM386 | OK | OK | OK |  |

B.  IBM  OS/2

Configuration:

·	Intel 80486DX4-100 CPU

·	32MB Memory (Panasonic MN41C4000SJ-08)

·	256KB Cache (Winbond W24257AK-15)

·	MS-3408 All-in-one Cirrus Logic 5428 VGA and UMC IDE Controller

·	Conner CP30254 (254MB) Hard Disk

·	One 1.44MB and one 1.2MB FDDs

·	Genius Mouse

| OS/2 | VER 2.0 | VER 2.1 | REMARKS |
| --- | --- | --- | --- |
| INSTALL | OK | OK |  |
| BOOT | OK | OK |  |
| FORMAT A: | OK | OK |  |
| COPY | OK | OK |  |
| COMP | OK | OK |  |
| XCOPY | OK | OK |  |
| DISKCOPY | OK | OK |  |
| DISKCOMP | OK | OK |  |
| M.C.R. DIR | OK | OK |  |
| TASK TEST | OK | OK |  |
| WIN-OS/2 | OK | OK |  |
| WIN PROGRAMS | OK | OK |  |

C.  SCO  UNIX

Configuration:

·	Intel 80486DX4-100 CPU

·	16MB Memory (Micron MT4C4001JDJ-7)

·	256KB Cache (Winbond W24257AK-20)

·	Adaptec AHA-1542C SCSI Controller

·	MS-3408 All-in-one Cirrus Logic 5428 VGA and UMC IDE Controller

·	Wangtek 5525ES Tape Drive

·	Maxtor LXT-340S (340MB) SCSI Hard Disk

·	One 1.44MB and one 1.2MB FDDs

·	Genius Mouse

| SCO UNIX | REL 3.2.4 | REMARKS |
| --- | --- | --- |
| INSTALL | OK |  |
| BOOT | OK |  |
| O.D.T. | OK |  |
| TASK TEST | OK |  |

D.  WINDOWS  N.T.

Configuration:

·	Intel 80486DX4-100 CPU

·	32MB Memory (Panasonic MN41C4000SJ-08)

·	256KB Cache (Winbond W24257AK-20)

·	Adaptec AHA-1542C SCSI Controller

·	MS-3408 All-in-one Cirrus Logic 5428 VGA and UMC IDE Controller

·	Maxtor LXT340SY (340MB) SCSI Hard Disk

·	NEC Multi-Spin 3XE CD-ROM

·	One 1.44MB and one 1.2MB FDDs

·	Logitech Mouse

| WINDOWS N.T. | VER 3.1 | REMARKS |
| --- | --- | --- |
| INSTALL | OK |  |
| BOOT | OK |  |
| FORMAT A: | OK |  |
| COMP | OK |  |
| XCOPY | OK |  |
| DISKCOPY | OK |  |
| DISKCOMP | OK |  |
| TASK TEST | OK |  |
| HCT | OK |  |

E.  NOVELL  NETWARE

Configuration:

·	Intel 80486DX2-66 CPU

·	16MB Memory (Micron MT4C4001JDJ-7)

·	256KB Cache (Winbond W24257AK-20)

·	Ultrastor U14F ISA SCSI Controller

·	MS-3408 All-in-one Cirrus Logic 5428 VGA and UMC IDE Controller

·	Quantum 240S (240MB) SCSI Hard Disk

·	Conner CP3044 (40MB) Hard Disk

| NETWARE | 386 V3.11 | REMARKS |
| --- | --- | --- |
| INSTALL | OK |  |
| BOOT | OK |  |
| 3COM | OK | File Server and Workstation |
| ARCNET | OK | File Server and Workstation |
| NE2000 | OK | File Server and Workstation |
| NE2100 | OK | File Server and Workstation |
| BOOT ROM | OK |  |
| FILE TRANSFER | OK |  |
| NWTEST | OK |  |

F.  PACKETS  AND  UTILITIES

| S O F T W A R E | REMARKS |
| --- | --- |
| AutoCAD R10 c10 | OK |
| AutoCAD R11 | OK |
| AutoCAD R12 | OK |
| Central Point Anti-Virus 1.4 | OK |
| Clipper 5.01 | OK |
| CorelDraw V3.0 | OK |
| DBase III Plus | OK |
| DBase 4 | OK |
| GeoWorks Ensemble V1.2 | OK |
| Harvard Graphics V2.12 | OK |
| Lotus 1-2-3 R2.4 | OK |
| Lotus 1-2-3 R3.1+ | OK |
| Microsoft Entertainment Pack Vol 1 | OK |
| Microsoft Entertainment Pack Vol 2 | OK |
| Microsoft Entertainment Pack Vol 3 | OK |
| Microsoft Excel V4.0 | OK |
| Microsoft PowerPoint for Windows V3.0 | OK |
| Microsoft Video for Windows | OK |
| Microsoft Visual Basic V2.0 | OK |
| Microsoft Windows 3.1 | OK |
| Microsoft Word for Windows V2.0 | OK |
| Microsoft Works for Windows V2.0 | OK |
| Norton Desktop for Windows V1.2 | OK |
| Norton Utilities V6.0 / V7.0 | OK |
| PC Tools V8.0 | OK |
| PE 2 | OK |
| QEMM 386 V5.11 | OK |
| QEMM 386 V6.0 | OK |
| QEMM 386 V7.01 | OK |
| QEMM 386 V7.02 | OK |
| Symantec Entertainment Pack for Windows | OK |
| TopDOS 3.00 | OK |
| Ventura Publisher V2.0 | OK |
| WordPerfect 5.1 | OK |
| XTree Gold V2.51 | OK |

G.  DIAGNOSTIC  PROGRAMS

| S O F T W A R E | REMARKS |
| --- | --- |
| '92 System Mark | OK |
| '93 System Mark | OK |
| Byte Benchmark V1.3 | OK |
| Byte Benchmark V2.2 | OK |
| CheckIt V3.0 | OK |
| CheckIt Pro | OK |
| Core Test V2.92 | OK |
| IBM Diagnostics V2.03 | OK |
| Landmark Speed Test V1.14 | OK |
| Landmark Speed Test V2.0 | OK |
| Microsoft Software Diagnostics (MSD) | OK |
| Norton Diagnostics V7.0 | OK |
| Norton System Information V 4.5 | OK |
| Norton System Information V7.0 | OK |
| PC Magazine Labs Benchmark 6.0 | OK |
| PC Magazine Labs Benchmark 7.0 | OK |
| PC Week Labs System Benchmarks V1.10 | OK |
| Power Meter V1.7 | OK |
| Power Meter V1.81 | OK |
| QA Plus V4.61 | OK |
| QA Plus V4.70 | OK |
| QA Plus/FE V5.03 | OK |
| QA Plus/FE V5.04 | OK |
| QA Plus/FE V5.12 | OK |
| Speedy | OK |
| Superscale 3D Benchmark | OK |
| WinBench V2.5 | OK |
| WinBench V3.11 | OK |
| WinBench V4.0 | OK |
| WindSock 3.1 | OK |
| WinStone 94 | OK |
| WinTach V1.0 | OK |
| WinTach V1.2 | OK |

II.		HARDWARE COMPATIBILITY TEST

A.	VIDEO  CARDS

| MANUFACTURER & MODEL NO. | CHIPSET | REMARKS |
| --- | --- | --- |
| ACUMOS | AVGA2 | ISA |
| IIT | AGX-014 | ISA |
| OAK | OTI-067 | ISA |
| OAK | OTI-077 | ISA |
| S3 | S3801 | ISA |
| TRIDENT | TVGA8900C | ISA |
| TRIDENT | TVGA8900CL | ISA |
| TRIDENT | TVGA8900D | ISA |
| TRIDENT | TVGA9000B | ISA |
| TSENG LABS | ET4000/W32 | ISA |
| WD | PARADISE | ISA |
| WD | WD90C30 | ISA |
| ATI | MACH 32 | VESA |
| CHAINTECH | ALI ALG2228 | VESA |
| DIAMOND SPEEDSTAR REV A2 | CIRRUS LOGIC 5426 | VESA |
| DIAMOND STEALTH PRO REV. 3.1 | S3928 | VESA |
| DIAMOND VIPER REV. B1 | WEITEK P9000 | VESA |
| MICRO-STAR MS-3405 | CIRRUS LOGIC 5426 | VESA |
| MICRO-STAR MS-3409 | CIRRUS LOGIC 5428 | VESA |
| ORCHID FAHRENHEIT VLB | S3805 | VESA |
| S3 | S3805 | VESA |
| TSENG LABS | ET4000/W32 | VESA |

B.	I/O  CONTROLLERS

| MANUFACTURER & MODEL NO. | CHIPSET | REMARKS |
| --- | --- | --- |
| GOLDSTAR | PRIME 2 | ISA |
| MICRO-STAR MS-3806 | SMC FDC37C665 (666) | ISA |
| MICRO-STAR MS-3807 | WINBOND W83757 / 58 | ISA |
| UMC | UM82C863F | ISA |
| DTC DTC2278VL |  | VESA |
| MICRO-STAR MS-3802 | PROMISE PDC20230B | VESA |
| HORNET VL-200 | ADAPTEC AIC-5VL01 | VESA W/ CACHE |
| PROMISE DC4030VL-2 | PROMISE PDC20130A | VESA W/ CACHE |
| TEKRAM DC-680C |  | VESA W/ CACHE |

C.	SCSI CONTROLLERS

| MANUFACTURER | MODEL NO. | REMARKS |
| --- | --- | --- |
| ADAPTEC | AHA-1542C | ISA |
| ADAPTEC | AHA-1542CF | ISA |
| ULTRASTOR | U14F | ISA |
| ULTRASTOR | U34F | VESA |

D.	NETWORK  ADAPTERS

| MANUFACTURER | TYPE | CHIPSET | REMARKS |
| --- | --- | --- | --- |
| 3COM | 3COM | 3C503 | ISA |
| 3COM | 3COM | 3C509 | ISA |
| SMC | ARCNET | COM90C65LJ | ISA |
|  | NE-2000 |  | ISA |
|  | NE-2100 |  | ISA |

E.	FLOPPY  DISK  DRIVES

| MANUFACTURER | MODEL NUMBER | CAPACITY |
| --- | --- | --- |
| MITSUBISHI | MF504C-315MG | 1.2 MB |
| MITSUBISHI | MF355C-255MG | 1.44 MB |
| TEAC | FD235HF | 1.44 MB |

F.	HARD  DISK  DRIVES

| MANUFACTURER | MODEL NUMBER | CAPACITY | REMARKS |
| --- | --- | --- | --- |
| CONNER | CP3104 | 104 MB | IDE |
| CONNER | CP3204F | 204 MB | IDE |
| CONNER | CP30254 | 254 MB | IDE |
| CONNER | CFS420A | 420 MB | IDE |
| CONNER | CFA540A | 540 MB | IDE |
| QUANTUM | 240AT | 240 MB | IDE |
| SEAGATE | ST3243A | 214 MB | IDE |
| WD | CAVIAR 2250 | 250 MB | IDE |
| WD | CAVIAR 2420 | 420 MB | IDE |
| WD | CAVIAR 2540 | 540 MB | IDE |
| MAXTOR | 7120S | 120 MB | SCSI |
| MAXTOR | 4170S | 170 MB | SCSI |
| MAXTOR | LXT340SY | 340 MB | SCSI |
| QUANTUM | ELS 170S | 170 MB | SCSI |
| QUANTUM | ELS 240S | 240 MB | SCSI |

G.	MEMORY  MODULES

| MANUFACTURER | MODEL NUMBER | TYPE | SIZE |
| --- | --- | --- | --- |
| MITSUBISHI | M5M44260AJ-8 | 72-pin single-sided | 4 MB |
| PANASONIC | MN41C41000SJ-8 | 72-pin single-sided | 4 MB |
| PANASONIC | MN414400ASJ-7 | 72-pin single-sided | 4 MB |
| SAMSUNG | KM416C256AJ-7 | 72-pin single-sided | 4 MB |
| MICRON TECH | MT4C4001JDJ-7 | 72-pin double-sided | 8 MB |
| NEC | D424100GS-70L-9JD | 72-pin double-sided | 32 MB |

H.	STATIC  RAMs

| MANUFACTURER | MODEL NUMBER | SIZE |
| --- | --- | --- |
| WINBOND | W24257AK-15 | 32K * 8 |

I.	MISCELLANEOUS  PERIPHERAL  DEVICES

| TYPE | MANUFACTURER | MODEL NUMBER | REMARKS |
| --- | --- | --- | --- |
| AUDIO CARD | CREATIVE LABS | SOUND BLASTER PRO |  |
| AUDIO CARD | OAK | MOZART |  |
| CD-ROM DRIVE | CREATIVE TECH. | CR-521-C |  |
| CD-ROM DRIVE | NEC | MULTI-SPIN 3XE |  |
| TAPE DRIVE | IRWIN BACKUP | ACCUTRAK PLUS + | 250 MB |
| TAPE DRIVE | QTRONIX | SDX-85 | 150 MB |
| TAPE DRIVE | WANGTEK | 5525ES | 525 MB |
| VIDEO CARD | CREATIVE LABS | VIDEO BLASTER |  |

III.	PERFORMANCE  TEST

Testing  Configuration:

Mainboard		:	MS-4138 Ver 1

System BIOS	:	AMI A75C

System Chipset:	SiS 85C471

VGA & I/O Card	:	MS-3408 All-in-one Cirrus Logic 5428 VGA and UMC IDE Controller

System Memory	:	8 MB (MS-4904 Micron Tech MT4C4001JDJ-7)

Cache Memory		:	256 KB (Winbond W24257AK-15)

Hard Disk drive	:	Conner CP30254 HDD (240 MB)

PC Magazine Labs BenchMark 7.0 and 8.0 were tested with the ff. AUTOEXEC.BAT and CONFIG.SYS:

AUTOEXEC.BAT:

PROMPT $P$G

PATH=C:\;C:\DOS

GMOUSE

CONFIG.SYS:

FILES=20

|  | INTEL | INTEL/AMD | INTEL |
| --- | --- | --- | --- |
|  | DX4-100 | DX2-66 | DX-50 |
| PC BENCHMARK 7.0 |  |  |  |
| DOSMark | 46.54 | 41.30 | 37.74 |
| Processor Harmonic | 24133.16 | 16142.06 | 13324.66 |
| Video Harmonic | 9052.89 | 8948.96 | 10230.63 |
| Memory Harmonic | 16458.92 | 15243.13 | 11969.79 |
| Disk Harmonic | 37.10 | 34.71 | 33.12 |
| PC BENCHMARK 8.0 |  |  |  |
| DOSMark | 406.27 | 375.31 | 346.62 |
| Processor Score | 61.22 | 41.58 | 35.09 |
| Video Score | 4717.17 | 4600.15 | 5831.20 |
| Disk Score | 236.00 | 257.00 | 248.00 |

|  | CYRIX | CYRIX | INTEL |
| --- | --- | --- | --- |
|  | DX2-50 | DX-40 | DX-33 |
| PC BENCHMARK 7.0 |  |  |  |
| DOSMark | 35.93 | 34.14 | 35.13 |
| Processor Harmonic | 11658.80 | 9669.86 | 9077.65 |
| Video Harmonic | 7369.69 | 9410.35 | 8919.12 |
| Memory Harmonic | 10223.33 | 9917.93 | 9522.24 |
| Disk Harmonic | 32.87 | 32.59 | 35.20 |
| PC BENCHMARK 8.0 |  |  |  |
| DOSMark | 299.20 | 261.27 | 281.04 |
| Processor Score | 29.38 | 21.37 | 24.41 |
| Video Score | 3643.83 | 5059.08 | 4570.34 |
| Disk Score | 225.00 | 228.00 | 232.00 |

All the Diagnostic Programs below were tested with the ff. AUTOEXEC.BAT and CONFIG.SYS:

AUTOEXEC.BAT:

PROMPT $P$G

PATH=C:\;C:\DOS

GMOUSE

CONFIG.SYS:

FILES=20

BUFFERS=20

|  | INTEL | INTEL/AMD | INTEL |
| --- | --- | --- | --- |
|  | DX4-100 | DX2-66 | DX-50 |
| BYTE BENCHMARK 1.3 |  |  |  |
| CPU Index | 23.90 | 15.87 | 11.99 |
| FPU Index | 88.51 | 59.81 | 45.64 |
| Disk Index | 24.59 | 23.28 | 19.78 |
| Video Index | 17.01 | 14.34 | 12.64 |
| BYTE BENCHMARK 2.2 |  |  |  |
| CPU Index | 2.85 | 2.21 | 1.73 |
| FPU Index | 6.89 | 4.59 | 3.46 |
| Disk Index | 1.20 | 1.06 | 1.25 |
| Video Index | 5.14 | 3.82 | 3.20 |

|  | CYRIX | CYRIX | INTEL |
| --- | --- | --- | --- |
|  | DX2-50 | DX-40 | DX-33 |
| BYTE BENCHMARK 1.3 |  |  |  |
| CPU Index | 11.42 | 10.01 | 9.48 |
| FPU Index | 41.05 | 33.91 | 29.38 |
| Disk Index | 20.37 | 19.10 | 20.55 |
| Video Index | 10.12 | 9.66 | 9.69 |
| BYTE BENCHMARK 2.2 |  |  |  |
| CPU Index | 1.58 | 1.43 | 1.40 |
| FPU Index | 3.70 | 2.96 | 2.30 |
| Disk Index | 1.15 | 1.24 | 1.20 |
| Video Index | 2.68 | 2.35 | 2.20 |

|  | INTEL | INTEL/AMD | INTEL |
| --- | --- | --- | --- |
|  | DX4-100 | DX2-66 | DX-50 |
| NORTON SI 4.5 |  |  |  |
| Computing Index | 198.5 | 104.5 | 78.7 |
| Disk Index | 4.4 | 4.8 | 4.7 |
| Performance Index | 133.8 | 71.2 | 54.0 |
| NORTON SI 7.0 |  |  |  |
| Computing Index | 197.1 | 143.4 | 108.0 |
| Disk Index | 7.1 | 7.5 | 7.5 |
| Performance Index | 133.5 | 98.1 | 74.4 |
| LANDMARK SPEED 1.14 |  |  |  |
| CPU Speed | 200+ | 200+ | 200+ |
| LANDMARK SPEED 2.0 |  |  |  |
| CPU Speed | 361.64 | 222.01 | 167.27 |
| FPU Speed | 814.82 | 543.19 | 408.86 |
| Video Speed | 10802.00 | 10802.00 | 14043.00 |

|  | CYRIX | CYRIX | INTEL |
| --- | --- | --- | --- |
|  | DX2-50 | DX-40 | DX-33 |
| NORTON SI 4.5 |  |  |  |
| Computing Index | 109.8 | 88.1 | 51.1 |
| Disk Index | 4.3 | 4.6 | 4.4 |
| Performance Index | 74.6 | 60.2 | 35.5 |
| NORTON SI 7.0 |  |  |  |
| Computing Index | 84.9 | 67.9 | 71.7 |
| Disk Index | 7.1 | 7.3 | 7.1 |
| Performance Index | 58.7 | 47.6 | 50.0 |
| LANDMARK SPEED 1.14 |  |  |  |
| CPU Speed | 151.9 | 121.4 | 151.3 |
| LANDMARK SPEED 2.0 |  |  |  |
| CPU Speed | 164.92 | 131.94 | 111.00 |
| FPU Speed | 444.31 | 355.45 | 271.31 |
| Video Speed | 8330.00 | 12136.00 | 10802.00 |

|  | INTEL | INTEL/AMD | INTEL |  |
| --- | --- | --- | --- | --- |
| CHECKIT 3.0 | DX4-100 | DX2-66 | DX-50 | UNITS |
| MAIN SYSTEM |  |  |  |  |
| CPU Speed | 47474 | 33609 | 24348 | Dhrystones |
| Math Speed | 16818.6 | 11445.2 | 8378.2 | M-Whetstones |
| VIDEO SYSTEM |  |  |  |  |
| BIOS Video Speed | 25566 | 26276 | 22088 | Char/S |
| Direct Video Speed | 315948 | 327484 | 303131 | Char/S |
| HARD DISK |  |  |  |  |
| Transfer Speed | 554.3 | 674.6 | 640.9 | K/Second |
| Average Seek Time | 13.6 | 13.6 | 13.0 | ms |
| Track Seek Time | 5.2 | 5.9 | 5.3 | ms |

|  | CYRIX | CYRIX | INTEL |  |
| --- | --- | --- | --- | --- |
| CHECKIT 3.0 | DX2-50 | DX-40 | DX-33 | UNITS |
| MAIN SYSTEM |  |  |  |  |
| CPU Speed | 23164 | 18415 | 16170 | Dhrystones |
| Math Speed | 9312.6 | 7379.2 | 5703.7 | M-Whetstones |
| VIDEO SYSTEM |  |  |  |  |
| BIOS Video Speed | 17573 | 17573 | 20013 | Char/S |
| Direct Video Speed | 289672 | 243530 | 210205 | Char/S |
| HARD DISK |  |  |  |  |
| Transfer Speed | 554.3 | 625.2 | 554.3 | K/Second |
| Average Seek Time | 13.3 | 13.3 | 13.3 | ms |
| Track Seek Time | 5.3 | 4.7 | 4.7 | ms |

|  | INTEL | INTEL/AMD | INTEL |  |
| --- | --- | --- | --- | --- |
| POWER METER 1.7 | DX4-100 | DX2-66 | DX-50 | UNITS |
| SYSTEM TEST |  |  |  |  |
| Agg. Hardware | 2730.6 | 1903.9 | 1510.8 | PMUs |
| Spreadsheet | 5347.6 | 4957.2 | 7344.2 | PMUs |
| Word Processing | 4276.7 | 5265.4 | 4669.8 | PMUs |
| Database App'n. | 2780.9 | 3632.4 | 3379.0 | PMUs |
| CAE / CAD | 66472.9 | 45953.1 | 36439.2 | PMUs |
| Throughput Test | 17807.1 | 14831.4 | 9722.0 | PMUs |
| CPU MEMORY |  |  |  |  |
| Agg. CPU/Memory | 40405.7 | 26757.0 | 19995.7 | PMUs |
| Clock Speed Test | 96.5 | 65.0 | 49.0 | MHertz |
| MIPS Test | 40.6 | 29.3 | 22.1 | MIPS |
| Dhrystone Test | 52.0 | 37.6 | 28.4 | K-Dhrystones/S |
| Whetstone Test | 9894.2 | 6531.5 | 4898.6 | K-Whetstones/S |
| Sieve Test | n.a. | 0.1 | 0.1 | Seconds |
| DISK DRIVE |  |  |  |  |
| Agg. Disk Test | 190.0 | 225.2 | 222.0 | PMUs |
| Trk-to-Trk Seek | 3.7 | 3.9 | 3.8 | MS/Seek |
| Mean Seek Test | 10.0 | 10.1 | 10.0 | MS/Seek |
| Data Transfer Rate | 1005.0 | 1252.2 | 1222.0 | Kb/Sec. |
| File Open/Close | 6.3 | 6.3 | 6.3 | MS/Op. |
| Seq File Rd/Wrt | 0.1 | 0.1 | 0.1 | MS/Op. |
| Ran File Rd/Wrt | 13.1 | 13.0 | 13.5 | MS/Op. |
| VIDEO DISPLAY |  |  |  |  |
| Agg Video Test | 7494.8 | 8030.2 | 7526.2 | PMUs Md 3 |
| Char Write Test | 0.5 | 0.5 | 0.5 | Sec. Md 3 |
| Line Drawing Test | 2.2 | 2.2 | 2.4 | sec. Md 18 |
| Fill Test | 11.4 | 10.7 | 11.6 | Sec. Md 18 |

|  | CYRIX | CYRIX | INTEL |  |
| --- | --- | --- | --- | --- |
| POWER METER 1.7 | DX2-50 | DX-40 | DX-33 | UNITS |
| SYSTEM TEST |  |  |  |  |
| Agg. Hardware | 1537.0 | 1302.9 | 1100.2 | PMUs |
| Spreadsheet | 3498.3 | 3398.2 | 3921.3 | PMUs |
| Word Processing | 4102.9 | 3229.5 | 3759.5 | PMUs |
| Database App'n. | 1350.5 | 1930.2 | 2391.8 | PMUs |
| CAE / CAD | 39054.1 | 32080.1 | 24439.8 | PMUs |
| Throughput Test | 12720.8 | 9018.2 | 6898.0 | PMUs |
| CPU MEMORY |  |  |  |  |
| Agg. CPU/Memory | 22025.7 | 17554.0 | 13252.7 | PMUs |
| Clock Speed Test | 48.5 | 39.0 | 32.5 | MHertz |
| MIPS Test | 19.7 | 15.8 | 14.8 | MIPS |
| Dhrystone Test | 25.2 | 20.3 | 18.9 | K-Dhrystones/S |
| Whetstone Test | 5582.8 | 4481.2 | 3255.1 | K-Whetstones/S |
| Sieve Test | 0.1 | 0.2 | 0.1 | Seconds |
| DISK DRIVE |  |  |  |  |
| Agg. Disk Test | 191.5 | 222.9 | 190.2 | PMUs |
| Trk-to-Trk Seek | 3.6 | 3.6 | 3.7 | MS/Seek |
| Mean Seek Test | 10.0 | 10.0 | 10.0 | MS/Seek |
| Data Transfer Rate | 1005.0 | 1210.4 | 1005.0 | Kb/Sec. |
| File Open/Close | 6.3 | 6.3 | 6.3 | MS/Op. |
| Seq File Rd/Wrt | 0.1 | 0.2 | 0.2 | MS/Op. |
| Ran File Rd/Wrt | 13.5 | 13.8 | 13.1 | MS/Op. |
| VIDEO DISPLAY |  |  |  |  |
| Agg Video Test | 5765.3 | 6401.3 | 7589.7 | PMUs Md 3 |
| Char Write Test | 0.6 | 0.6 | 0.5 | Sec. Md 3 |
| Line Drawing Test | 3.4 | 3.3 | 2.7 | sec. Md 18 |
| Fill Test | 15.8 | 15.0 | 12.0 | Sec. Md 18 |

|  | INTEL | INTEL/AMD | INTEL |  |
| --- | --- | --- | --- | --- |
| POWER METER 1.81 | DX4-100 | DX2-66 | DX-50 | UNITS |
| SYSTEM TEST |  |  |  |  |
| Agg. Hardware | 2652.3 | 1876.5 | 1477.6 | PMUs |
| Spreadsheet | 5602.6 | 2229.6 | 4612.2 | PMUs |
| Word Processing | 5720.1 | 3833.4 | 5980.4 | PMUs |
| Database App'n. | 3297.0 | 2641.8 | 3305.1 | PMUs |
| CAE / CAD | 67297.5 | 45010.7 | 37157.5 | PMUs |
| Throughput Test | 18737.4 | 14604.2 | 13360.8 | PMUs |
| CPU MEMORY |  |  |  |  |
| Agg. CPU/Memory | 39257.0 | 25769.7 | 19440.3 | PMUs |
| Clock Speed Test | 97.0 | 66.0 | 50.0 | MHertz |
| MIPS Test | 29.9 | 21.2 | 16.3 | MIPS |
| Dhrystone Test | 53.2 | 37.3 | 28.6 | K-Dhrystones/S |
| Whetstone Test | 9894.2 | 6489.0 | 4898.6 | K-Whetstones/S |
| Sieve Test | n.a. | 0.1 | 0.1 | Seconds |
| DISK DRIVE |  |  |  |  |
| Agg. Disk Test | 188.5 | 223.6 | 221.9 | PMUs |
| Trk-to-Trk Seek | 4.0 | 3.9 | 3.8 | MS/Seek |
| Mean Seek Test | 10.1 | 10.2 | 10.0 | MS/Seek |
| Data Transfer Rate | 1012.1 | 1263.2 | 1233.9 | Kb/Sec. |
| File Open/Close | 6.2 | 6.2 | 6.2 | MS/Op. |
| Seq File Rd/Wrt | 0.1 | 0.1 | 0.1 | MS/Op. |
| Ran File Rd/Wrt | 13.2 | 15.1 | 12.9 | MS/Op. |
| VIDEO DISPLAY |  |  |  |  |
| Agg Video Test | 7494.8 | 8030.2 | 7557.8 | PMUs Md 3 |
| Char Write Test | 0.5 | 0.5 | 0.5 | Sec. Md 3 |
| Line Drawing Test | 2.2 | 2.2 | 2.4 | sec. Md 18 |
| Fill Test | 11.3 | 10.7 | 11.6 | Sec. Md 18 |

|  | CYRIX | CYRIX | INTEL |  |
| --- | --- | --- | --- | --- |
| POWER METER 1.81 | DX2-50 | DX-40 | DX-33 | UNITS |
| SYSTEM TEST |  |  |  |  |
| Agg. Hardware | 1494.4 | 1262.0 | 1078.8 | PMUs |
| Spreadsheet | 3825.4 | 3857.7 | 3779.5 | PMUs |
| Word Processing | 4450.2 | 4604.5 | 4854.2 | PMUs |
| Database App'n. | 2454.6 | 2946.4 | 3050.6 | PMUs |
| CAE / CAD | 38270.1 | 31526.6 | 24376.1 | PMUs |
| Throughput Test | 12114.8 | 10136.3 | 8332.7 | PMUs |
| CPU MEMORY |  |  |  |  |
| Agg. CPU/Memory | 20937.0 | 16854.7 | 12886.3 | PMUs |
| Clock Speed Test | 48.7 | 39.2 | 33.0 | MHertz |
| MIPS Test | 14.4 | 11.6 | 10.8 | MIPS |
| Dhrystone Test | 25.4 | 20.3 | 18.9 | K-Dhrystones/S |
| Whetstone Test | 5582.8 | 4461.2 | 3244.5 | K-Whetstones/S |
| Sieve Test | 0.1 | 0.2 | 0.1 | Seconds |
| DISK DRIVE |  |  |  |  |
| Agg. Disk Test | 189.2 | 222.1 | 189.6 | PMUs |
| Trk-to-Trk Seek | 3.7 | 3.6 | 3.8 | MS/Seek |
| Mean Seek Test | 10.0 | 10.0 | 10.0 | MS/Seek |
| Data Transfer Rate | 1011.0 | 1220.6 | 1012.1 | Kb/Sec. |
| File Open/Close | 6.2 | 6.2 | 6.2 | MS/Op. |
| Seq File Rd/Wrt | 0.2 | 0.2 | 0.2 | MS/Op. |
| Ran File Rd/Wrt | 14.7 | 12.9 | 13.2 | MS/Op. |
| VIDEO DISPLAY |  |  |  |  |
| Agg Video Test | 5765.3 | 6378.6 | 7621.9 | PMUs Md 3 |
| Char Write Test | 0.6 | 0.6 | 0.5 | Sec. Md 3 |
| Line Drawing Test | 3.4 | 3.3 | 2.7 | sec. Md 18 |
| Fill Test | 15.8 | 15.0 | 12.0 | Sec. Md 18 |

|  | INTEL | INTEL/AMD | INTEL |  |
| --- | --- | --- | --- | --- |
| QA PLUS 4.61 | DX4-100 | DX2-66 | DX-50 | UNITS |
| SYSTEM PERF. |  |  |  |  |
| CPU Speed | 45517 | 45517 | 45517 | Dhrystones |
| Direct Video Speed | 1136085 | 757390 | 473369 | CPS |
| Math Speed | 16570.8 | 11287.3 | 8525.2 | K-Whetstones |
| DISK PERF. |  |  |  |  |
| Trk-Trk Seek | 4.56 | 4.85 | 4.61 | ms |
| Average Seek | 11.4 | 11.8 | 11.3 | ms |
| Transfer Rate | 528.5 | 712.3 | 682.7 | K/sec |

|  | CYRIX | CYRIX | INTEL |  |
| --- | --- | --- | --- | --- |
| QA PLUS 4.61 | DX2-50 | DX-40 | DX-33 | UNITS |
| SYSTEM PERF. |  |  |  |  |
| CPU Speed | 22758 | 22758 | 22758 | Dhrystones |
| Direct Video Speed | 473369 | 473369 | 284021 | CPS |
| Math Speed | 9167.5 | 7372.6 | 5683.5 | K-Whetstones |
| DISK PERF. |  |  |  |  |
| Trk-Trk Seek | 4.57 | 4.35 | 4.49 | ms |
| Average Seek | 11.1 | 11.1 | 11.2 | ms |
| Transfer Rate | 260.1 | 442.8 | 282.5 | K/sec |

|  | INTEL | INTEL/AMD | INTEL |  |
| --- | --- | --- | --- | --- |
| QA PLUS 4.70 | DX4-100 | DX2-66 | DX-50 | UNITS |
| SYSTEM PERF. |  |  |  |  |
| CPU Speed | 68.3 | 45.5 | 45.5 | K-Dhrystones |
| Direct Video Speed | 1.1M | 757.4K | 473.4K | CPS |
| Math Speed | 16.7 | 10.6 | 8.6 | M-Whetstones |
| DISK PERF. |  |  |  |  |
| Trk-Trk Seek | 4.58 | 4.83 | 4.61 | ms |
| Average Seek | 11.3 | 12.0 | 11.4 | ms |
| Transfer Rate | 420.1 | 744.7 | 455.1 | K/sec |

|  | CYRIX | CYRIX | INTEL |  |
| --- | --- | --- | --- | --- |
| QA PLUS 4.70 | DX2-50 | DX-40 | DX-33 | UNITS |
| SYSTEM PERF. |  |  |  |  |
| CPU Speed | 22.8 | 22.8 | 22.8 | K-Dhrystones |
| Direct Video Speed | 473.4 | 473.4 | 284.0 | K-CPS |
| Math Speed | 8.7 | 7.1 | 5.7 | M-Whetstones |
| DISK PERF. |  |  |  |  |
| Trk-Trk Seek | 4.57 | 4.35 | 4.45 | ms |
| Average Seek | 11.2 | 11.1 | 11.1 | ms |
| Transfer Rate | 356.2 | 468.1 | 252.1 | K/sec |

The following programs below were tested under Windows 3.1 with the ff. AUTOEXEC.BAT and CONFIG.SYS:

AUTOEXEC.BAT:

PROMPT $P$G

PATH=C:\;C:\DOS

C:\WINDOWS\SMARTDRV.EXE

GMOUSE

CONFIG.SYS:

C:\WINDOWS\HIMEM.SYS

FILES=20

BUFFERS=20

Tested using Cirrus Logic GD-5428 Ver. 1.32B Windows driver, 1024 x 768 x 256C resolution.

|  | INTEL | INTEL/AMD | INTEL |  |
| --- | --- | --- | --- | --- |
| WINDOWS TEST | DX4-100 | DX2-66 | DX-50 | UNITS |
| SPEEDY | 38.37 | 29.17 | 27.20 | SPEEDMARK |
| WINTACH 1.2 | 52.34 | 37.44 | 33.58 | RPM |
| WINBENCH 3.11 | 26.62 | 23.17 | 21.82 | WINMARK |
| WINBENCH 4.0 | 11.8 | 8.37 | 7.72 | WINMARK |

|  | CYRIX | CYRIX | INTEL |  |
| --- | --- | --- | --- | --- |
| WINDOWS TEST | DX2-50 | DX-40 | DX-33 | UNITS |
| SPEEDY | 22.66 | 20.56 | 19.48 | SPEEDMARK |
| WINTACH 1.2 | 28.86 | 25.83 | 25.25 | RPM |
| WINBENCH 3.11 | 18.97 | 17.49 | 17.02 | WINMARK |
| WINBENCH 4.0 | 6.67 | 6.09 | 5.76 | WINMARK |

|  | INTEL | INTEL/AMD | INTEL |
| --- | --- | --- | --- |
| WINDSOCK | DX4-100 | DX2-66 | DX-50 |
| CPU | 857 | 500 | 374 |
| VIDEO | SU ¾ 715 | SU ¾ 496 | GR ¾ 425 |
| DISK | GR ¾ 449 | GR ¾ 308 | GR ¾ 288 |
| MEMORY | GO ¾ 202 | GO ¾ 198 | GO ¾ 156 |
| WINDOWS (OVERALL) | SU ¾ 611 | GR ¾ 396 | GR ¾ 323 |

|  | CYRIX | CYRIX | INTEL |
| --- | --- | --- | --- |
| WINDSOCK | DX2-50 | DX-40 | DX-33 |
| CPU | 452 | 361 | 248 |
| VIDEO | GR ¾ 336 | GR ¾ 299 | GR ¾ 289 |
| DISK | GR ¾ 240 | GR ¾ 216 | GR ¾ 218 |
| MEMORY | GO ¾ 128 | GO ¾ 137 | GO ¾ 184 |
| WINDOWS (OVERALL) | GR ¾ 324 | GR ¾ 276 | GR ¾ 233 |

Note:

SU	=	Superb

GR	=	Great

GO 	=	Good