MICRO-STAR INTERNATIONAL CO., LTD.

7F, NO. 4, LANE 497, CHUNG CHENG RD.HSIN TIEN CITY, TAIPEI HSIEN, TAIWAN, R.O.C.TEL:  886-2-218-2292 (REP)FAX:  886-2-218-5552, 218-3361, 218-6049µ §

MS-4143  ALI  486  PCI / ISA  MAINBOARD

PRELIMINARY  TEST  REPORT

Date:  November  9,  1994

MAINBOARD	MS-4143

SYS. CHIPSET	ALI M1439 + M1445 + On-chip 					IDE controller

ON-BOARD I/O	Winbond W83787F / W83758F

I.		HARDWARE COMPATIBILITY TEST

A.		SCSI CONTROLLERS

| MANUFACTURER | MODEL NO. | REMARKS |
| --- | --- | --- |
| ADAPTEC | AHA-1542C | ISA |
| ADAPTEC | AHA-1542CF | ISA |
| ADAPTEC | AHA-2940 | PCI |
| ADAPTEC | AHA-7850 | PCI |
| BUSLOGIC | BT-946C | PCI |
| MICRO-STAR MS-3805 | NCR | PCI |

B.		VGA  CARDS

Tested using Intel DX2-66, Windows 3.1 and WinBench 4.0.

| MANUFACTURER & MODEL NO. | CHIPSET | WINBENCH 4.0 1024x768x256C |
| --- | --- | --- |
| ARK LOGIC | ARK1000PV | 11.3 |
| ATI MACH 32 (1MB) | MACH 32 | 6.21 |
| ATI MACH 64 (2MB) | MACH 64 | 15.9 |
| DIAMOND SPEEDSTAR 64 (1MB) | CIRRUS LOGIC 5434 | 8.75 |
| DIAMOND STEALTH 32 (2MB) | ET4000/W32P Ver A | 12.4 |
| DIAMOND VIPER PCI (2MB) | WEITEK P9000/P9001 | 11.3 |
| ELSA WINNER 1000PRO (2MB) | S3 864 | 8.33 |
| GENOA PHANTOM 32I (1MB) | ET4000/W32P Ver A | 9.81 |
| INTEL V7 MERCURY LITE (1MB) | S3 928 | 8.62 |
| MATROX MGA (2MB) | MATROX MGA | 13.0 |
| MICRO-STAR MS-4402 (1MB) | ALI ALG2301 | 9.17 |
| MICRO-STAR MS-4403 (1MB) | IIT AGX-016 | 12.6 |
| MICRO-STAR MS-4405 (1MB) | ET4000/W32P Ver C | 10.7 |
| PROLINK (2MB) | S3 928 | 9.11 |
| S3 (2MB) | S3 764 | 11.6 |

C.		NETWORK  ADAPTERS

| MANUFACTURER | TYPE | CHIPSET | REMARKS |
| --- | --- | --- | --- |
| 3COM | 3COM | 3C503 | ISA |
| 3COM | 3COM | 3C509 | ISA |
| SMC | ARCNET | COM90C65LJ | ISA |
|  | NE-2000 |  | ISA |
|  | NE-2100 |  | ISA |
| IBM | TOKEN RING |  | ISA |
| SURECOM |  | AMD PCNET | PCI |
| SVEC |  | AMD PCNET | PCI |
| SURECOM |  | DEC 21040 | PCI |

II.		SOFTWARE COMPATIBILITY TEST

| OPERATING SYSTEM / SOFTWARE | VERSION | REMARKS |
| --- | --- | --- |
| MS-DOS | 5.0, 6.0, 6.2 |  |
| IBM OS/2 | 2.1 |  |
| WINDOWS NT | 3.1 |  |
| SCO UNIX | 3.2.4 |  |
| NOVELL NETWARE | 3.12 |  |
| WINSTONE 94 | 94 |  |
| BAPCO '93 SYSTEM MARK | 93 |  |

III.	PERFORMANCE  TEST

Testing  Configuration:

Mainboard		:	MS-4143

Version 		:	Ver 0B

System BIOS	:	AMI AD5L

System Chipset:	ALI M1439 + M1445 + On-chip IDE controller

VGA Card		:	MS-4405 ET4000/W32P PCI VGA Card (1 MB)

I/O Controller		:	On-board Winbond W83787F / W83758F Super I/O Controller

System Memory	:	8 MB (Samsung KM416C1200J-7)

Cache Memory		:	256 KB (Winbond W24257AK-15)

Hard Disk drive	:	Conner CP30254 (240MB) HDD

|  | INTEL | AMD | INTEL P24D |
| --- | --- | --- | --- |
|  | DX4-100 | DX2-80 | DX2-66 |
| PC BENCHMARK 7.0 |  |  |  |
| DOSMark | 28.84 | 33.74 | 27.94 |
| Processor Harmonic | 18128.88 | 16063.96 | 14647.67 |
| Video Harmonic | 7698.19 | 7154.78 | 7692.92 |
| Memory Harmonic | 13445.19 | 13189.97 | 12458.13 |
| Disk Harmonic | 21.76 | 27.04 | 21.73 |

|  | INTEL/AMD | CYRIX | AMD |
| --- | --- | --- | --- |
|  | DX2-66 | DX2-V50 | DX-40 |
| PC BENCHMARK 7.0 |  |  |  |
| DOSMark | 35.24 | 32.58 | 25.59 |
| Processor Harmonic | 14460.56 | 9257.75 | 10383.42 |
| Video Harmonic | 7787.45 | 5954.58 | 7105.78 |
| Memory Harmonic | 12337.39 | 9617.86 | 11067.92 |
| Disk Harmonic | 29.45 | 21.31 | 20.98 |

|  | INTEL | AMD | INTEL P24D |
| --- | --- | --- | --- |
|  | DX4-100 | DX2-80 | DX2-66 |
| BYTE BENCHMARK 1.3 |  |  |  |
| CPU Index | 19.85 | 14.57 | 13.57 |
| FPU Index | 82.38 | 70.60 | 58.95 |
| Disk Index | N.A. | 25.80 | N.A. |
| Video Index | 14.75 | 12.96 | 12.84 |
| BYTE BENCHMARK 2.2 |  |  |  |
| CPU Index | 1.78 | 1.63 | 1.50 |
| FPU Index | 6.89 | 5.54 | 4.59 |
| Disk Index | 1.01 | 1.19 | 1.01 |
| Video Index | 4.51 | 3.79 | 3.49 |

|  | INTEL/AMD | CYRIX | AMD |
| --- | --- | --- | --- |
|  | DX2-66 | DX2-V50 | DX-40 |
| BYTE BENCHMARK 1.3 |  |  |  |
| CPU Index | 13.50 | 10.83 | 10.15 |
| FPU Index | 56.51 | 42.84 | 35.70 |
| Disk Index | 22.78 | 22.88 | 24.35 |
| Video Index | 11.29 | 9.47 | 9.52 |
| BYTE BENCHMARK 2.2 |  |  |  |
| CPU Index | 1.76 | 1.17 | 1.28 |
| FPU Index | 4.59 | 3.68 | 2.77 |
| Disk Index | 1.00 | 0.98 | 1.01 |
| Video Index | 3.53 | 2.45 | 2.36 |

|  | INTEL | AMD | INTEL P24D |
| --- | --- | --- | --- |
|  | DX4-100 | DX2-80 | DX2-66 |
| NORTON SI 4.5 |  |  |  |
| Computing Index | 197.3 | 125.2 | 104.2 |
| Disk Index | 7.6 | 7.5 | 7.6 |
| Performance Index | 134.0 | 85.9 | 72.0 |
| NORTON SI 7.0 |  |  |  |
| Computing Index | 131.4 | 118.8 | 143.4 |
| Disk Index | 12.6 | 12.5 | 12.4 |
| Performance Index | 91.7 | 83.1 | 99.5 |

|  | INTEL/AMD | CYRIX | AMD |
| --- | --- | --- | --- |
|  | DX2-66 | DX2-V50 | DX-40 |
| NORTON SI 4.5 |  |  |  |
| Computing Index | 110.0 | 109.5 | 66.5 |
| Disk Index | 7.5 | 7.3 | 7.3 |
| Performance Index | 75.8 | 75.4 | 46.7 |
| NORTON SI 7.0 |  |  |  |
| Computing Index | 131.4 | 84.9 | 86.4 |
| Disk Index | 12.5 | 12.4 | 12.5 |
| Performance Index | 91.7 | 60.5 | 61.7 |

|  | INTEL | AMD | INTEL P24D |
| --- | --- | --- | --- |
|  | DX4-100 | DX2-80 | DX2-66 |
| LANDMARK SPEED 1.14 |  |  |  |
| CPU Speed | 200+ | 200+ | 200+ |
| LANDMARK SPEED 2.0 |  |  |  |
| CPU Speed | 358.07 | 267.61 | 222.01 |
| FPU Speed | 806.60 | 653.88 | 548.55 |
| Video Speed | 7391.00 | 6687.00 | 7391.00 |

|  | INTEL/AMD | CYRIX | AMD |
| --- | --- | --- | --- |
|  | DX2-66 | DX2-V50 | DX-40 |
| LANDMARK SPEED 1.14 |  |  |  |
| CPU Speed | 200+ | 151.9 | 182.3 |
| LANDMARK SPEED 2.0 |  |  |  |
| CPU Speed | 222.0 | 164.91 | 133.81 |
| FPU Speed | 543.02 | 444.25 | 327.07 |
| Video Speed | 7391.00 | 5553.00 | 6687.00 |

|  | INTEL | AMD | INTEL P24D |  |
| --- | --- | --- | --- | --- |
| CHECKIT 3.0 | DX4-100 | DX2-80 | DX2-66 | UNITS |
| MAIN SYSTEM |  |  |  |  |
| CPU Speed | 41282 | 39211 | 33609 | Dhrystones |
| Math Speed | 13915.0 | 12587.5 | 11371.7 | M-Whetstones |
| VIDEO SYSTEM |  |  |  |  |
| BIOS Video Speed | 33681 | 31293 | 34636 | Char/S |
| Direct Video Speed | 314666 | 292236 | 264038 | Char/S |
| HARD DISK |  |  |  |  |
| Transfer Speed | 1139.3 | 1139.3 | 1139.3 | K/Second |
| Average Seek Time | 13.3 | 13.6 | 13.3 | ms |
| Track Seek Time | 4.5 | 5.2 | 5.3 | ms |

|  | INTEL/AMD | CYRIX | AMD |  |
| --- | --- | --- | --- | --- |
| CHECKIT 3.0 | DX2-66 | DX2-V50 | DX-40 | UNITS |
| MAIN SYSTEM |  |  |  |  |
| CPU Speed | 33609 | 22554 | 19303 | Dhrystones |
| Math Speed | 11417.5 | 8615.7 | 6880.1 | M-Whetstones |
| VIDEO SYSTEM |  |  |  |  |
| BIOS Video Speed | 30447 | 26276 | 25566 | Char/S |
| Direct Video Speed | 294799 | 288391 | 252502 | Char/S |
| HARD DISK |  |  |  |  |
| Transfer Speed | 1139.3 | 1079.4 | 1139.3 | K/Second |
| Average Seek Time | 13.6 | 13.6 | 13.6 | ms |
| Track Seek Time | 5.1 | 5.3 | 4.7 | ms |

|  | INTEL | AMD | INTEL P24D |  |
| --- | --- | --- | --- | --- |
| POWER METER 1.7 | DX4-100 | DX2-80 | DX2-66 | UNITS |
| SYSTEM TEST |  |  |  |  |
| Agg. Hardware | 2640.1 | 2208.9 | 1926.3 | PMUs |
| Spreadsheet | 3404.4 | 3570.5 | 3828.3 | PMUs |
| Word Processing | 3707.1 | 3518.7 | 3799.7 | PMUs |
| Database App'n. | 2543.7 | 2430.9 | 3457.1 | PMUs |
| CAE / CAD | 64412.0 | 53055.3 | 45093.0 | PMUs |
| Throughput Test | 19723.1 | 15726.0 | 14962.1 | PMUs |
| CPU MEMORY |  |  |  |  |
| Agg. CPU/Memory | 38396.3 | 31351.3 | 26657.7 | PMUs |
| Clock Speed Test | 96.5 | 78.0 | 65.0 | MHertz |
| MIPS Test | 33.9 | 29.1 | 28.9 | MIPS |
| Dhrystone Test | 43.4 | 37.3 | 37.0 | K-Dhrystones/S |
| Whetstone Test | 9608.8 | 7868.6 | 6531.5 | K-Whetstones/S |
| Sieve Test | 0.1 | 0.1 | 0.1 | Seconds |
| DISK DRIVE |  |  |  |  |
| Agg. Disk Test | 417.3 | 405.3 | 405.5 | PMUs |
| Trk-to-Trk Seek | 3.8 | 4.0 | 4.0 | MS/Seek |
| Mean Seek Test | 10.2 | 10.3 | 10.3 | MS/Seek |
| Data Transfer Rate | 2470.9 | 2470.9 | 2470.9 | Kb/Sec. |
| File Open/Close | 6.5 | 6.3 | 6.3 | MS/Op. |
| Seq File Rd/Wrt | 0.1 | 0.2 | 0.1 | MS/Op. |
| Ran File Rd/Wrt | 15.1 | 12.8 | 13.3 | MS/Op. |
| VIDEO DISPLAY |  |  |  |  |
| Agg Video Test | 8213.5 | 7720.0 | 8647.9 | PMUs Md 3 |
| Char Write Test | 0.5 | 0.5 | 0.5 | Sec. Md 3 |
| Line Drawing Test | 4.1 | 4.3 | 4.0 | sec. Md 18 |
| Fill Test | 21.9 | 23.8 | 22.4 | Sec. Md 18 |

|  | INTEL/AMD | CYRIX | AMD |  |
| --- | --- | --- | --- | --- |
| POWER METER 1.7 | DX2-66 | DX2-V50 | DX-40 | UNITS |
| SYSTEM TEST |  |  |  |  |
| Agg. Hardware | 1907.7 | 1573.7 | 1266.1 | PMUs |
| Spreadsheet | 6152.4 | 3480.7 | 3274.6 | PMUs |
| Word Processing | 4470.1 | 3815.3 | 3048.6 | PMUs |
| Database App'n. | 2895.4 | 2214.1 | 2600.6 | PMUs |
| CAE / CAD | 44081.6 | 37685.2 | 27906.0 | PMUs |
| Throughput Test | 13813.3 | 11659.2 | 9992.1 | PMUs |
| CPU MEMORY |  |  |  |  |
| Agg. CPU/Memory | 26708.7 | 21610.7 | 15954.3 | PMUs |
| Clock Speed Test | 64.5 | 48.5 | 39.5 | MHertz |
| MIPS Test | 27.1 | 17.5 | 17.6 | MIPS |
| Dhrystone Test | 34.7 | 24.6 | 22.6 | K-Dhrystones/S |
| Whetstone Test | 6531.5 | 5259.5 | 3918.9 | K-Whetstones/S |
| Sieve Test | 0.1 | 0.1 | 0.1 | Seconds |
| DISK DRIVE |  |  |  |  |
| Agg. Disk Test | 416.3 | 392.1 | 408.0 | PMUs |
| Trk-to-Trk Seek | 4.0 | 4.0 | 3.7 | MS/Seek |
| Mean Seek Test | 10.2 | 10.0 | 10.3 | MS/Seek |
| Data Transfer Rate | 2470.9 | 2378.1 | 2470.9 | Kb/Sec. |
| File Open/Close | 6.3 | 6.3 | 6.3 | MS/Op. |
| Seq File Rd/Wrt | 0.2 | 0.2 | 0.1 | MS/Op. |
| Ran File Rd/Wrt | 16.5 | 12.3 | 12.8 | MS/Op. |
| VIDEO DISPLAY |  |  |  |  |
| Agg Video Test | 8102.5 | 6493.7 | 7312.0 | PMUs Md 3 |
| Char Write Test | 0.5 | 0.6 | 0.5 | Sec. Md 3 |
| Line Drawing Test | 4.3 | 5.2 | 4.7 | sec. Md 18 |
| Fill Test | 23.5 | 27.0 | 25.8 | Sec. Md 18 |

|  | INTEL | AMD | INTEL P24D |  |
| --- | --- | --- | --- | --- |
| POWER METER 1.81 | DX4-100 | DX2-80 | DX2-66 | UNITS |
| SYSTEM TEST |  |  |  |  |
| Agg. Hardware | 2584.9 | 2166.5 | 1902.7 | PMUs |
| Spreadsheet | 4172.2 | 4841.3 | 4305.6 | PMUs |
| Word Processing | 4563.7 | 5753.7 | 5104.8 | PMUs |
| Database App'n. | 2187.7 | 3841.0 | 2938.9 | PMUs |
| CAE / CAD | 63938.9 | 54124.4 | 44809.2 | PMUs |
| Throughput Test | 18606.3 | 17447.2 | 13966.0 | PMUs |
| CPU MEMORY |  |  |  |  |
| Agg. CPU/Memory | 37328.7 | 30640.7 | 25675.3 | PMUs |
| Clock Speed Test | 96.3 | 77.9 | 66.0 | MHertz |
| MIPS Test | 24.9 | 21.2 | 21.2 | MIPS |
| Dhrystone Test | 43.8 | 37.3 | 37.3 | K-Dhrystones/S |
| Whetstone Test | 9608.8 | 7807.1 | 6489.0 | K-Whetstones/S |
| Sieve Test | 0.1 | 0.1 | 0.1 | Seconds |
| DISK DRIVE |  |  |  |  |
| Agg. Disk Test | 412.7 | 412.3 | 415.2 | PMUs |
| Trk-to-Trk Seek | 3.8 | 3.9 | 4.0 | MS/Seek |
| Mean Seek Test | 10.2 | 10.1 | 10.3 | MS/Seek |
| Data Transfer Rate | 2513.8 | 2513.8 | 2495.2 | Kb/Sec. |
| File Open/Close | 6.4 | 6.2 | 6.2 | MS/Op. |
| Seq File Rd/Wrt | 0.2 | 0.2 | 0.2 | MS/Op. |
| Ran File Rd/Wrt | 15.7 | 13.1 | 17.2 | MS/Op. |
| VIDEO DISPLAY |  |  |  |  |
| Agg Video Test | 8213.5 | 7753.3 | 8647.9 | PMUs Md 3 |
| Char Write Test | 0.5 | 0.5 | 0.5 | Sec. Md 3 |
| Line Drawing Test | 4.1 | 4.3 | 4.0 | sec. Md 18 |
| Fill Test | 21.9 | 23.8 | 22.4 | Sec. Md 18 |

|  | INTEL/AMD | CYRIX | AMD |  |
| --- | --- | --- | --- | --- |
| POWER METER 1.81 | DX2-66 | DX2-V50 | DX-40 | UNITS |
| SYSTEM TEST |  |  |  |  |
| Agg. Hardware | 1883.9 | 1573.9 | 1234.8 | PMUs |
| Spreadsheet | 4099.0 | 3685.6 | 3965.1 | PMUs |
| Word Processing | 3935.0 | 4376.6 | 4145.5 | PMUs |
| Database App'n. | 3114.6 | 1358.7 | 2725.3 | PMUs |
| CAE / CAD | 45680.4 | 36569.1 | 27375.8 | PMUs |
| Throughput Test | 15813.8 | 10496.5 | 9893.8 | PMUs |
| CPU MEMORY |  |  |  |  |
| Agg. CPU/Memory | 25721.3 | 21635.7 | 15599.0 | PMUs |
| Clock Speed Test | 66.0 | 48.6 | 40.0 | MHertz |
| MIPS Test | 19.7 | 14.0 | 12.9 | MIPS |
| Dhrystone Test | 34.7 | 24.6 | 22.6 | K-Dhrystones/S |
| Whetstone Test | 6489.0 | 5710.4 | 3918.9 | K-Whetstones/S |
| Sieve Test | 0.1 | 0.1 | 0.1 | Seconds |
| DISK DRIVE |  |  |  |  |
| Agg. Disk Test | 412.7 | 399.4 | 418.6 | PMUs |
| Trk-to-Trk Seek | 4.0 | 4.0 | 3.8 | MS/Seek |
| Mean Seek Test | 10.3 | 10.0 | 10.1 | MS/Seek |
| Data Transfer Rate | 2507.6 | 2406.3 | 2513.8 | Kb/Sec. |
| File Open/Close | 6.2 | 6.2 | 6.2 | MS/Op. |
| Seq File Rd/Wrt | 0.1 | 0.2 | 0.3 | MS/Op. |
| Ran File Rd/Wrt | 12.7 | 14.0 | 14.4 | MS/Op. |
| VIDEO DISPLAY |  |  |  |  |
| Agg Video Test | 8102.5 | 6493.7 | 7312.0 | PMUs Md 3 |
| Char Write Test | 0.5 | 0.6 | 0.5 | Sec. Md 3 |
| Line Drawing Test | 4.3 | 5.2 | 4.7 | sec. Md 18 |
| Fill Test | 23.5 | 27.0 | 25.8 | Sec. Md 18 |

|  | INTEL | AMD | INTEL P24D |  |
| --- | --- | --- | --- | --- |
| QA PLUS 4.61 | DX4-100 | DX2-80 | DX2-66 | UNITS |
| SYSTEM PERF. |  |  |  |  |
| CPU Speed | 45517 | 45517 | 45517 | Dhrystones |
| Direct Video Speed | 757390 | 757390 | 757390 | CPS |
| Math Speed | 14817.9 | 12732.4 | 11287.3 | K-Whetstones |
| DISK PERF. |  |  |  |  |
| Trk-Trk Seek | 4.35 | 4.57 | 4.60 | ms |
| Average Seek | 11.8 | 12.0 | 11.8 | ms |
| Transfer Rate | 481.9 | 862.3 | 910.2 | K/sec |

|  | INTEL/AMD | CYRIX | AMD |  |
| --- | --- | --- | --- | --- |
| QA PLUS 4.61 | DX2-66 | DX2-V50 | DX-40 | UNITS |
| SYSTEM PERF. |  |  |  |  |
| CPU Speed | 45517 | 22758 | 22758 | Dhrystones |
| Direct Video Speed | 757390 | 473369 | 473369 | CPS |
| Math Speed | 105240 | 9167.5 | 6918.8 | K-Whetstones |
| DISK PERF. |  |  |  |  |
| Trk-Trk Seek | 4.72 | 4.59 | 4.33 | ms |
| Average Seek | 12.1 | 12.1 | 11.5 | ms |
| Transfer Rate | 248.2 | 356.2 | 1365.3 | K/sec |

|  | INTEL | AMD | INTEL P24D |  |
| --- | --- | --- | --- | --- |
| QA PLUS 4.70 | DX4-100 | DX2-80 | DX2-66 | UNITS |
| SYSTEM PERF. |  |  |  |  |
| CPU Speed | 45.5 | 45.5 | 45.5 | K-Dhrystones |
| Direct Video Speed | 757.4 | 757.4 | 757.4 | K-CPS |
| Math Speed | 14.4 | 11.9 | 10.6 | M-Whetstones |
| DISK PERF. |  |  |  |  |
| Trk-Trk Seek | 4.35 | 4.53 | 4.59 | ms |
| Average Seek | 11.7 | 12.0 | 11.9 | ms |
| Transfer Rate | 712.3 | 496.5 | 963.8 | K/sec |

|  | INTEL/AMD | CYRIX | AMD |  |
| --- | --- | --- | --- | --- |
| QA PLUS 4.70 | DX2-66 | DX2-V50 | DX-40 | UNITS |
| SYSTEM PERF. |  |  |  |  |
| CPU Speed | 45.5 | 22.8 | 22.8 | K-Dhrystones |
| Direct Video Speed | 757.4 | 473.4 | 473.4 | K-CPS |
| Math Speed | 10.6 | 8.7 | 6.4 | M-Whetstones |
| DISK PERF. |  |  |  |  |
| Trk-Trk Seek | 4.70 | 4.57 | 4.35 | ms |
| Average Seek | 12.1 | 12.0 | 11.5 | ms |
| Transfer Rate | 356.2 | 431.2 | 546.1 | K/sec |

The following Windows programs were tested using MS-4405 Tseng Labs' ET4000/W32P (Ver. C, 1 MB memory) Windows driver Ver 1.2, 1024 x 768 x 256C resolution.

|  | INTEL | AMD | INTEL P24D |  |
| --- | --- | --- | --- | --- |
| WINDOWS TEST | DX4-100 | DX2-80 | DX2-66 | UNITS |
| SPEEDY | 63.63 | 58.48 | 57.63 | SPEEDMARK |
| WINTACH 1.2 | 35.32 | 30.24 | 30.40 | RPM |
| WINBENCH 3.11 | 18.18 | 17.39 | 17.39 | WINMARK |
| WINBENCH 4.0 | 13.33 | 10.8 | 10.9 | WINMARK |

|  | INTEL/AMD | CYRIX | AMD |  |
| --- | --- | --- | --- | --- |
| WINDOWS TEST | DX2-66 | DX2-V50 | DX-40 | UNITS |
| SPEEDY | 56.66 | 41.13 | 50.07 | SPEEDMARK |
| WINTACH 1.2 | 29.54 | 21.33 | 25.28 | RPM |
| WINBENCH 3.11 | 17.28 | 15.51 | 16.47 | WINMARK |
| WINBENCH 4.0 | 10.5 | 7.77 | 9.51 | WINMARK |

|  | INTEL | AMD | INTEL P24D |
| --- | --- | --- | --- |
| WINDSOCK | DX4-100 | DX2-80 | DX2-66 |
| CPU | 835 | 592 | 471 |
| VIDEO | SU - 1101 | SU - 923 | SU - 883 |
| DISK | GR - 423 | GR - 322 | GR - 306 |
| MEMORY | GO - 163 | GO - 172 | GO - 161 |
| WINDOWS (OVERALL) | SU - 679 | SU - 522 | SU - 458 |

|  | INTEL/AMD | CYRIX | AMD |
| --- | --- | --- | --- |
| WINDSOCK | DX2-66 | DX2-V50 | DX-40 |
| CPU | 492 | 392 | 295 |
| VIDEO | SU - 855 | SU - 495 | SU - 628 |
| DISK | GR - 302 | GR - 291 | GR - 262 |
| MEMORY | GO - 160 | GO - 117 | GO - 170 |
| WINDOWS (OVERALL) | SU - 464 | GR - 312 | GR - 329 |

Note:

SU	=	Superb

GR	=	Great

GO 	=	Good

All brand names and trademarks mentioned herein are the properties of their respective owners.