MS-5103  PCI / ISA  MAINBOARD  TEST  REPORT

December  28,  1993

Mainboard      MS-5103

Chipset            Intel  Mercury

Processor         Intel  Pentium  60 MHz

Version	          Ver.  1

BIOS               Phoenix  PM61  and  Award  WM61 / WM62

I.		SOFTWARE COMPATIBILITY TEST

A.  DOS

| MS-DOS | VER 5.0 | VER 6.0 | REMARKS |
| --- | --- | --- | --- |
| INSTALL | OK | OK |  |
| BOOT | OK | OK |  |
| FORMAT A: | OK | OK |  |
| BOOT FROM A | OK | OK |  |
| COPY | OK | OK |  |
| COMP | OK | OK |  |
| XCOPY | OK | OK |  |
| DISKCOPY | OK | OK |  |
| DISKCOMP | OK | OK |  |
| M.C.R. DIR | OK | OK |  |
| EMM386 | OK | OK | no sofware speed switching * |

* This is a known occurrence for the Pentium CPU.

B.  IBM  OS/2

Configuration:

·	16MB Memory  (Panasonic MN414400ASJ-7)

·	256KB Cache (Winbond W24257AK-20)

·	Winbond Multi-I/O Controller

·	Conner CP30254 (254MB) Hard Disk

·	MS-4401 (ALI 2201) PCI VGA Card

·	One 1.44MB and one 1.2MB FDDs

·	Genius Mouse

| OS/2 | VER 2.0 | VER 2.1 | REMARKS |
| --- | --- | --- | --- |
| INSTALL | OK | OK |  |
| BOOT | OK | OK |  |
| FORMAT A: | OK | OK |  |
| COPY | OK | OK |  |
| COMP | OK | OK |  |
| XCOPY | OK | OK |  |
| DISKCOPY | OK | OK |  |
| DISKCOMP | OK | OK |  |
| M.C.R. DIR | OK | OK |  |
| TASK TEST | OK | OK |  |
| WIN-OS/2 | OK | OK |  |
| WIN PROGRAMS | OK | OK |  |

C.  SCO  UNIX

Configuration:

·	32MB Memory  (Micron MT4C4001JDJ-7)

·	256KB Cache (Winbond W24257AK-20)

·	Adaptec AHA-1542C SCSI Controller

·	Maxtor 4170S (170MB) SCSI Hard Disk

·	IIT AGX-014 ISA VGA Card

·	HDD/FDD Controller

·	1S/1P Card

·	One 1.44MB and one 1.2MB FDDs

·	Genius Mouse

| SCO UNIX | REL 3.2.1 | REL 3.2.4 | REMARKS |
| --- | --- | --- | --- |
| INSTALL | OK | OK |  |
| BOOT | OK | OK |  |
| O.D.T. | OK | OK |  |
| DOS MODE | OK | OK |  |
| CP FILE | OK | OK |  |
| TASK TEST | OK | OK |  |

D.  WINDOWS  N.T.

Configuration:

·	16MB Memory  (Micron MT4C4001JDJ-7)

·	256KB Cache (Winbond W24257AK-20)

·	Symphony PCI IDE Controller

·	Conner CP3104 (104MB) Hard Disk

·	S3805 PCI VGA Card

·	UMC Multi-I/O Controller

·	One 1.44MB and one 1.2MB FDDs

·	Genius Mouse

| WINDOWS N.T. | VER 3.1 | REMARKS |
| --- | --- | --- |
| INSTALL | OK |  |
| BOOT | OK |  |
| FORMAT A: | OK |  |
| COMP | OK |  |
| XCOPY | OK |  |
| DISKCOPY | OK |  |
| DISKCOMP | OK |  |
| TASK TEST | OK |  |

E.  NOVELL  NETWARE

Configuration:

·	32MB Memory  (Micron MT4C4001JDJ-7)

·	256KB Cache (Winbond W24257AK-20)

·	MS-3805 NCR 53C810PT PCI SCSI Controller

·	Maxtor 4170S (170MB) SCSI Hard Disk

·	Winbond Multi-I/O Controller

·	Conner CP3044 (40MB) Hard Disk

·	MS-4401 (ALI 2201) PCI VGA Card

| NETWARE | 386 V3.11 | REMARKS |
| --- | --- | --- |
| INSTALL | OK |  |
| BOOT | OK |  |
| NE2000 | OK | File Server |
| NE2100 | OK | File Server |
| NE2000 | OK | Work Station |
| NE2100 | OK | Work Station |
| BOOT ROM | OK |  |
| FILE Trans. | OK |  |
| FILE Comp. | OK |  |
| NWTEST | OK |  |

F.  PACKETS  AND  UTILITIES

| S O F T W A R E | R E M A R K S |
| --- | --- |
| AutoCAD R10 c10 | OK |
| AutoCAD R11 | OK |
| AutoCAD R12 | OK |
| Central Point Anti-Virus 1.1 | OK |
| Clipper 5.01 | OK |
| CorelDraw V3.0 | OK |
| DBase III Plus | OK |
| DBase 4 | OK |
| GeoWorks Ensemble V1.2 | OK |
| Harvard Graphics V2.12 | OK |
| Lotus 1-2-3 R2.4 | OK |
| Lotus 1-2-3 R3.1+ | OK |
| Microsoft Entertainment Pack Vol 1 | OK |
| Microsoft Entertainment Pack Vol 2 | OK |
| Microsoft Entertainment Pack Vol 3 | OK |
| Microsoft Excel V4.0 | OK |
| Microsoft Visual Basic V2.0 | OK |
| Microsoft Windows 3.1 | OK |
| Microsoft Word for Windows V2.0 | OK |
| Microsoft Works | OK |
| Norton Desktop for Windows V1.2 | OK |
| Norton Utilities V7.0 | OK |
| PC Tools V8.0 | OK |
| PE 2 | OK |
| Power Point for Windows | OK |
| QEMM 386 V5.11 | OK, no software speed switching * |
| QEMM 386 V6.0 | OK, no software speed switching * |
| QEMM 386 V7.01 | OK, no software speed switching * |
| QEMM 386 V7.02 | OK, no software speed switching * |
| Symantec Entertainment Pack for Windows | OK |
| TopDOS 3.00 | OK |
| Ventura Publisher V2.0 | OK |
| WordPerfect 5.1 | OK |
| XTree Gold V2.51 | OK |

* This is a known occurrence for the Pentium CPU.

G.  DIAGNOSTIC  PROGRAMS

| S O F T W A R E | R E M A R K S |
| --- | --- |
| Byte Benchmark V1.3 | OK |
| Byte Benchmark V2.2 | OK |
| CheckIt V3.0 | OK |
| Core Test V2.92 | OK |
| IBM Diagnostics V2.03 | OK |
| Landmark Speed Test V1.14 | OK |
| Landmark Speed Test V2.0 | OK |
| Microsoft Software Diagnostics (MSD) | OK |
| Norton Diagnostics V7.0 | OK |
| Norton System Information V 4.5 | OK |
| Norton System Information V7.0 | OK |
| PC Magazine Labs Benchmark 6.0 | OK |
| PC Magazine Labs Benchmark 7.0 | OK |
| PC Week Labs System Benchmarks V1.10 | OK |
| Power Meter V1.7 | OK |
| Power Meter V1.81 | OK |
| QA Plus V4.61 | OK |
| QA Plus V4.70 | OK |
| QA Plus/FE V5.03 | OK, Interval Timer Failed * |
| QA Plus/FE V5.04 | OK, Interval Timer Failed * |
| QA Plus/FE V5.12 | OK |
| Speedy | OK |
| Superscale 3D Benchmark | OK |
| WinBench V2.5 | OK |
| WinBench V3.11 | OK |
| WinBench V4.0 | OK |
| WindSock 3.1 | OK |
| WinStone 94 | OK |
| WinTach V1.0 | OK |
| WinTach V1.2 | OK |

* This is a known occurrence for the Pentium CPU.

II.		HARDWARE COMPATIBILITY TEST

A.	VIDEO  CARDS

| MANUFACTURER | CHIPSET | REMARKS |
| --- | --- | --- |
| IIT | AGX-014 | ISA |
| OAK | OTI-077 | ISA |
| S3 | S3801 | ISA |
| TRIDENT | TVGA8900C | ISA |
| TSENG LABS | ET4000/W32 | ISA |
| WD | WD90C30 | ISA |
| AVANCE LOGIC | ALI 2201 | PCI |
| AVANCE LOGIC | ALI 2301 | PCI |
| CIRRUS LOGIC | CL5428 | PCI |
| IIT | AGX-016 | PCI |
| S3 | S3805 | PCI |
| S3 | S3928 | PCI |
| S3 | S3928P | PCI |
| TSENG LABS | ET4000/W32P | PCI |

B.	I/O  CONTROLLERS

| MANUFACTURER | CHIPSET | REMARKS |
| --- | --- | --- |
| COMBAT | 90C50 | ISA |
| GOLDSTAR | GM82C765B | ISA |
| HMC | HM83453 | ISA |
| UMC | UM82C862F | ISA |
| WINBOND | WF3757F | ISA |
| SYMPHONY | SL82C101 | PCI |
| TEKRAM | DC-690B | PCI |

C.	SCSI CONTROLLERS

| MANUFACTURER | CHIPSET | REMARKS |
| --- | --- | --- |
| ADAPTEC | AHA-1542B | ISA |
| ADAPTEC | AHA-1542C | ISA |
| NCR | 53C810PT | PCI |

D.	NETWORK  ADAPTERS

| MANUFACTURER | CHIPSET | REMARKS |
| --- | --- | --- |
| 3COM | 3C503 | ISA |
| NE-2000 |  | ISA |
| NE-2100 |  | ISA |

E.	HARD  DISK  DRIVES

| MANUFACTURER | MODEL NUMBER | CAPACITY | REMARKS |
| --- | --- | --- | --- |
| CONNER | CP3104 | 104 MB | IDE |
| CONNER | CP3204F | 204 MB | IDE |
| CONNER | CP30254 | 254 MB | IDE |
| QUANTUM | 240AT | 240 MB | IDE |
| SEAGATE | ST3243A | 214 MB | IDE |
| WD | CAVIAR 2250 | 250 MB | IDE |
| CONNER | CP3100 | 100 MB | SCSI |
| MAXTOR | 7120S | 120 MB | SCSI |
| MAXTOR | 4170S | 170 MB | SCSI |
| QUANTUM | ELS 170S | 170 MB | SCSI |

F.	FLOPPY  DISK  DRIVES

| MANUFACTURER | MODEL NUMBER | CAPACITY |
| --- | --- | --- |
| MITSUBISHI | MF504C-315MG | 1.2 MB |
| MITSUBISHI | MF355C-255MG | 1.44 MB |
| TEAC | FD235HF | 1.44 MB |

G.	MEMORY  MODULES

| MANUFACTURER | MODEL NUMBER | SIZE |
| --- | --- | --- |
| MICRON TECH | MT4C4001JDJ-7 | 8 MB |
| MITSUBISHI | M5M44260AJ-8 | 4 MB |
| PANASONIC | MN41C41000SJ-8 | 4 MB |
| PANASONIC | MN414400ASJ-7 | 4 MB |
| SAMSUNG | KM416C256AJ-7 | 4 MB |

H.	STATIC  RAMs

| MANUFACTURER | MODEL NUMBER | SIZE |
| --- | --- | --- |
| CYPRESS | CY7C199-20 | 32K * 8 |
| MICRON TECH | MT5C2568-20 | 32K * 8 |
| TOSHIBA | TC55328P-20 | 32K * 8 |
| WINBOND | W24257AK-20 | 32K * 8 |

I.	MISCELLANEOUS  STORAGE  DEVICES

| TYPE | MANUFACTURER | MODEL NUMBER | CAPACITY |
| --- | --- | --- | --- |
| TAPE DRIVE | IRWIN BACKUP | ACCUTRAK PLUS + | 250 MB |

III.	PERFORMANCE  TEST

Configuration:

·	Intel  Pentium  60 MHz

·	16MB Memory  (Micron Tech MT4C4001JDJ-7)

·	256KB Cache (Winbond W24257AK-20)

·	Winbond ISA Multi-I/O Controller

·	Conner CP30254 (254MB) Hard Disk

·	MS-4401 (ALI 2201) PCI VGA Card

·	One 1.44MB and one 1.2MB FDDs

·	Genius Mouse

A.	BYTE  BENCHMARKS  VERSION   1.3

Test  Machine          IBM  AT           Units

CPU TESTS

¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾

Matrix:		0.53	11.69	sec

String move:

Byte-wide:  	17.25	80.41	sec

Word-wide(odd): 	13.00	80.41	sec

Word-wide(even):   	8.77	40.26	sec

Sieve:   		2.32               	73.65	sec

Sort:   			1.81	84.39	sec

¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾

CPU  index  is   16.13

FPU TESTS

¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾

Math:   		0.39	46.46	sec

Sin(x):   		0.23	20.05	sec

e^x:   			0.13	17.20	sec

¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾

FPU  index  is  101.70

DISK TESTS

¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾

DOS seek:

Sector read:   	0.02	14.95	sec

32-Sector read:	0.02	65.18	sec

File I/O:

Seek:   	0.07	0.29	sec

Read rate: 	0.57 	1.33                 sec/Kb

Write rate:	0.64 	1.23                 sec/Kb

One megabyte:

Write:   	2.58 	8.97	sec

Read:   	1.54 	9.17	sec

Hard disk seek:

Outer track:   	0.18 	3.28	sec

Inner track:   	0.15 	3.30	sec

Half Platter:   	0.20 	11.46	sec

Full Platter:   	0.18 	16.59	sec

¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾

Disk  index  is   21.86

VIDEO TESTS

¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾

VGA mode test:

Text:

Mode 0:   	0.71	11.55	sec

Mode 1:   	0.71 	11.53	sec

Mode 2:   	0.66 	13.15	sec

Mode 3:   	0.66 	13.13	sec

Graphics:

Mode 4:   	0.22 	4.69	sec

Mode 5:   	0.25 	4.69	sec

Mode 6:   	0.24 	11.00	sec

Mode 13:   	0.77                    N/A	sec

Mode 14:   	0.77                    N/A	sec

Mode 16:   	0.80                    N/A	sec

Mode 18:   	0.79                    N/A	sec

Mode 19:   	0.16                    N/A	sec

¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾

Video  index  is   13.11

B.	BYTE  DOS  BENCHMARKS  VERSION 2.2

Test Machine  CYRIX486DLC/40 AVAX 486DX/33    Units

CPU Tests

Sieve:			635.48          	96.97	116.10			ips.

Sort:          		31.20          	12.07	12.17			ips.

Integer Math:         	1276229.50     	769258.20	512086.51   		ips.

Move (byte):         	124.58         	141.90	195.07      	 	ips.

Move (word-odd):         	170.78         	179.69         	187.14      		ips.

Move (word-even):         	270.73         	283.71      	353.73      		ips.

Move (Dword-odd):         	271.14         	249.12      	240.58      		ips.

Move (Dword-even):    	658.73         	567.18      	603.01      		ips.

¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾

CPU Index (Notebook Class):	5.04           	3.32           	3.48

CPU Index (Desktop Class):   	2.45           	1.42           	1.49

¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾

FPU Tests

Fmath:		1170445.43	142335.91	248421.56			ips.

Fourier:		14.09	3.42	4.24			ips.

¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾

FPU Index (Notebook Class): 	23.61	4.05	5.96

FPU Index (Desktop Class):   	9.12	1.57	2.30

¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾

Disk Tests

File I/O  Read:		323.20                     N/A                     N/A		KBps.

File I/O Write:		247.03                     N/A                     N/A		KBps.

¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾

Disk Index (Notebook Class): 	1.32                     N/A                     N/A

Disk Index (Desktop Class):  	1.25                     N/A                     N/A

¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾

Video Tests

Graphics:           		2.50                     N/A                     N/A		ips.

¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾

Video Index (Notebook Class):	8.33                     	N/A                     N/A

Video Index (Desktop Class):  	4.28                     N/A                     N/A

¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾

NOTE:	For all tests and all indexes, higher numbers indicate better performance.

Notebook Class indexes show performance relative to a Toshiba T2200SX.

Desktop Class indexes are calculated relative to a Compaq DeskPro 386/33L.

Key:

ips.	=  iterations per second

KBps.	=  kilobytes per second

N/A	=  not  applicable

C.	DIAGSOFT  POWER  METER  V1.7 / V1.81

PM  1.7         PM  1.81      Units

System Test Results

¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾

Aggregate Hardware Test	2998.1	2901.7	PMUs

Spreadsheet Simulation	3969.5	4264.9	PMUs

Word Processing Simulation	3903.1	4425.1	PMUs

Database Applic Simulation	2379.8	1539.2	PMUs

CAE/CAD Simulation Test	78853.2	79477.8	PMUs

Throughput Test	22470.7	21433.4	PMUs

¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾

CPU/Memory Test Results

¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾

Aggregate CPU/Memory Test	44940.7	43539.0	PMUs

Clock Speed Test	121.0	120.6	MHertz

MIPS Test	41.0	29.3	MIPS

Dhrystone Test	52.6	51.5	K-Dhrystones/Sec

Whetstone Test	11756.6	11619.9	K-Whetstones/sec

Sieve Test		0.1	0.1	Seconds

¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾

Disk Drive Test Results

¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾

Aggregate Disk Test	225.9	226.9	PMUs

Track-to-Track Seek Test	3.9	3.9	MS/Seek

Mean Seek Test	10.1	10.1	MS/Seek

Data Transfer Rate Test	1256.9	1247.6	KB/Sec.

File Open/Close Test	6.5	6.5	MS/Op.

Sequential File Rd/Wrt Test	0.2	0.2	MS/Op.

Random File Rd/Wrt Test	12.8	14.1	MS/Op.

¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾

Video Display Test Results

¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾

Aggregate Video Test	7463.7	7432.9	PMUs		Mode:3

Character Write Test	0.5	0.5	Seconds	Mode:3

Line Drawing Test	1.5	1.5	Seconds	Mode:18

Fill Test		6.6	6.6	Seconds	Mode:18

¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾

D.	PC  MAGAZINE  LABORATORY  BENCHMARK  SERIES  R7.0

DosMark(tm)	37.58

Processor Harmonic                   	25850.14

Video Harmonic                        	4276.42

Memory Harmonic                       	6904.76

Disk Harmonic                           	32.28

Processor Tests

16 Bit Prot. Mode Small Mix	215192	pass/sec

16 Bit Real Mode Small Mix	217700	pass/sec

16 Bit Prot. Mode Standard Mix	817.9	pass/sec

16 Bit Real Mode Standard Mix	826.7	pass/sec

String Sort and Move	NOT RUN	ops/sec

Prime Number Sieve	NOT RUN	ops/sec

Floating Point Emulation	1545.02	ops/sec

Math Coprocessor	34201.60	ops/sec

Memory Timing Tests

Ext Mem 8-bit Read	7825	Kbytes/sec

8-bit Read Start Address	1048576	(decimal)

8-bit Read End Address	16773120	(decimal)

Ext Mem 8-bit Write	2858	Kbytes/sec

8-bit Write Start Address	1048576	(decimal)

8-bit Write End Address            	16773120	(decimal)

Ext Mem 16-bit Read                     	13259 	Kbytes/sec

16-bit Read Start Address           	1048576 	(decimal)

16-bit Read End Address            	16773120 	(decimal)

Ext Mem 16-bit Write                     	5682 	Kbytes/sec

16-bit Write Start Address          	1048576 	(decimal)

16-bit Write End Address           	16773120 	(decimal)

Ext Mem 32-bit Read                     	19888 	Kbytes/sec

32-bit Read Start Address           	1048576 	(decimal)

32-bit Read End Address            	16773120 	(decimal)

Ext Mem 32-bit Write                    	11100 	Kbytes/sec

32-bit Write Start Address          	1048576 	(decimal)

32-bit Write End Address           	16773120 	(decimal)

Gr. Mem 8-bit Read                    	NOT RUN 	Kbytes/sec

Gr. Mem 8-bit Write                   	NOT RUN 	Kbytes/sec

Gr. Mem 16-bit Read                   	NOT RUN 	Kbytes/sec

Gr. Mem 16-bit Write                  	NOT RUN 	Kbytes/sec

Gr. Mem 32-bit Read                   	NOT RUN 	Kbytes/sec

Gr. Mem 32-bit Write                  	NOT RUN 	Kbytes/sec

BIOS Mem 8-bit Read                   	NOT RUN 	Kbytes/sec

BIOS Mem 16-bit Read                  	NOT RUN 	Kbytes/sec

BIOS Mem 32-bit Read                  	NOT RUN 	Kbytes/sec

Text Mem 8-bit Read                   	NOT RUN 	Kbytes/sec

Text Mem 8-bit Write                  	NOT RUN 	Kbytes/sec

Text Mem 16-bit Read                  	NOT RUN 	Kbytes/sec

Text Mem 16-bit Write                 	NOT RUN 	Kbytes/sec

Conv. Mem 8-bit Read                     	7825 	Kbytes/sec

Conv. Mem 8-bit Write                    	2858 	Kbytes/sec

Conv. Mem 16-bit Read                   	14464 	Kbytes/sec

Conv. Mem 16-bit Write                   	5682 	Kbytes/sec

EGA/VGA Text Mode Tests

Unscrolled BIOS Write                 	NOT RUN 	K chars/sec

Scrolled BIOS Write                   	NOT RUN 	K chars/sec

Direct Screen Write (8-bit)           	1133.40 	K chars/sec

Direct Screen Write (16-bit)          	2229.40 	K chars/sec

EGA/VGA Graphics Mode Tests

Write Mode O Fill (8-bit)             	NOT RUN 	K pixels/sec

Write Mode O Fill (16-bit)            	2990.18 	K pixels/sec

Write Mode O Fill (32-bit)            	4350.19 	K pixels/sec

Mem to Scr BitBlt (16-bit)            	7334.40 	K pixels/sec

Mem to Scr BitBlt (32-bit)           	10543.20 	K pixels/sec

Scr to Mem BitBlt (16-bit)            	4125.60 	K pixels/sec

Scr to Mem BitBlt (32-bit)            	5134.08 	K pixels/sec

Scr to Scr BitBlt (16-bit)           	11796.48 	K pixels/sec

Scr to Scr BitBlt (32-bit)           	14730.24 	K pixels/sec

Disk Access Tests

BIOS Disk Seek - Sequential           	NOT RUN	msec

BIOS Disk Seek - Random               	NOT RUN 	msec

DOS Disk Access                       	NOT RUN 	sec

Disk Throughput Tests

32M File

32M-200 Byte Records

32M/200 Sequential Read             	514.10 	Kbytes/sec

32M/200 Sequential Write             	18.59 	Kbytes/sec

32M/200 Random Read                   	2.40 	Kbytes/sec

32M/200 Random Write                  	1.57 	Kbytes/sec

32M-512 Byte Records

32M/512 Sequential Read             	541.33 	Kbytes/sec

32M/512 Sequential Write            	511.22 	Kbytes/sec

32M/512 Random Read                   	6.35 	Kbytes/sec

32M/512 Random Write                  	6.58 	Kbytes/sec

32M-2K Byte Records

32M/2K Sequential Read              	648.37 	Kbytes/sec

32M/2K Sequential Write             	539.74 	Kbytes/sec

32M/2K Random Read                   	24.10 	Kbytes/sec

32M/2K Random Write                  	24.83 	Kbytes/sec

32M-4K Byte Records

32M/4K Sequential Read              	674.37 	Kbytes/sec

32M/4K Sequential Write             	600.00 	Kbytes/sec

32M/4K Random Read                   	47.28 	Kbytes/sec

32M/4K Random Write                  	51.05 	Kbytes/sec

16M File

16M-200 Byte Records

16M/200 Sequential Read             	514.10 	Kbytes/sec

16M/200 Sequential Write             	18.59 	Kbytes/sec

16M/200 Random Read                   	3.58 	Kbytes/sec

16M/200 Random Write                  	2.14 	Kbytes/sec

16M-512 Byte Records

16M/512 Sequential Read             	541.33 	Kbytes/sec

16M/512 Sequential Write            	513.95 	Kbytes/sec

16M/512 Random Read                   	9.78 	Kbytes/sec

16M/512 Random Write                 	12.23 	Kbytes/sec

16M-2K Byte Records

16M/2K Sequential Read              	648.37 	Kbytes/sec

16M/2K Sequential Write             	539.74 	Kbytes/sec

16M/2K Random Read                   	37.76 	Kbytes/sec

16M/2K Random Write                  	41.00 	Kbytes/sec

16M-4K Byte Records

16M/4K Sequential Read              	674.37 	Kbytes/sec

16M/4K Sequential Write             	600.00 	Kbytes/sec

16M/4K Random Read                   	74.90 	Kbytes/sec

16M/4K Random Write                  	78.30 	Kbytes/sec

8M File

8M-200 Byte Records

8M/200 Sequential Read              	522.49 	Kbytes/sec

8M/200 Sequential Write              	18.53 	Kbytes/sec

8M/200 Random Read                    	5.46 	Kbytes/sec

8M/200 Random Write                   	2.98 	Kbytes/sec

8M-512 Byte Records

8M/512 Sequential Read              	539.74 	Kbytes/sec

8M/512 Sequential Write             	518.00 	Kbytes/sec

8M/512 Random Read                   	23.88 	Kbytes/sec

8M/512 Random Write                  	25.18 	Kbytes/sec

8M-2K Byte Records

8M/2K Sequential Read               	648.37 	Kbytes/sec

8M/2K Sequential Write              	564.44 	Kbytes/sec

8M/2K Random Read                    	88.57 	Kbytes/sec

8M/2K Random Write                   	98.42 	Kbytes/sec

8M-4K Byte Records

8M/4K Sequential Read               	672.43 	Kbytes/sec

8M/4K Sequential Write              	617.01 	Kbytes/sec

8M/4K Random Read                   	164.74 	Kbytes/sec

8M/4K Random Write                  	185.27 	Kbytes/sec

4M File

4M-200 Byte Records

4M/200 Sequential Read              	522.49 	Kbytes/sec

4M/200 Sequential Write              	18.73 	Kbytes/sec

4M/200 Random Read                   	10.34 	Kbytes/sec

4M/200 Random Write                   	4.44 	Kbytes/sec

4M-512 Byte Records

4M/512 Sequential Read              	537.50 	Kbytes/sec

4M/512 Sequential Write             	519.54 	Kbytes/sec

4M/512 Random Read                   	32.27 	Kbytes/sec

4M/512 Random Write                  	34.04 	Kbytes/sec

4M-2K Byte Records

4M/2K Sequential Read               	644.84 	Kbytes/sec

4M/2K Sequential Write              	567.37 	Kbytes/sec

4M/2K Random Read                   	121.35 	Kbytes/sec

4M/2K Random Write                  	127.60 	Kbytes/sec

4M-4K Byte Records

4M/4K Sequential Read               	667.30 	Kbytes/sec

4M/4K Sequential Write              	617.01 	Kbytes/sec

4M/4K Random Read                   	216.29 	Kbytes/sec

4M/4K Random Write                  	242.13 	Kbytes/sec

2M File

2M-200 Byte Records

2M/200 Sequential Read              	514.83 	Kbytes/sec

2M/200 Sequential Write              	18.78 	Kbytes/sec

2M/200 Random Read                   	11.61 	Kbytes/sec

2M/200 Random Write                   	4.94 	Kbytes/sec

2M-512 Byte Records

2M/512 Sequential Read              	529.03 	Kbytes/sec

2M/512 Sequential Write             	512.44 	Kbytes/sec

2M/512 Random Read                   	34.91 	Kbytes/sec

2M/512 Random Write                  	36.02 	Kbytes/sec

2M-2K Byte Records

2M/2K Sequential Read               	634.45 	Kbytes/sec

2M/2K Sequential Write              	571.97 	Kbytes/sec

2M/2K Random Read                   	126.42 	Kbytes/sec

2M/2K Random Write                  	135.19 	Kbytes/sec

2M-4K Byte Records

2M/4K Sequential Read               	660.37 	Kbytes/sec

2M/4K Sequential Write              	612.69 	Kbytes/sec

2M/4K Random Read                   	222.46 	Kbytes/sec

2M/4K Random Write                  	256.95 	Kbytes/sec

1M File

1M-200 Byte Records

1M/200 Sequential Read              	499.55 	Kbytes/sec

1M/200 Sequential Write              	18.73 	Kbytes/sec

1M/200 Random Read                   	11.98 	Kbytes/sec

1M/200 Random Write                   	5.00 	Kbytes/sec

1M-512 Byte Records

1M/512 Sequential Read              	513.95 	Kbytes/sec

1M/512 Sequential Write             	502.94 	Kbytes/sec

1M/512 Random Read                   	34.04 	Kbytes/sec

1M/512 Random Write                  	35.09 	Kbytes/sec

1M-2K Byte Records

1M/2K Sequential Read               	613.20 	Kbytes/sec

1M/2K Sequential Write              	557.36 	Kbytes/sec

1M/2K Random Read                   	124.37 	Kbytes/sec

1M/2K Random Write                  	135.50 	Kbytes/sec

1M-4K Byte Records

1M/4K Sequential Read               	637.23 	Kbytes/sec

1M/4K Sequential Write              	595.23 	Kbytes/sec

1M/4K Random Read                   	222.46 	Kbytes/sec

1M/4K Random Write                  	247.62 	Kbytes/sec

256K File

256K-200 Byte Records

256K/200 Sequential Read            	431.65 	Kbytes/sec

256K/200 Sequential Write            	18.78 	Kbytes/sec

256K/200 Random Read                 	12.15 	Kbytes/sec

256K/200 Random Write                 	4.73 	Kbytes/sec

256K-512 Byte Records

256K/512 Sequential Read            	436.40 	Kbytes/sec

256K/512 Sequential Write          	440.52 	Kbytes/sec

256K/512 Random Read                 	31.67 	Kbytes/sec

256K/512 Random Write                	30.95 	Kbytes/sec

256K-2K Byte Records

256K/2K Sequential Read             	516.18 	Kbytes/sec

256K/2K Sequential Write            	484.26 	Kbytes/sec

256K/2K Random Read                 	119.76 	Kbytes/sec

256K/2K Random Write                	120.00 	Kbytes/sec

256K-4K Byte Records

256K/4K Sequential Read             	532.85 	Kbytes/sec

256K/4K Sequential Write            	523.78 	Kbytes/sec

256K/4K Random Read                 	214.59 	Kbytes/sec

256K/4K Random Write                	227.53 	Kbytes/sec

E.	LANDMARK  SPEED  2.0

CPU Speed	346.25	MHz

FPU Speed	1014.59	MHz

F.	DIAGSOFT  CHECKIT  3.0

CPU Speed	47474	Dhrystones

Math Speed	19290.6	K-Whetstones

G.	DIAGSOFT  QAPLUS  4.61

CPU Speed	45517	Dhrystones

FPU Speed	18287.2	K-Whetstones

H.	DIAGSOFT  QAPLUS  4.70

CPU Speed	45.5	K-Dhrystones

FPU Speed	17.8	M-Whetstones

I.	NORTON  SYSTEM  INFO  4.5

Computing  Index	127.4

Disk  Index	5.1

Performance  Index	86.6

J.	NORTON  SYSTEM  INFO  7.0

CPU Speed	190.0	MHz

IV.	IMPORTANT  REMINDERS  FOR  THIS  VERSION

1.		Hardware  speed  switching  function  is  not  operational.  (This  is  already  				corrected  in  the  next  version.)

2.		When  EMM386  or  some  memory  managers  are  installed,  software speed  			switching  function  is  disabled.  (This   is  a  known  occurrence  with  the  					Pentium  CPU.)

3.		Jumper  JP15  is  set  as  IRQ11  (level  trigger),  configured  for  use  with

MS-3805.  If  you  are  installing  any  adapter  which  also  makes  use  of   IRQ11,  		set  JP15  to  open  or  change  the  IRQ  number  of  the  adapter.  Install

MS-3805  in  PCI1  slot  only.

4.		In  case  you're  using  MS-3805,  you  can  set  the  following  options  in  the  				the  "BIOS  Features  Setup"  for  a  better  performance:

For  Award  BIOS:

E8000 - EBFFF                 Enabled

For  Phoenix  BIOS:

Option  ROMS	      Shadowing  Only  Enabled

5.		For  256KB  cache,  set  Cache  Write  Policy  for  Write-Back.  For  512KB  				cache,  set  the  Cache  Write  Policy  for  Write-Through.

6.		For  Pentium  60MHz,  you  can  set  the  following  options  in  the  "BIOS  					Chipset  Features  Setup"  for  a  better  performance:

For  Award  BIOS:

CPU  to  PCI  write  buffer                  Enabled

PCI  Memory  Burst  Write                Enabled

7.  	Jumper  JP13  (Clear  RTC  Selector)  is  not  functional  in  Version  1.  					However,  in  	cases  where  you  need  to  clear  the  CMOS,  use  the  Award  				BIOS  	clear  CMOS  function  instead.  Press  and  hold  the  <Ins>  key,  then  				power  on  the  system  and  wait  till  the  system  is  up.  You  can  enter  the  				Award  BIOS  program  as  normal.  (This  is  already  corrected  in  Version  1.1.)

8.		When  using  the  Tekram  DC-690B  PCI  IDE  card,  a  GAL  component  					(G503-41-01)  	in  the  mainboard  must  be  changed.  The  jumper  in  the  					mainboard  	must  be  set  for  level  trigger,  INT#D,  IRQ14.  The  Tekram

DC-690B  must  be  set  to  used  INT#D.

For  MS-5103:    Set  JP15    short 13-14  only

JP7       short 1-2

For  Tekram:        Set  to  use  INT#D

9.		When  using  Award  WM61  BIOS:

A.	In  OS/2  2.1,  running  windows  programs  from  the  windows  programs  				  	icon   	will  cause  the  system  to  hang.  The  WIN-OS/2  session  is  working  				fine.

B.	OS/2  2.1  cannot  be  installed  using  the  High  Performance  File  System.

C.	Do  not  install  any  adapter  which  uses  the  C8000  address.

D.	Do  not  use  Panasonic  MN414400ASJ-7  memory  modules.

E.	If  you  install  two  network  adapters  with  the  first  one  using  IRQ3,  I/O  				port  300  and  the  second  one  using  IRQ2,  I/O  port  320,   the  system  will  				not  boot  up.  (Solution:  Set  the  second  card  to  use  other  I/O  ports.)

10.	When  using  Award  WM62  BIOS:

A.	In  OS/2  2.1,  running  windows  programs  from  the  windows  programs  				  	icon   	will  cause  the  system  to  hang.  The  WIN-OS/2  session  is  working  				fine.

B.	OS/2  2.1  cannot  be  installed  using  the  High  Performance  File  System.

C.	Do  not  install  any  adapter  which  uses  the  C8000  address.

D.	If  you  install  two  network  adapters  with  the  first  one  using  IRQ3,  I/O  				port  300  and  the  second  one  using  IRQ2,  I/O  port  320,   the  system  will  				not  boot  up.  (Solution:  Set  the  second  card  to  use  other  I/O  ports.)

11.	When  using  Phoenix  PM61  BIOS:

Software  speed  switching  is  not  operational.

Note:		These  conditions  are  seen  at  the  time  of  testing.  It  may  not  be  true     			for  	all  versions.  Please  check  with  your  dealers  regularly  for  updates.