MS-5103  PCI  MAINBOARD

TEST  REPORT

DATE:  December  6,  1993

TESTING  CONFIGURATION:

Motherboard		:	MS-5103  PCI  Mainboard

Version			:	Ver  1

BIOS				:	Award  Ver.  WM61

Chipsets			:	Intel  Mercury

System  Memory	:	16MB

Cache  Memory	:	256KB  (write-back)

I/O  Controller		:	NS-PC87311  ISA  IDE  Controller

VGA  Card			:	MS-4401  (ALI  ALG2201.OF)  PCI  with  1MB  DRAM

Hard  Disk  Drive	:	Seagate  ST3243A  HDD  (203MB)

I. 		Performance

CPU  SPEED:	Pentium  60 MHz

LANDMARK  SPEEDCOM  2.0

CPU Speed	346.25	MHz

FPU Speed	1014.59	MHz

PC  LABS  BENCHMARK  7.0

Processor Harmonic	25850.14

Memory Harmonic	6904.76

POWER  METER  1.7

Aggregate CPU/Memory Test	44940.7	PMUs

MIPS Test	41.0	MIPS

Dhrystone Test	52.6	K-Dhrystones/sec

Whetstone Test	11756.6	K-Whetstones/sec

Sieve Test	0.1	sec

POWER  METER  1.81

Aggregate CPU/Memory Test	43539.0	PMUs

MIPS Test	29.3	MIPS

Dhrystone Test	51.5	K-Dhrystones/sec

Whetstone Test	11619.9	K-Whetstones/sec

Sieve Test	0.1	sec

CHECK  IT  3.0

CPU Speed	47474	Dhrystones

Math Speed	19290.6	K-Whetstones

BYTE  1.3  BENCHMARK  TESTS

CPU Index	16.13

FPU Index	101.70

BYTE  2.2  BENCHMARK  TESTS

CPU Index	2.45

FPU Index	9.12

QA  PLUS  4.61

CPU Speed	45517	Dhrystones

FPU Speed	18287.2	K-Whetstones

QA  PLUS  4.70

CPU Speed	45.5	K-Dhrystones

FPU Speed	17.8	M-Whetstones

Norton  System  Info  4.5

Computing  Index	127.4

Disk  Index	5.1

Performance  Index	86.6

Norton  System  Info  7.0

CPU Speed	190.0	MHz

II. 	Hardware  Compatibility

A.		PCI  VGA  Cards

| Vendor | Model No. | Chipset | Remarks |
| --- | --- | --- | --- |
| Micro-Star | MS-4401 | ALG2201 | OK |
| Micro-Star | MS-4403 | AGX016 | OK |
| Leadtek | - | S3 P86C805 | OK |
| FIC | - | S3 P86C928P | OK |
| ChainTech | - | CL GD5428 | OK |

Note:	The  cards  above  were  tested  under  Windows  3.1.

B.		PCI  SCSI  Controller

| Vendor | Model No. | Chipset | Remarks |
| --- | --- | --- | --- |
| Micro-Star | MS-3805 | NCR 53C810PT | OK |

C.		PCI  IDE  Controller

| Chipset | Remarks |
| --- | --- |
| Symphony SL82C101 | OK in Windows, but cannot boot in OS/2 |

III. 	Software  Compatibility

A.		Operating  Systems

MS-DOS  5.0  and  6.0

OS/2  2.0  and  2.1

Windows  N.T.  				-	tested  using  MS-3805  (NCR 53C810PT)  PCI  												SCSI  controller  and  ET4/W32  ISA  VGA  card

SCO  Unix  Rel  3.2.1  and

SCO  Unix  Rel  3.2.4	-	tested  using  Adaptec  1542C  ISA  SCSI  															controller  and  Trident  8900C  ISA  VGA  card

B.		Application  Softwares

AutoCAD  Release  12

AutoDesk  3D  Studio  2.0

Microsoft  Excel  4.0

Microsoft  Windows  3.1

Microsoft  Word  for  Windows  2.0

Microsoft  Works  2.0

C.		Diagnostic  Softwares

Byte  Magazine  Benchmark  1.3  and  2.2

DiagSoft  CheckIt  3.0

DiagSoft  Power  Meter  1.7  and  1.81

DiagSoft  QAPlus  4.61

DiagSoft  QAPlus  4.70

Hercules  Speedy

Landmark  Speedcom  1.14  and  2.0

Norton  Diagnostics  7.0

Norton  System  Info  4.5  and  7.0

PC  Magazine  Labs  Benchmark  7.0

PC  Magazine  Labs  WinBench  3.11  and  4.0

WinTach  1.0  and  1.2

IV.	Important  Reminders

1.		Hardware  speed  switching  function  is  not  operational.  (This is  					already  corrected  in  the  next  version.)

2.		When  EMM386  or  some  memory  managers  are  installed,  software  			speed  switching  function  is  disabled.  (This   is  a  known  occurrence  			with  the  Pentium  CPU.)

3.		Jumper  JP15  is  set  as  IRQ11  (level  trigger),  configured  for  use  with  		MS-3805.  If  you  are  installing  any  adapter  which  also  makes  use  of  			IRQ11,  set  JP15  to  open  or  change  the  IRQ  number  of  the  					adapter.  Install  MS-3805  in  PCI1  slot  only.

4.		In  case  you're  using  MS-3805,  you  can  set  the  following  options  in  			the  "BIOS  Features  Setup"  for  a  better  performance:

E8000 - EBFFF								Enabled

5.		For  256KB  cache,  set  Cache  Write  Policy  for  Write-Back.  For  					512KB  cache,  set  the  Cache  Write  Policy  for  Write-Through.

6.		For  Pentium  60MHz,  you  can  set  the  following  options  in  the  "BIOS  		Chipset  Features  Setup"  for  a  better  performance:

CPU  to  PCI  write  buffer			Enabled

PCI  Memory  Burst  Write			Enabled

7.		When  using  the  Tekram  DC-690B  PCI  IDE  card,  a  GAL  component  			in  the  mainboard  must  be  changed.  The  jumper  in  the  mainboard  				must  be  set  for  level  trigger,  INT#D,  IRQ14.  The  Tekram  DC-690B  			must  be  set  to  used  INT#D.

For  MS-5103:	Set  	JP15		short 13-14  only

JP7		short 1-2

For  Tekram:		Set  to  use  INT#D