MICRO-STAR INTERNATIONAL CO., LTD.

7F, NO. 4, LANE 497, CHUNG CHENG RD.HSIN TIEN CITY, TAIPEI HSIEN, TAIWAN, R.O.C.TEL:  886-2-218-2292 (REP)FAX:  886-2-218-5552, 218-3361, 218-6049µ §

Date:  May 5, 1994

MS-5103  PCI / ISA  MAINBOARD

SUPPLEMENTS  (Rel. 1)

MAINBOARD	MS-5103

CHIPSET        	Intel  Mercury

Please find below the proposed jumper and BIOS settings for different configurations.  Use default values and settings for the other jumpers and BIOS not specified.

MS-3805  is  the  NCR  PCI  SCSI  Controller.

MS-4803  is  the  ALI  PCI  IDE  Controller.

CASE I:		MS-5103 + MS-3805

For MS-5103 Version 1.0, 1.1, 1.2  and 2.0 Mainboards:

1.		For  MS-5103:

Set  JP4  to  1-2.

Set  JP8  to  1-2.

Set  JP15  to  IRQ11.

2.		For  MS-3805:

Set  JP2  to  1-2.

Install  MS-3805  in  PCI  Slot  1.

3.		For  Award  BIOS:

Onboard  SCSI  Controller	:	Enabled

NCR  Latency  Timer			:	96 PCI  Clock

NCR  Using  IRQ				:	11

4.		For  Phoenix  BIOS:

Device  Select				:	NCR SCSI

Latency  Timer			:	Default

Enable  Device			:	Enabled

Enable  Master			:	Enabled

NCR  SCSI  IRQ		:	11

CASE II:		MS-5103 + MS-4803

For MS-5103 Version 1.0, 1.1 and 1.2 Mainboards:

1.		For  MS-5103:

Set  JP7  to  2-3.

Leave  JP15  open.

2.		For  MS-4803:

Set  JPIDE1,  short  INTD  1-2.

Install  MS-4803  in  PCI  Slot  2.

3.		For  Award  BIOS:

Slot  2  Location			:	CDXX

Latency  Timer	:	0  PCI  Clock

Using  IRQ		:	14

4.		For  Phoenix  BIOS:

Device  Select				:	PCI  Slot  2

Latency  Timer			:	Default

Enable  Device			:	Enabled

Enable  Master			:	Disabled

PCI  Slot  2  IRQ		:	14

For MS-5103 Version 2.0 Mainboards:

1.		For  MS-5103:

Set  JP5  to  2-3.

Leave  JP15  open.

2.		For  MS-4803:

Set  JPIDE1,  short  INTA  1-2.

Install  MS-4803  in  the  PCI  Slot  2.

3.		For  Award  BIOS:

Slot  2  Location			:	CDXX

Latency  Timer	:	0  PCI  Clock

Using  IRQ		:	14

4.		For  Phoenix  BIOS:

Device  Select				:	PCI  Slot  2

Latency  Timer			:	Default

Enable  Device			:	Enabled

Enable  Master			:	Disabled

PCI  Slot  2  IRQ		:	14

CASE III:		MS-5103 + MS-3805 + MS-4803

For MS-5103 Version 1.0, 1.1 and 1.2 Mainboards:

1.		For  MS-5103:

Set  JP7  to  2-3.

Set  JP4  to  1-2.

Set  JP8  to  1-2.

Set  JP15  to  IRQ11.

2.		For  MS-3805:

Set  JP2  to  1-2.

Install  MS-3805  in  PCI  Slot  1.

3.		For  MS-4803:

Set  JPIDE1,  short  INTD  1-2.

Install  MS-4803  in  PCI  Slot  2.

4.		For  Award  BIOS:

Onboard  SCSI  Controller	:	Enabled

NCR  Latency  Timer			:	96 PCI  Clock

NCR  Using  IRQ				:	11

Slot  2  Location					:	CDXX

Latency  Timer			:	0  PCI  Clock

Using  IRQ				:	14

5.		For  Phoenix  BIOS:

Device  Select			 			:	NCR SCSI

Latency  Timer					:	Default

Enable  Device					:	Enabled

Enable  Master					:	Enabled

NCR  SCSI  IRQ				:	11

Device  Select						:	PCI  Slot  2

Latency  Timer					:	Default

Enable  Device					:	Enabled

Enable  Master					:	Disabled

PCI  Slot  2  IRQ				:	14

For MS-5103 Version 2.0 Mainboards:

1.		For  MS-5103:

Set  JP5  to  2-3.

Set  JP4  to  1-2.

Set  JP8  to  1-2.

Set  JP15  to  IRQ11.

2.		For  MS-3805:

Set  JP2  to  1-2.

Install  MS-3805  in  PCI  Slot  1.

3.		For  MS-4803:

Set  JPIDE1,  short  INTA  1-2.

Install  MS-4803  in  the  PCI  Slot  2.

4.		For  Award  BIOS:

Onboard  SCSI  Controller	:	Enabled

NCR  Latency  Timer			:	96 PCI  Clock

NCR  Using  IRQ				:	11

Slot  2  Location		  			:	CDXX

Latency  Timer			:	0  PCI  Clock

Using  IRQ				:	14

5.		For Phoenix BIOS:

Device  Select						:	NCR SCSI

Latency  Timer					:	Default

Enable  Device					:	Enabled

Enable  Master					:	Enabled

NCR  SCSI  IRQ				:	11

Device  Select						:	PCI  Slot  2

Latency  Timer					:	Default

Enable  Device					:	Enabled

Enable  Master					:	Disabled

PCI  Slot  2  IRQ				:	14