MICRO-STAR INTERNATIONAL CO., LTD.

7F, NO. 4, LANE 497, CHUNG CHENG RD.HSIN TIEN CITY, TAIPEI HSIEN, TAIWAN, R.O.C.TEL:  886-2-218-2292 (REP)FAX:  886-2-218-5552, 218-3361, 218-6049µ §

January 31, 1994

MS-5103  PCI / ISA  MAINBOARD

µ §

NEWS  UPDATES

MAINBOARD	MS-5103

CHIPSET        	Intel  Mercury

IMPORTANT  REMINDERS:

For All Versions of Mainboard:

1.		When  EMM386  or  some  memory  managers  are  installed,  software speed  			switching  function  is  disabled.  This   is  a  known  occurrence  with  the  Pentium  		CPU.

For Version 1 Mainboard:

1.		Hardware  speed  switching  function  is  not  operational.  (This  is  already  				corrected  in  Version  2  and  later.)

2.		Jumper  JP15  is  set  as  IRQ11  (level  trigger),  configured  for  use  with

MS-3805  (NCR  SCSI  Controller).  If  you  are  installing  any  adapter  which  			also  makes  use  of   IRQ11, set  JP15  to  open  or  change  the  IRQ  number  			of  the  adapter.  Install  MS-3805  in  PCI1  slot  only.

3.  	Jumper  JP13  (Clear  RTC  Selector)  is  not  functional.  However,  in  cases  			where  you  need  to  clear  the  CMOS,  use  the  Award  BIOS  clear  CMOS  				function  instead.  Press  and  hold  the  <Ins>  key,  then  power  on  the  system  			and  wait  till  the  system  is  up.  You  can  enter  the  	Award  BIOS  program  as  			normal.  (This  is  already  corrected  in  Version  1.1  and  later.)

4.		When  using  the  Tekram  DC-690B  PCI  IDE  card,  a  GAL  component  					(G503-41-01)  	in  the  mainboard  must  be  changed.  The  jumper  in  the  					mainboard  	must  be  set  for  level  trigger,  INT#D,  IRQ14.  The  Tekram

DC-690B  must  be  set  to  used  INT#D.

For  MS-5103:  	Set	JP15	short 13-14  only

JP7	short 1-2

For  Tekram:   	Set  to  use  INT#D

For Version 1.1 Mainboard:

1.		Hardware  speed  switching  function  is  not  operational.  (This  is  already  				corrected  in  Version  2  and  later.)

2.		Jumper  JP15  is  set  as  IRQ11  (level  trigger),  configured  for  use  with

MS-3805  (NCR  SCSI  Controller).  If  you  are  installing  any  adapter  which  			also  makes  use  of   IRQ11,  set  JP15  to  open  or  change  the  IRQ  number  			of  the  adapter.  Install  MS-3805  in  PCI1  slot  only.

3.		When  using  the  Tekram  DC-690B  PCI  IDE  card,  a  GAL  component  					(G503-41-01)  	in  the  mainboard  must  be  changed.  The  jumper  in  the  					mainboard  	must  be  set  for  level  trigger,  INT#D,  IRQ14.  The  Tekram

DC-690B  must  be  set  to  used  INT#D.

For  MS-5103:  	Set	JP15	short 13-14  only

JP7	short 1-2

For  Tekram:   	Set  to  use  INT#D

For Version 1.2 Mainboard:

1.		Hardware  speed  switching  function  is  not  operational.  (This  is  already  				corrected  in  Version  2  and  later.)

2.		Jumper  JP15  is  set  as  IRQ11  (level  trigger),  configured  for  use  with

MS-3805  (NCR  SCSI  Controller).  If  you  are  installing  any  adapter  which  			also  makes  use  of   IRQ11,  set  JP15  to  open  or  change  the  IRQ  number  			of  the  adapter.  Install  MS-3805  in  PCI1  slot  only.

3.		When  using  the  Tekram  DC-690B  PCI  IDE  card,  a  GAL  component  					(G503-41-01)  	in  the  mainboard  must  be  changed.  The  jumper  in  the  					mainboard  	must  be  set  for  level  trigger,  INT#D,  IRQ14.  The  Tekram

DC-690B  must  be  set  to  used  INT#D.

For  MS-5103:  	Set	JP15	short 13-14  only

JP7	short 1-2

For  Tekram:   	Set  to  use  INT#D

4.		A  CPU  fan  (cooler)  connector  is  added  for  12V  or  5V  CPU  fan  (cooler).

For Version 2 Mainboard:

1.		The  12V  Flash  Memory  is  used  for  the  System  BIOS.  A  utility  program,  				FLASH12V.EXE  is  provided  for  future  BIOS  upgrade.

2.		The  64K*8  SRAM  is  already  supported  in  this  version.  With  this,  you  can  			use  one  bank  (8  pieces)  of  64K*8  to  make  up  512K  cache  memory.

3.		A  CPU  fan  (cooler)  connector  is  added  for  12V  or  5V  CPU  fan  (cooler).

4.		Interrupt  pins  on  PCI  are  optional  and  defined  as  "level sensitive,"  asserted 			low  (negative  true),  using  open  drain  output  drivers.  However,  even  though 			the  PCI  specifications  define  interrupts  as  level  sensitive,  many  add-on  cards 			vendors  use  edge  sensitive  interrupts  in  their  card  designs.  Two  methods  of 			producing  edge  sensitive  interrupt  are  currently  defined:

Method 1:	Rising  Edge  Sensitive  Interrupt

This  method  uses  INTx#  rising  edge  to  request  an  interrupt.  This  is  the  				same  as  IRQx  of  the  ISA  specifications,  and  can  be  directly  connected  to  			the IRQx.

Method 2:	Falling  Edge  Sensitive  Interrupt

This  method  uses  INTx#  falling  edge  to  request  an  interrupt.  This  method  			of 	  interrupt  request  should  first  let  the  INTx#  pass  through  an  inverter, 				for  the  interrupt  controller  to  recognize.

In  order  to  recognize  add-on  cards  which  do  not  conform  to  the  PCI  					interrupt specifications,  modifications  were  made  to  our  mainboard.  Now,  all  			three  types  of  interrupts  can  be  used  in  this  version:  level  trigger,  edge  				trigger  (falling)  and  edge  trigger  (rising).

5.		A  GAL  component  in  the  mainboard  is  changed  to  G503-41-02.

6.		When  using  MS-3805  (NCR  SCSI  Controller)  for  edge  trigger,  set  the  				following  jumpers:

For  MS-5103:  	Set	JP8		short 1-2

JP19		IRQ11  short 1-2  (for  G1  group)

JP4 (G1)	short 2-3  (edge  trigger,  rising)

Install  in  PCI1  slot  only.

For  MS-3805:  	Set    	JP2	short 2-3  (edge  trigger,  INT#A)

7.		When  using  MS-3805  (NCR  SCSI  Controller)  for  level  trigger,  set  the  				following  jumpers:

For  MS-5103:  	Set	JP8		short 1-2

JP15		short IRQ11

JP4 (G1)	short 1-2  (level  trigger)

Install  in  PCI1  slot  only.

For  MS-3805:  	Set    	JP2	short 1-2  (level  trigger,  INT#A)

8.		When  using  the  Tekram  DC-690B  PCI  IDE  card,  set  the  following  jumpers:

For  MS-5103:  	Set	JP19		IRQ14  short 2-3  (for  G2  group)

JP5 (G2)	short 3-4  (edge  trigger,  falling)

Install  in  PCI2  slot  (for  this  setting  only).

For  Tekram:  	Set  to  use  INT#A  (for  G2  in  PCI2  slot).

For Award BIOS WM61:

1.		In  OS/2  2.1,  running  Windows  programs  from  the  Windows  programs icon 			will  cause  the  system  to  hang.  Use  the  WIN-OS/2  session  instead  to run 				Windows programs.

2.		OS/2  2.1  cannot  be  installed  using  the  High  Performance  File  System.  Use  			the  FAT  instead.

3.		Do  not  install  any  adapter  which  uses  the  C8000  address.

4.		Do  not  use  Panasonic  MN414400ASJ-7  memory  modules.

5.		If  you  install  two  network  adapters  with  the  first  one  using  IRQ3,  I/O  port   		300  and  the  second  one  using  IRQ2,  I/O  port  320,   the  system  will  not  				boot  up.  (Solution:  Set  the  second  card  to  use  other  I/O  ports.)

6.		For  256KB  cache,  set  Cache  Write  Policy  for  Write-Back.  For  512KB  				cache,  set  the  Cache  Write  Policy  for  Write-Through.

7.		For  Pentium  60MHz,  you  can  set  the  following  options  in  the  "BIOS  					Chipset  Features  Setup"  for  a  better  performance:

CPU  to  PCI  write  buffer              	Enabled

PCI  Memory  Burst  Write                	Enabled

*  This  might  not  work  for  some  configurations.

8.		In  case  you're  using  MS-3805,  you  can  set  the  following  options  in  the  				the  "BIOS  Features  Setup"  for  a  better  performance:

E8000 - EBFFF                 	Enabled

*  This  might  not  work  for  some  configurations.

For Award BIOS WM62:

1.		In  OS/2  2.1,  running  Windows  programs  from  the  Windows  programs icon 			will  cause  the  system  to  hang.  Use  the  WIN-OS/2  session  instead  to run 				Windows programs.

2.		OS/2  2.1  cannot  be  installed  using  the  High  Performance  File  System.  Use  			the  FAT  instead.

3.		Do  not  install  any  adapter  which  uses  the  C8000  address.

4.		If  you  install  two  network  adapters  with  the  first  one  using  IRQ3,  I/O  port   		300  and  the  second  one  using  IRQ2,  I/O  port  320,   the  system  will  not  				boot  up.  (Solution:  Set  the  second  card  to  use  other  I/O  ports.)

5.		For  256KB  cache,  set  Cache  Write  Policy  for  Write-Back.  For  512KB  				cache,  set  the  Cache  Write  Policy  for  Write-Through.

6.		For  Pentium  60MHz,  you  can  set  the  following  options  in  the  "BIOS  					Chipset  Features  Setup"  for  a  better  performance:

CPU  to  PCI  write  buffer                  	Enabled

PCI  Memory  Burst  Write                	Enabled

*  This  might  not  work  for  some  configurations.

7.		In  case  you're  using  MS-3805  (NCR  SCSI  Controller),  you  can  set  the  				following  options  in  the  	the  "BIOS  Features  Setup"  for  a  better  performance:

E8000 - EBFFF                 	Enabled

*  This  might  not  work  for  some  configurations.

For Award BIOS WM63:

1.		In  OS/2  2.1,  running  Windows  programs  from  the  Windows  programs icon 			will  cause  the  system  to  hang.  Use  the  WIN-OS/2  session  instead  to run 				Windows programs.

2.		OS/2  2.1  cannot  be  installed  using  the  High  Performance  File  System.  Use  			the  FAT  instead.

3.		For  256KB  cache,  set  Cache  Write  Policy  for  Write-Back.  For  512KB  				cache,  set  the  Cache  Write  Policy  for  Write-Through.

4.		For  Pentium  60MHz,  you  can  set  the  following  options  in  the  "BIOS  					Chipset  Features  Setup"  for  a  better  performance:

CPU  to  PCI  write  buffer                  	Enabled

PCI  Memory  Burst  Write                	Enabled

*  This  might  not  work  for  some  configurations.

5.		In  case  you're  using  MS-3805  (NCR  SCSI  Controller),  you  can  set  the  				following  options  in  the  	the  "BIOS  Features  Setup"  for  a  better  performance:

E8000 - EBFFF                 	Enabled

*  This  might  not  work  for  some  configurations.

For Phoenix BIOS PM61:

1.		Software  speed  switching  is  not  operational.

2.		In  case  you're  using  MS-3805 (NCR  SCSI  Controller),  you  can  set  the  				following  options  in  the  the  "BIOS  Features  Setup"  for  a  better  performance:

Option  ROMS	Shadowing  Only  Enabled

*  This  might  not  work  for  some  configurations.

3.		In  case  you're  using  MS-3805 (NCR  SCSI  Controller),  do  not  install  					EMM386.EXE  when  DOSCAM.SYS  and  SCSIDISK.SYS  are  installed.

F		Note:     Check  with  your  dealers  regularly  for  updates.