MICRO-STAR INTERNATIONAL CO., LTD.

7F, NO. 4, LANE 497, CHUNG CHENG RD.HSIN TIEN CITY, TAIPEI HSIEN, TAIWAN, R.O.C.TEL:  886-2-218-2292 (REP)FAX:  886-2-218-5552, 218-3361, 218-6049µ §

Date:  May 3, 1994

MS-5103  PCI / ISA  MAINBOARD

µ §

NEWS  UPDATES  (Rel. 2)

MAINBOARD	MS-5103

CHIPSET        	Intel  Mercury

BIOS  UPDATES:

For Award BIOS WM64:

1.		In  OS/2  2.1,  running  Windows  programs  from  the  Windows  programs icon 			will  cause  the  system  to  hang.  Use  the  WIN-OS/2  session  instead  to run 				Windows programs.

2.		OS/2  2.1  cannot  be  installed  using  the  High  Performance  File  System.  Use  			the  FAT  instead.

3.		For  256KB  cache,  set  Cache  Write  Policy  for  Write-Back.  For  512KB  				cache,  set  the  Cache  Write  Policy  for  Write-Through.

4.		For  Pentium  60MHz,  you  can  set  the  following  options  in  the  "BIOS  					Chipset  Features  Setup"  for  a  better  performance:

CPU  to  PCI  write  buffer                  	Enabled

PCI  Memory  Burst  Write                	Enabled

*  This  might  not  work  for  some  configurations.

5.		The  Memory  Parity  Check  option  in  the  "BIOS  	Chipset  Features  Setup"  			has  been  hidden  and  cannot  be  set.

6.		In  the  "BIOS  PCI  Slot  Configuration", the  following  default  values  are loaded:

PCI  1  Slot                  	IRQ11

PCI  2  Slot                  	IRQ14

PCI  3  Slot                  	IRQ5

PCI  4  Slot                  	IRQ9

For Award BIOS WM65:

*  This  BIOS  was  not  formally  released.  This  was  used  for  internal  testing  only.

For Award BIOS WM66:

1.		In  OS/2  2.1,  running  Windows  programs  from  the  Windows  programs icon 			will  cause  the  system  to  hang.  Use  the  WIN-OS/2  session  instead  to run 				Windows programs.

2.		OS/2  2.1  cannot  be  installed  using  the  High  Performance  File  System.  Use  			the  FAT  instead.

3.		For  256KB  cache,  set  Cache  Write  Policy  for  Write-Back.  For  512KB  				cache,  set  the  Cache  Write  Policy  for  Write-Through.

4.		For  Pentium  60MHz,  you  can  set  the  following  options  in  the  "BIOS  					Chipset  Features  Setup"  for  a  better  performance:

CPU  to  PCI  write  buffer                  	Enabled

PCI  Memory  Burst  Write                	Enabled

*  This  might  not  work  for  some  configurations.

5.		The  Memory  Parity  Check  option  in  the  "BIOS  	Chipset  Features  Setup"  			has  been  hidden  and  cannot  be  set.

6.		In  the  "BIOS  PCI  Slot  Configuration", the  following  default  values  are loaded:

PCI  1  Slot                  	IRQ11

PCI  2  Slot                  	IRQ14

PCI  3  Slot                  	IRQ5

PCI  4  Slot                  	IRQ9

7.		The  new  NCR  BIOS  (Ver  3.04.00)  is  used.  The  old  NCR  BIOS  is  3.01.04.

8.		The  Adaptec  PCI  SCSI  Controller  is  now  supported.

9.		The  problem  when  running  Diamond  Viper  PCI  VGA  card  in  Windows  NT  			with  drivers  installed  is  solved.

IMPORTANT  REMINDERS  WHEN  USING  MS-4803  (ALI  PCI  IDE  CONTROLLER):

For Version 1.0, 1.1 and 1.2 Mainboards:

Step # 1:		Set  MS-4803  to  use  edge  trigger,  INTD.

F To do:		Set  JPIDE1,  short  INTD  1-2.

Step # 2:		Set  MS-5103  to  use  IRQ14,  INTD,  edge  trigger.

F To do:		Set  JP7  to  2-3.

Step # 3:		Install  MS-4803  in  the  MS-5103.

F To do:		Install  in  PCI  Slot  2.

For Version 2.0 Mainboards:

Step # 1:		Set  MS-4803  to  use  edge  trigger,  INTA.

F To do:		Set  JPIDE1,  short  INTA  1-2.

Step # 2:		Set  MS-5103  to  use  G2,  IRQ14,  INTA,  edge  trigger.

F To do:		Set  JP5  to  2-3.

Step # 3:		Install  MS-4803  in  the  MS-5103.

F To do:		Install  in  PCI  Slot  2.

For Award BIOS:

In  the  "BIOS  PCI  Slot  Configuration", set  the  following:

Slot 2 Location	:	CDXX

Latency Timer	:	0  PCI  Clock

Using  IRQ	:	14

For Phoenix BIOS:

In  the  "PCI  Device  Configuration", set  the  following:

Device  Select	:	PCI  Slot  2

Latency Timer	:	Default

Enable  Device	:	Enabled

Enable  Master	:	Disabled

PCI  Slot  2  IRQ	:	14

F		Note:     Check  with  your  dealers  regularly  for  updates.