MICRO-STAR INTERNATIONAL CO., LTD.

7F, NO. 4, LANE 497, CHUNG CHENG RD.HSIN TIEN CITY, TAIPEI HSIEN, TAIWAN, R.O.C.TEL:  886-2-218-2292 (REP)FAX:  886-2-218-5552, 218-3361, 218-6049µ §

MS-5106  P54C  PCI / ISA  MAINBOARD

TEST  REPORT

Date:  August  1,  1994

MAINBOARD	MS-5106

CHIPSET	Intel  Mercury

I.		SOFTWARE COMPATIBILITY TEST

A.  DOS

| MS-DOS | VER 5.0 | VER 6.0 | VER 6.2 | REMARKS |
| --- | --- | --- | --- | --- |
| INSTALL | OK | OK | OK |  |
| BOOT | OK | OK | OK |  |
| FORMAT A: | OK | OK | OK |  |
| BOOT FROM A | OK | OK | OK |  |
| COPY | OK | OK | OK |  |
| COMP | OK | OK | OK |  |
| XCOPY | OK | OK | OK |  |
| DISKCOPY | OK | OK | OK |  |
| DISKCOMP | OK | OK | OK |  |
| M.C.R. DIR | OK | OK | OK |  |
| EMM386 | OK | OK | OK |  |

B.  IBM  OS/2

Configuration:

·	Intel P54C-90 CPU

·	8MB Memory (Micron Tech MT4C4001JDJ-7)

·	256KB Cache (Winbond W24257AK-15)

·	MS-4803 ALI PCI IDE Controller

·	MS-4402 Avance Logic ALG2301A PCI VGA Card

·	Winbond Multi-I/O Controller

·	Conner CP30254 (254MB) Hard Disk

·	One 1.44MB and one 1.2MB FDDs

·	Genius Mouse

| OS/2 | VER 2.0 | VER 2.1 | REMARKS |
| --- | --- | --- | --- |
| INSTALL | OK | OK |  |
| BOOT | OK | OK |  |
| FORMAT A: | OK | OK |  |
| COPY | OK | OK |  |
| COMP | OK | OK |  |
| XCOPY | OK | OK |  |
| DISKCOPY | OK | OK |  |
| DISKCOMP | OK | OK |  |
| M.C.R. DIR | OK | OK |  |
| TASK TEST | OK | OK |  |
| WIN OS/2 | OK | OK |  |

C.  SCO  UNIX

Configuration:

·	Intel P54C-90 CPU

·	16MB Memory (Micron MT4C4001JDJ-7)

·	256KB Cache (Winbond W24257AK-15)

·	Adaptec AHA-1542CF SCSI Controller

·	MS-4402 Avance Logic ALG2301A PCI VGA Card

·	Wangtek 5525ES Tape Drive

·	MAxtor LXT-340S (340MB) SCSI Hard Disk

·	One 1.2MB FDD

·	Genius Mouse

| SCO UNIX | REL 3.2.4 | REMARKS |
| --- | --- | --- |
| INSTALL | OK |  |
| BOOT | OK |  |
| O.D.T. | OK |  |
| DOS MODE | OK |  |
| CP FILE | OK |  |
| TASK TEST | OK |  |

D.  WINDOWS  N.T.

Configuration:

·	Intel P54C-90 CPU

·	16MB Memory (Micron MT4C4001JDJ-7)

·	256KB Cache (Winbond W24257AK-15)

·	MS-3805 NCR PCI SCSI Controller

·	MS-4402 Avance Logic ALG2301 PCI VGA Card

·	UMC Multi-I/O Controller

·	Maxtor LXT340SY (340MB) SCSI Hard Disk

·	NEC Multi-Spin 3XE CD-ROM

·	One 1.44MB and one 1.2MB FDDs

·	Logitech Mouse

| WINDOWS N.T. | VER 3.1 | REMARKS |
| --- | --- | --- |
| INSTALL | OK |  |
| BOOT | OK |  |
| FORMAT A: | OK |  |
| COMP | OK |  |
| XCOPY | OK |  |
| DISKCOPY | OK |  |
| DISKCOMP | OK |  |
| TASK TEST | OK |  |
| HCT | OK |  |

E.  NOVELL  NETWARE

Configuration:

·	Intel P54C-90 CPU

·	8MB Memory (Micron MT4C4001JDJ-7)

·	256KB Cache (Winbond W24257AK-15)

·	MS-4803 ALI PCI IDE Controller

·	MS-4402 Avance Logic ALG2301 PCI VGA Card

·	Conner CP3044 (40MB) Hard Disk

| NETWARE | 386 V3.11 | REMARKS |
| --- | --- | --- |
| INSTALL | OK |  |
| BOOT | OK |  |
| 3COM | OK | File Server and Workstation |
| ARCNET | OK | File Server and Workstation |
| NE2000 | OK | File Server and Workstation |
| NE2100 | OK | File Server and Workstation |
| BOOT ROM | OK |  |
| FILE TRANSFER | OK |  |
| NWTEST | OK |  |

F.  PACKETS  AND  UTILITIES

| S O F T W A R E | REMARKS |
| --- | --- |
| AutoCAD R10 c10 | OK |
| AutoCAD R11 | OK |
| AutoCAD R12 | OK |
| Central Point Anti-Virus 1.4 | OK |
| Clipper 5.01 | OK |
| CorelDraw V3.0 | OK |
| DBase III Plus | OK |
| DBase 4 | OK |
| GeoWorks Ensemble V1.2 | OK |
| Harvard Graphics V2.12 | OK |
| Lotus 1-2-3 R2.4 | OK |
| Lotus 1-2-3 R3.1+ | OK |
| Microsoft Entertainment Pack Vol 1 | OK |
| Microsoft Entertainment Pack Vol 2 | OK |
| Microsoft Entertainment Pack Vol 3 | OK |
| Microsoft Excel V4.0 | OK |
| Microsoft PowerPoint for Windows V3.0 | OK |
| Microsoft Visual Basic V2.0 | OK |
| Microsoft Windows 3.1 | OK |
| Microsoft Word for Windows V2.0 | OK |
| Microsoft Works for Windows V2.0 | OK |
| Norton Desktop for Windows V1.2 | OK |
| Norton Utilities V6.0 / V7.0 | OK |
| PC Tools V8.0 | OK |
| PE 2 | OK |
| QEMM 386 V5.11 | OK |
| QEMM 386 V6.0 | OK |
| QEMM 386 V7.01 | OK |
| QEMM 386 V7.02 | OK |
| Symantec Entertainment Pack for Windows | OK |
| TopDOS 3.00 | OK |
| Ventura Publisher V2.0 | OK |
| WordPerfect 5.1 | OK |
| XTree Gold V2.51 | OK |

G.  DIAGNOSTIC  PROGRAMS

| S O F T W A R E | REMARKS |
| --- | --- |
| '92 System Mark | OK |
| '93 System Mark | OK |
| Byte Benchmark V1.3 | OK |
| Byte Benchmark V2.2 | OK |
| CheckIt V3.0 | OK |
| CheckIt Pro | OK |
| Core Test V2.92 | OK |
| IBM Diagnostics V2.03 | OK |
| Landmark Speed Test V1.14 | OK |
| Landmark Speed Test V2.0 | OK |
| Microsoft Software Diagnostics (MSD) | OK |
| Norton Diagnostics V7.0 | OK |
| Norton System Information V 4.5 | OK |
| Norton System Information V7.0 | OK |
| PC Magazine Labs Benchmark 6.0 | OK |
| PC Magazine Labs Benchmark 7.0 | OK |
| PC Week Labs System Benchmarks V1.10 | OK |
| Power Meter V1.7 | OK |
| Power Meter V1.81 | OK |
| QA Plus V4.61 | OK |
| QA Plus V4.70 | OK |
| QA Plus/FE V5.03 | OK |
| QA Plus/FE V5.04 | OK |
| QA Plus/FE V5.12 | OK |
| Speedy | OK |
| Superscale 3D Benchmark | OK |
| WinBench V2.5 | OK |
| WinBench V3.11 | OK |
| WinBench V4.0 | OK |
| WindSock 3.1 | OK |
| WinStone 94 | OK |
| WinTach V1.0 | OK |
| WinTach V1.2 | OK |

II.		HARDWARE COMPATIBILITY TEST

A.	VIDEO  CARDS

| MANUFACTURER & MODEL NO. | CHIPSET | REMARKS |
| --- | --- | --- |
| CIRRUS LOGIC | GD-5422 | ISA |
| OAK | OTI-067 | ISA |
| OAK | OTI-077 | ISA |
| OAK | OTI-087 | ISA |
| OAK | OTI-087X | ISA |
| TRIDENT | TVGA8900C | ISA |
| TRIDENT | TVGA9000B | ISA |
| TSENG LABS | ET4000/W32 | ISA |
| WD | WD90C30 | ISA |
| ATI MACH 32 | MACH 32 | PCI |
| ATI MACH 64 | MACH 64 | PCI |
| CARDEX | ET4000/W32P | PCI |
| DIAMOND SPEEDSTAR 64 | CIRRUS LOGIC 5434 | PCI |
| DIAMOND STEALTH 32 | ET4000/W32P | PCI |
| DIAMOND VIPER PCI | WEITEK P9000/P9001 | PCI |
| ELSA WINNER 1000PRO | S3 864 | PCI |
| GENOA PHANTOM 32I | ET4000/W32P | PCI |
| INTEL SPEA V7 MERCURY LITE | S3 928 | PCI |
| MATROX MGA | MATROX MGA | PCI |
| MICRO-STAR MS-4401 | ALI ALG2201 | PCI |
| MICRO-STAR MS-4402 | ALI ALG2301 | PCI |
| MICRO-STAR MS-4403 | IIT AGX-016 | PCI |
| MICRO-STAR MS-4405 | ET4000/W32P | PCI |
| ORCHID KELVIN 64 | CIRRUS LOGIC 5434 | PCI |
| PROLINK | CIRRUS LOGIC 5434 | PCI |
| PROLINK | S3 928 | PCI |
| TNC 2 THE MAX | ET4000/W32P | PCI |

B.	I/O  CONTROLLERS

| MANUFACTURER & MODEL NO. | CHIPSET | REMARKS |
| --- | --- | --- |
| GOLDSTAR | PRIME 2 | ISA |
| MICRO-STAR MS-3806 | SMC FDC37C665 (666) | ISA |
| MICRO-STAR MS-3807 | WINBOND W83757 / 58 | ISA |
| UMC | UM82C863F | ISA |
| CMD TECHNOLOGY |  | PCI |
| DTC TECHNOLOGY | DTC 2130 | PCI |
| MICRO-STAR MS-4803 | ALI M5215 | PCI |
| TEKRAM DC-690B |  | PCI |

C.	SCSI CONTROLLERS

| MANUFACTURER | MODEL NO. | REMARKS |
| --- | --- | --- |
| ADAPTEC | AHA-1542C | ISA |
| ADAPTEC | AHA-1542CF | ISA |
| ADAPTEC | AHA-2940 | PCI |
| BUSLOGIC | BT-946C | PCI |
| FUTURE DOMAIN | TMC-3260 | PCI |
| MICRO-STAR MS-3805 | NCR | PCI |

D.	NETWORK  ADAPTERS

| MANUFACTURER | TYPE | CHIPSET | REMARKS |
| --- | --- | --- | --- |
| 3COM | 3COM | 3C509 | ISA |
| SMC | ARCNET | COM90C65LJ | ISA |
|  | NE-2000 |  | ISA |
|  | NE-2100 |  | ISA |
| AMD |  | PCNET | PCI |
| DEC |  | 21040 | PCI |

E.	FLOPPY  DISK  DRIVES

| MANUFACTURER | MODEL NUMBER | CAPACITY |
| --- | --- | --- |
| MITSUBISHI | MF504C-315MG | 1.2 MB |
| MITSUBISHI | MF355C-255MG | 1.44 MB |
| TEAC | FD235HF | 1.44 MB |

F.	HARD  DISK  DRIVES

| MANUFACTURER | MODEL NUMBER | CAPACITY | REMARKS |
| --- | --- | --- | --- |
| CONNER | CP3204F | 204 MB | IDE |
| CONNER | CP30254 | 254 MB | IDE |
| CONNER | CFS420A | 420 MB | IDE |
| CONNER | CFS540A | 540 MB | IDE |
| QUANTUM | 240AT | 240 MB | IDE |
| QUANTUM | 540AT | 540 MB | IDE |
| SEAGATE | ST3243A | 214 MB | IDE |
| WD | CAVIAR 2250 | 250 MB | IDE |
| WD | CAVIAR 2420 | 420 MB | IDE |
| WD | CAVIAR 2540 | 540 MB | IDE |
| FUJITSU | M2694ESA | 1 GB | SCSI |
| MAXTOR | 7120S | 120 MB | SCSI |
| MAXTOR | 4170S | 170 MB | SCSI |
| MAXTOR | LXT340SY | 340 MB | SCSI |
| QUANTUM | ELS 170S | 170 MB | SCSI |
| QUANTUM | ELS 240S | 240 MB | SCSI |
| SEAGATE | ST31200N | 1 GB | SCSI |

G.	MEMORY  MODULES

| MANUFACTURER | MODEL NUMBER | TYPE | SIZE |
| --- | --- | --- | --- |
| MITSUBISHI | M5M44260AJ-8 | 72-pin single-sided | 4 MB |
| PANASONIC | MN41C41000SJ-8 | 72-pin single-sided | 4 MB |
| PANASONIC | MN414400ASJ-7 | 72-pin single-sided | 4 MB |
| SAMSUNG | KM416C256AJ-7 | 72-pin single-sided | 4 MB |
| MICRON TECH | MT4C4001JDJ-7 | 72-pin double-sided | 8 MB |
| NEC | D424100GS-70L-9JD | 72-pin double-sided | 32 MB |

H.	STATIC  RAMs

| MANUFACTURER | MODEL NUMBER | SIZE |
| --- | --- | --- |
| WINBOND | W24257AK-15 | 32K * 8 |

I.	MISCELLANEOUS  PERIPHERAL  DEVICES

| TYPE | MANUFACTURER | MODEL NUMBER | REMARKS |
| --- | --- | --- | --- |
| CD-ROM DRIVE | NEC | MULTI-SPIN 3XE |  |
| TAPE DRIVE | IRWIN BACKUP | ACCUTRAK PLUS + | 250 MB |
| TAPE DRIVE | QTRONIX | SDX-85 | 150 MB |
| TAPE DRIVE | WANGTEK | 5525ES | 525 MB |

III.	PERFORMANCE  TEST

Testing  Configuration:

Mainboard		:	MS-5106  P54C-90  MHz

Version 		:	Ver 1

System BIOS	:	AWARD WM71

System Chipset:	INTEL Mercury

VGA Card		:	MS-4405 ET4000/W32P PCI VGA Card

IDE Controller		:	MS-4803 ALI PCI IDE Controller

I/O Controller		:	MS-3806 SMC Super I/O Controller

System Memory	:	8 MB (Samsung KM416C1200J-7)

Cache Memory		:	256 KB (Winbond W24257AK-15)

Hard Disk drive	:	Conner CFS420A (420MB) HDD

| BYTE BENCHMARK | Ver 1.3 | Ver 2.2 |
| --- | --- | --- |
| CPU Index | 15.51 | 2.98 |
| FPU Index | 185.42 | 14.21 |
| Disk Index | 24.05 | 1.28 |
| Video Index | 17.59 | 6.07 |

| PC BENCHMARK | Ver 7.0 |
| --- | --- |
| DOSMark | 44.68 |
| Processor Harmonic | 37411.55 |
| Video Harmonic | 11916.77 |
| Memory Harmonic | 7583.53 |
| Disk Harmonic | 37.34 |

| SYSMARK 93 |  |
| --- | --- |
| Word Processing | 275.15 |
| Spreadsheet | 278.09 |
| Database | 221.14 |
| Desktop Graphics | 293.70 |
| Desktop Presentation | 261.38 |
| Desktop Publishing | 209.35 |
| SYSmarkWIN93 | 263.55 |

| NORTON SYSTEM INFO | Ver 4.5 | Ver 7.0 |
| --- | --- | --- |
| Computing Index | 191.0 | 285.0 |
| Disk Index | 4.1 | 8.5 |
| Performance Index | 128.7 | 192.6 |

| LANDMARK SPEED | Ver 2.0 |
| --- | --- |
| CPU Speed | 519.11 |
| FPU Speed | 1514.53 |
| Video Speed | 15123.00 |

| CHECKIT | Ver 3.0 | UNITS |
| --- | --- | --- |
| MAIN SYSTEM |  |  |
| CPU Speed | 70577 | Dhrystones |
| Math Speed | 26897.5 | M-Whetstones |
| VIDEO SYSTEM |  |  |
| BIOS Video Speed | 45701 | Char/S |
| Direct Video Speed | 510772 | Char/S |
| HARD DISK |  |  |
| Transfer Speed | 645.1 | K/Second |
| Average Seek Time | 13.6 | ms |
| Track Seek Time | 4.1 | ms |

| QA PLUS | Ver 4.61 | Ver 4.70 | UNITS |
| --- | --- | --- | --- |
| SYSTEM PERF. |  |  |  |
| CPU Speed | 68275 | 68.3 K | Dhrystones |
| Direct Video Speed | 1704127 | 1.7 M | CPS |
| Math Speed | 27439.5 K | 26.7 M | Whetstones |
| DISK PERF. |  |  |  |
| Trk-Trk Seek | 4.34 | 4.38 | ms |
| Average Seek | 12.6 | 12.8 | ms |
| Transfer Rate | 565.0 | 481.9 | K/sec |

| POWER METER | Ver 1.81 | UNITS |
| --- | --- | --- |
| SYSTEM TEST |  |  |
| Agg. Hardware | 4399.3 | PMUs |
| Spreadsheet | 5826.5 | PMUs |
| Word Processing | 4841.6 | PMUs |
| Database App'n. | 3282.7 | PMUs |
| CAE / CAD | 117533.9 | PMUs |
| Throughput Test | 32999.9 | PMUs |
| CPU MEMORY |  |  |
| Agg. CPU/Memory | 66088.7 | PMUs |
| Clock Speed Test | 179.9 | MHertz |
| MIPS Test | 43.8 | MIPS |
| Dhrystone Test | 76.9 | K-Dhrystones/S |
| Whetstone Test | 17531.8 | K-Whetstones/S |
| DISK DRIVE |  |  |
| Agg. Disk Test | 185.8 | PMUs |
| Trk-to-Trk Seek | 4.1 | MS/Seek |
| Mean Seek Test | 13.6 | MS/Seek |
| Data Transfer Rate | 1007.3 | Kb/Sec. |
| File Open/Close | 8.2 | MS/Op. |
| Seq File Rd/Wrt | 0.1 | MS/Op. |
| Ran File Rd/Wrt | 13.4 | MS/Op. |
| VIDEO DISPLAY |  |  |
| Agg Video Test | 11103.5 | PMUs Md 3 |
| Char Write Test | 0.4 | Sec. Md 3 |
| Line Drawing Test | 3.0 | sec. Md 18 |
| Fill Test | 16.0 | Sec. Md 18 |

The following programs below were tested under Windows 3.1, using Tseng Labs' ET4000/W32P Windows driver, 1024 x 768 x 256C resolution.

| WINDOWS TEST |  | UNITS |
| --- | --- | --- |
| SPEEDY | 149.75 | SPEEDMARK |
| WINTACH 1.2 | 78.54 | RPM |
| WINBENCH 3.11 | 62.09 | WINMARK |
| WINBENCH 4.0 | 30.3 | WINMARK |

| WINDSOCK |  |
| --- | --- |
| CPU | 930 |
| VIDEO | SU ¾ 1771 |
| DISK | GR ¾ 271 |
| MEMORY | GO ¾ 197 |
| WINDOWS (OVERALL) | SU ¾ 816 |

Note:

SU	=	Superb

GR	=	Great

GO 	=	Good