MICRO-STAR INTERNATIONAL CO., LTD.

7F, NO. 4, LANE 497, CHUNG CHENG RD.HSIN TIEN CITY, TAIPEI HSIEN, TAIWAN, R.O.C.TEL:  886-2-218-2292 (REP)FAX:  886-2-218-5552, 218-3361, 218-6049µ §

Date:  August 1, 1994

MS-5106  P54C  PCI / ISA  MAINBOARD

SUPPLEMENTS  (Rel. 1)

MAINBOARD	MS-5106

CHIPSET        	Intel  Mercury

Please find below the proposed jumper and BIOS settings for different configurations.  Use default values and settings for the other jumpers and BIOS not specified.

MS-3805  is  the  NCR  PCI  SCSI  Controller.

MS-4803  is  the  ALI  PCI  IDE  Controller.

CASE I:		MS-5106 + MS-3805

For MS-5106 Version 1 Mainboard:

1.		For  MS-5106:

Set  JP6  to  2-3.

2.		For  MS-3805:

Set  JP2  to  1-2.

Install  MS-3805  in  PCI  Slot  1.

3.		For  Award  BIOS:

Onboard  SCSI  Controller	:	Enabled

PIRQ0#  Assigned  IRQ		:	11  Level

CASE II:		MS-5106 + MS-4803

For MS-5106 Version 1 Mainboard:

1.		For  MS-5106:

Set  JP1  to  1-2.

2.		For  MS-4803:

Set  JPIDE1,  short  INTA  1-2.

Install  MS-4803  in  PCI  Slot  2.

3.		For  Award  BIOS:

PIRQ1#  Assigned  IRQ		:	NA  Level*

*	Take note that the IRQ need not be set in the BIOS, since it is already set in the 				mainboard with the JP1 jumper.

CASE III:		MS-5106 + MS-3805 + MS-4803

For MS-5106 Version 1 Mainboard:

1.		For  MS-5106:

Set  JP6  to  2-3.

Set  JP1  to  1-2.

2.		For  MS-3805:

Set  JP2  to  1-2.

Install  MS-3805  in  PCI  Slot  1.

3.		For  MS-4803:

Set  JPIDE1,  short  INTA  1-2.

Install  MS-4803  in  PCI  Slot  2.

4.		For  Award  BIOS:

Onboard  SCSI  Controller	:	Enabled

PIRQ0#  Assigned  IRQ		:	11  Level

PIRQ1#  Assigned  IRQ		:	NA  Level *

*	Take note that the IRQ need not be set in the BIOS, since it is already set in the 				mainboard with the JP1 jumper.

CASE IV:		MS-5106 + DTC Symphony or CMD or Tekram 								PCI IDE Controller (Using Level Trigger)

For MS-5106 Version 1 Mainboard:

1.		For  MS-5106:

Set  JP1  to  2-3.

2.		For  the  PCI  IDE  Controller:

Set  the  PCI  IDE  Controller  to  use  INTA,  Level  Trigger .

Install  the  PCI  IDE  Controller  in  PCI  Slot  2.

3.		For  Award  BIOS:

PIRQ1#  Assigned  IRQ		:	NA  Level*

*	Take note that the IRQ need not be set in the BIOS, since it is already set in the 				mainboard with the JP1 jumper.

CASE V:		MS-5106 + Adaptec AHA-2940 or  Buslogic

BT-946C or Future Domain PCI SCSI

For MS-5106 Version 1 Mainboard:

1.		For  MS-5106:

Set  JP6  to  1-2.

2.		For  the  PCI  SCSI  Controller:

Install  the  PCI  SCSI  Controller  in  PCI  Slot  1.

3.		For  Award  BIOS:

PIRQ0#  Assigned  IRQ		:	11  Level