MICRO-STAR INTERNATIONAL CO., LTD.

7F, NO. 4, LANE 497, CHUNG CHENG RD.HSIN TIEN CITY, TAIPEI HSIEN, TAIWAN, R.O.C.TEL:  886-2-218-2292 (REP)FAX:  886-2-218-5552, 218-3361, 218-6049µ §

MS-5109  SiS  P54C  PCI / ISA  MAINBOARD

TEST  REPORT

Date:  November  10,  1994

MAINBOARD	MS-5109

SYS. CHIPSET	SiS 82C501 + 502 + 503

ON-BOARD IDE 	Winbond W83769F

ON-BOARD I/O	Winbond W83787F / W83758F

TABLE OF CONTENTS:

I.	SOFTWARE  COMPATIBILITY  TEST	1

A.	DOS	2

B.	IBM OS/2	2

C.	SCO UNIX	3

D.	WINDOWS NT	3

E.	NOVELL NETWARE	4

F.	PACKETS & UTILITIES	7

G.	DIAGNOSTIC PROGRAMS	8

II.	HARDWARE COMPATIBILITY TEST	9

A.	VGA CARDS	9

B.	SCSI CONTROLLERS	9

C.	NETWORK ADAPTERS	10

D.	HARD DISK DRIVES	10

E.	FLOPPY DISK DRIVES	10

F.	MEMORY MODULES	11

G.	STATIC RAMS	11

H.	MISCELLANEOUS PERIPHERAL DEVICES	11

III.	PERFORMANCE TEST	12

I.		SOFTWARE COMPATIBILITY TEST

A.  DOS

| MS-DOS | VER 5.0 | VER 6.0 | VER 6.2 | REMARKS |
| --- | --- | --- | --- | --- |
| INSTALL | OK | OK | OK |  |
| BOOT | OK | OK | OK |  |
| FORMAT A: | OK | OK | OK |  |
| BOOT FROM A | OK | OK | OK |  |
| COPY | OK | OK | OK |  |
| COMP | OK | OK | OK |  |
| XCOPY | OK | OK | OK |  |
| DISKCOPY | OK | OK | OK |  |
| DISKCOMP | OK | OK | OK |  |
| M.C.R. DIR | OK | OK | OK |  |

B.  IBM  OS/2

Configuration:

·	Intel P54C Pentium-100 CPU

·	16MB Memory (Micron Tech MT4C4001JDJ-7) 4MB x 4

·	256KB Cache (Winbond W24257AK-15)

·	MS-4405 ET4000/W32P PCI VGA Card

·	Conner CP30254 (254MB) Hard Disk

·	One 1.44MB and one 1.2MB FDDs

·	Genius Mouse

| OS/2 | VER 2.0 | VER 2.1 | REMARKS |
| --- | --- | --- | --- |
| INSTALL | OK | OK |  |
| BOOT | OK | OK |  |
| FORMAT A: | OK | OK |  |
| COPY & COMP | OK | OK |  |
| XCOPY | OK | OK |  |
| DISKCOPY | OK | OK |  |
| DISKCOMP | OK | OK |  |
| M.C.R. DIR | OK | OK |  |
| TASK TEST | OK | OK |  |
| DOS WINDOW | OK | OK |  |
| OS/2 WINDOW | OK | OK |  |
| WIN-OS/2 | OK | OK |  |
| DOS PROGRAMS | OK | OK |  |
| WIN PROGRAMS | OK | OK |  |

C.  SCO  UNIX

Configuration:

·	Intel P54C Pentium-100 CPU

·	24MB Memory (Micron MT4C4001JDJ-7) 4MB x 6

·	256KB Cache (Winbond W24257AK-15)

·	MS-3805 NCR PCI SCSI Controller

·	MS-4405 ET4000/W32P PCI VGA Card

·	Wangtek 5525ES Tape Drive

·	Maxtor LXT-340S (340MB) SCSI Hard Disk

·	One 1.2MB FDD

·	Genius Mouse

| SCO UNIX | REL 3.2.4 | REMARKS |
| --- | --- | --- |
| INSTALL | OK |  |
| BOOT | OK |  |
| O.D.T. | OK |  |
| TASK TEST | OK |  |

D.  WINDOWS  N.T.

Configuration:

·	Intel P54C Pentium-100 CPU

·	24MB Memory (Samsung KM44C1000BJ-7) 4MB x 6

·	256KB Cache (Winbond W24257AK-15)

·	MS-3805 NCR PCI SCSI Controller

·	MS-4405 ET4000/W32P PCI VGA Card

·	Maxtor LXT340SY (340MB) SCSI Hard Disk

·	NEC Multi-Spin 3XE CD-ROM

·	One 1.44MB and one 1.2MB FDDs

·	Logitech Mouse

| WINDOWS N.T. | VER 3.1 | REMARKS |
| --- | --- | --- |
| INSTALL | OK |  |
| BOOT | OK |  |
| FORMAT A: | OK |  |
| COMP | OK |  |
| XCOPY | OK |  |
| DISKCOPY | OK |  |
| DISKCOMP | OK |  |
| TASK TEST | OK |  |
| HCT | OK |  |

E.  NOVELL  NETWARE

Server Configuration:

·	Intel P54C Pentium-100 CPU

·	128MB Memory (Panasonic MN4117400SJ-08) 32MB x 4

·	256KB Cache (Winbond W24257AK-15)

·	MS-3805 NCR PCI SCSI Controller

·	Cirrus Logic GD-5422 ISA VGA Card

·	Conner CFA540S (540MB) SCSI Hard Disk

·	One 1.44MB and one 1.2MB FDDs

| NETWARE | 386 V3.12 | REMARKS |
| --- | --- | --- |
| INSTALL | OK |  |
| BOOT | OK |  |
| LOAD DRIVERS | OK |  |
| 3COM 3C509 | OK | ISA Network Card |
| AMD PC-NET | OK | PCI Network Card (2 pieces) |
| ARCNET | OK | ISA Network Card (1 piece) |
| DEC 21040 | OK | PCI Network Card |
| TOKEN RING | OK | ISA Network Card |

Workstation 1 Configuration:

·	Intel P54C Pentium-90 CPU

·	8MB Memory (Samsung KM416C1200J-7) 4MB x 2

·	256KB Cache (Winbond W24257AK-15)

·	Trident 8900C ISA VGA Card

·	One 1.2MB FDD

| NETWARE | 386 V3.12 | REMARKS |
| --- | --- | --- |
| LOGIN | OK |  |
| LAN TEST | OK |  |
| NWTEST | OK |  |
| FILE TRANSFER | OK |  |
| INSTALL WINDOWS FROM SERVER | OK |  |
| AMD PC-NET | OK | PCI Network Card |

Workstation 2 Configuration:

·	Intel P54C Pentium-90 CPU

·	8MB Memory (Samsung KM416C1200J-7) 4MB x 2

·	256KB Cache (Winbond W24257AK-15)

·	Trident 8900C ISA VGA Card

·	One 1.2MB FDD

| NETWARE | 386 V3.12 | REMARKS |
| --- | --- | --- |
| LOGIN | OK |  |
| LAN TEST | OK |  |
| NWTEST | OK |  |
| FILE TRANSFER | OK |  |
| INSTALL WINDOWS FROM SERVER | OK |  |
| DEC 21040 | OK | PCI Network Card |

Workstation 3 Configuration:

·	Intel P54C Pentium-90 CPU

·	32MB Memory (Mitsubishi M5M417400AJ-7) 16MB x 2

·	256KB Cache (Winbond W24257AK-15)

·	Trident 8900C ISA VGA Card

·	One 1.2MB FDD

| NETWARE | 386 V3.12 | REMARKS |
| --- | --- | --- |
| LOGIN | OK |  |
| LAN TEST | OK |  |
| NWTEST | OK |  |
| FILE TRANSFER | OK |  |
| INSTALL WINDOWS FROM SERVER | OK |  |
| UMC NE-2000 | OK | ISA Network Card |

Workstation 4 Configuration:

·	Intel P54C Pentium-100 CPU

·	32MB Memory (Mitsubishi M5M417400AJ-7) 16MB x 2

·	256KB Cache (Winbond W24257AK-15)

·	Trident 8900C ISA VGA Card

·	One 1.2MB FDD

| NETWARE | 386 V3.12 | REMARKS |
| --- | --- | --- |
| LOGIN | OK |  |
| LAN TEST | OK |  |
| NWTEST | OK |  |
| FILE TRANSFER | OK |  |
| INSTALL WINDOWS FROM SERVER | OK |  |
| ARCNET | OK | ISA Network Card |

Workstation 5 Configuration:

·	Intel P54C Pentium-100 CPU

·	32MB Memory (Mitsubishi M5M417400AJ-7) 16MB x 2

·	256KB Cache (Winbond W24257AK-15)

·	Trident 8900C ISA VGA Card

·	One 1.2MB FDD

| NETWARE | 386 V3.12 | REMARKS |
| --- | --- | --- |
| LOGIN | OK |  |
| LAN TEST | OK |  |
| NWTEST | OK |  |
| FILE TRANSFER | OK |  |
| INSTALL WINDOWS FROM SERVER | OK |  |
| TOKEN RING | OK | ISA Network Card |

F.  PACKETS  AND  UTILITIES

| S O F T W A R E | REMARKS |
| --- | --- |
| AutoCAD R12 | OK |
| Central Point Anti-Virus 1.4 | OK |
| Clipper 5.01 | OK |
| CorelDraw V3.0 | OK |
| DBase III Plus | OK |
| DBase 4 | OK |
| GeoWorks Ensemble V1.2 | OK |
| Harvard Graphics V2.12 | OK |
| Lotus 1-2-3 R2.4 | OK |
| Lotus 1-2-3 R3.1+ | OK |
| Microsoft Entertainment Pack Vol 1 | OK |
| Microsoft Entertainment Pack Vol 2 | OK |
| Microsoft Entertainment Pack Vol 3 | OK |
| Microsoft Excel V4.0 | OK |
| Microsoft PowerPoint for Windows V3.0 | OK |
| Microsoft Video for Windows | OK |
| Microsoft Visual Basic V2.0 | OK |
| Microsoft Windows 3.1 | OK |
| Microsoft Word for Windows V2.0 | OK |
| Microsoft Works for Windows V2.0 | OK |
| Norton Desktop for Windows V1.2 | OK |
| Norton Utilities V7.0 / V8.0 | OK |
| PC Tools V8.0 | OK |
| PE 2 | OK |
| QEMM 386 V5.11 | OK |
| QEMM 386 V6.0 | OK |
| QEMM 386 V7.01 | OK |
| QEMM 386 V7.02 | OK |
| Symantec Entertainment Pack for Windows | OK |
| TopDOS 3.00 | OK |
| Ventura Publisher V2.0 | OK |
| WordPerfect 5.1 | OK |
| XTree Gold V2.51 | OK |

G.  DIAGNOSTIC  PROGRAMS

| S O F T W A R E | REMARKS |
| --- | --- |
| '92 System Mark | OK |
| '93 System Mark | OK |
| Byte Benchmark V1.3 | OK |
| Byte Benchmark V2.2 | OK |
| CheckIt V3.0 | OK |
| CheckIt Pro | OK |
| Core Test V2.92 | OK |
| IBM Diagnostics V2.03 | OK |
| Landmark Speed Test V1.14 | OK |
| Landmark Speed Test V2.0 | OK |
| Microsoft Software Diagnostics (MSD) | OK |
| Norton Diagnostics V7.0 | OK |
| Norton System Information V 4.5 | OK |
| Norton System Information V7.0 | OK |
| PC Magazine Labs Benchmark 6.0 | OK |
| PC Magazine Labs Benchmark 7.0 | OK |
| PC Week Labs System Benchmarks V1.10 | OK |
| Power Meter V1.7 | OK |
| Power Meter V1.81 | OK |
| QA Plus V4.61 | OK |
| QA Plus V4.70 | OK |
| QA Plus/FE V5.03 | OK |
| QA Plus/FE V5.04 | OK |
| QA Plus/FE V5.12 | OK |
| QA Plus/FE V5.13 | OK |
| Speedy | OK |
| Superscale 3D Benchmark | OK |
| WinBench V2.5 | OK |
| WinBench V3.11 | OK |
| WinBench V4.0 | OK |
| WindSock 3.1 | OK |
| WinStone 94 | OK |
| WinTach V1.0 | OK |
| WinTach V1.2 | OK |

II.		HARDWARE COMPATIBILITY TEST

A.		VGA  CARDS

ISA:

| MANUFACTURER & MODEL NO. | CHIPSET | REMARKS |
| --- | --- | --- |
| CIRRUS LOGIC | GD-5422 |  |
| OAK | OTI-087 |  |
| OAK | OTI-087X |  |
| TRIDENT | TVGA8900C |  |
| TRIDENT | TVGA9000B |  |
| TSENG LABS | ET4000/W32 |  |

PCI:

| MANUFACTURER & MODEL NO. | CHIPSET | WINBENCH 4.0 1024x768x256C |
| --- | --- | --- |
| ARK LOGIC (1MB) | ARK1000PV | 23.3 |
| ATI MACH 32 (1MB) | MACH 32 | 9.94 |
| ATI MACH 64 (2MB) | MACH 64 | 34.2 |
| DIAMOND SPEEDSTAR 64 (1MB) | CIRRUS LOGIC 5434 | 15.2 |
| DIAMOND STEALTH 32 (2MB) | ET4000/W32P (Ver A) | 24.8 |
| DIAMOND VIPER PCI (2MB) | WEITEK P9000/P9001 | 21.4 |
| ELSA WINNER 1000PRO (2MB) | S3 864 | 18.1 |
| INTEL V7 MERCURY LITE (1MB) | S3 928 | 18.6 |
| MATROX MGA (2MB) * | MATROX MGA | 28.8 |
| MICRO-STAR MS-4402 (1MB) | ALI ALG2301 | 16.9 |
| MICRO-STAR MS-4403 (1MB) | IIT AGX-016 | 22.9 |
| MICRO-STAR MS-4405 (2MB) | ET4000/W32P (Ver C) | 25.7 |
| PROLINK (2MB) | S3 928 | 16.8 |
| TRIUMPHONY VISUAL FORGE V10 (2MB) | ET4000/W32P (Ver A) | 27.2 |

* When using Matrox MGA, set the "CPU to Mem Write Burst" option in the BIOS Setup to "Disabled".

B.		SCSI CONTROLLERS

| MANUFACTURER | MODEL NO. | REMARKS |
| --- | --- | --- |
| ADAPTEC | AHA-1542C | ISA |
| ADAPTEC | AHA-1542CF | ISA |
| ADAPTEC | AHA-2940 | PCI |
| ADAPTEC | AHA-7850 | PCI |
| MICRO-STAR MS-3805 | NCR | PCI |

* Buslogic BT-946C PCI SCSI Controller is not supported by the SiS chipset.

C.		NETWORK  ADAPTERS

| MANUFACTURER | TYPE | CHIPSET | REMARKS |
| --- | --- | --- | --- |
| 3COM | 3COM | 3C503 | ISA |
| 3COM | 3COM | 3C509 | ISA |
| SMC | ARCNET | COM90C65LJ | ISA |
|  | NE-2000 |  | ISA |
|  | NE-2100 |  | ISA |
| IBM | TOKEN RING |  | ISA |
| AMD |  | PCNET | PCI |
| DEC |  | 21040 | PCI |

D.		HARD  DISK  DRIVES

| MANUFACTURER | MODEL NUMBER | CAPACITY | REMARKS |
| --- | --- | --- | --- |
| CONNER | CP3204F | 204 MB | IDE |
| CONNER | CP30254 | 254 MB | IDE |
| CONNER | CFS420A | 420 MB | IDE |
| CONNER | CFS540A | 540 MB | IDE |
| QUANTUM | 240AT | 240 MB | IDE |
| QUANTUM | 540AT | 540 MB | IDE |
| SEAGATE | ST3243A | 214 MB | IDE |
| WD | CAVIAR 2250 | 250 MB | IDE |
| WD | CAVIAR 2420 | 420 MB | IDE |
| WD | CAVIAR 2540 | 540 MB | IDE |
| FUJITSU | M2694ESA | 1 GB | SCSI |
| MAXTOR | 7120S | 120 MB | SCSI |
| MAXTOR | 4170S | 170 MB | SCSI |
| MAXTOR | LXT340SY | 340 MB | SCSI |
| QUANTUM | ELS 170S | 170 MB | SCSI |
| QUANTUM | ELS 240S | 240 MB | SCSI |
| SEAGATE | ST31200N | 1 GB | SCSI |

E.		FLOPPY  DISK  DRIVES

| MANUFACTURER | MODEL NUMBER | CAPACITY |
| --- | --- | --- |
| MITSUBISHI | MF504C-315MG | 1.2 MB |
| MITSUBISHI | MF355C-255MG | 1.44 MB |
| TEAC | FD235HF | 1.44 MB |

F.		MEMORY  MODULES

Without Parity:

| MANUFACTURER | MODEL NUMBER | TYPE | SIZE |
| --- | --- | --- | --- |
| MITSUBISHI | M5M44260AJ-8 | 72-pin single-sided | 4 MB |
| SAMSUNG | KM416C256AJ-7 | 72-pin single-sided | 4 MB |
| SAMSUNG | KM416C256AJ-7 | 72-pin single-sided | 4 MB |
| MICRON TECH | MT4C4001JDJ-7 | 72-pin double-sided | 8 MB |
| MITSUBISHI | M5M417400AJ-7 | 72-pin double-sided | 16 MB |

With Parity:

| MANUFACTURER | MODEL NUMBER | TYPE | SIZE |
| --- | --- | --- | --- |
| SAMSUNG | KM44C1000BJ-7 | 72-pin single-sided | 4 MB |
| MITSUBISHI | M5M417400ATP-7 | 72-pin double-sided | 32 MB |
| NEC | D424100GS-70L-9JD | 72-pin double-sided | 32 MB |
| PANASONIC | MN4117400SJ-08 | 72-pin double-sided | 32 MB |

G.		STATIC  RAMs

| MANUFACTURER | MODEL NUMBER | SIZE |
| --- | --- | --- |
| WINBOND | W24257AK-15 | 32K * 8 |

H.		MISCELLANEOUS  PERIPHERAL  DEVICES

| TYPE | MANUFACTURER | MODEL NUMBER | REMARKS |
| --- | --- | --- | --- |
| AUDIO CARD | CREATIVE LABS | SOUND BLASTER PRO |  |
| CD-ROM DRIVE | CREATIVE TECH. | CR-521-C |  |
| CD-ROM DRIVE | NEC | MULTI-SPIN 3XE | SCSI |
| CD-ROM DRIVE | PHILIPS | LMS TYPE CM207 | IDE |
| CD-ROM DRIVE | WEARNES | CDD-120 | IDE |
| TAPE DRIVE | WANGTEK | 5525ES | 525 MB |
| VIDEO CARD | CREATIVE LABS | VIDEO BLASTER |  |

III.	PERFORMANCE  TEST

Testing  Configuration:

Mainboard		:	MS-5109  P54C  Pentium-100  MHz

Version 		:	Ver 1

System BIOS	:	AMI A772

System Chipset:	SiS 82C501 + 502 + 503

VGA Card		:	MS-4405 ET4000/W32P PCI VGA Card (2 MB)

IDE Controller		:	On-board Winbond W83769F PCI IDE Controller

I/O Controller		:	On-board Winbond W83787F / W83758F Super I/O Controller

System Memory	:	8 MB (Samsung KM416C1200J-7)

Cache Memory		:	256 KB (Winbond W24257AK-15)

Hard Disk drive	:	Conner CP30254 (240MB) HDD

PC Magazine Labs BenchMark 7.0 was tested with the ff. AUTOEXEC.BAT and CONFIG.SYS:

AUTOEXEC.BAT:

PROMPT $P$G

PATH=C:\;C:\DOS

GMOUSE

CONFIG.SYS:

FILES=20

C:\WBIDE\WBIDE.EXE (driver for the Winbond PCI IDE Controller)

| PC BENCHMARK | Ver 7.0 |
| --- | --- |
| DOSMark | 27.29 |
| Processor Harmonic | 37725.52 |
| Video Harmonic | 9317.75 |
| Memory Harmonic | 25204.26 |
| Disk Harmonic | 18.52 |

All the Diagnostic Programs below were tested with the ff. AUTOEXEC.BAT and CONFIG.SYS:

AUTOEXEC.BAT:

PROMPT $P$G

PATH=C:\;C:\DOS

GMOUSE

CONFIG.SYS:

FILES=20

BUFFERS=20

C:\WBIDE\WBIDE.EXE (driver for the Winbond PCI IDE Controller)

| LANDMARK SPEED | Ver 2.0 |
| --- | --- |
| CPU Speed | 574.56 |
| FPU Speed | 1676.30 |
| Video Speed | 8856.00 |

| BYTE BENCHMARK | Ver 1.3 | Ver 2.2 |
| --- | --- | --- |
| CPU Index | 22.11 | 2.82 |
| FPU Index | 206.61 | 14.14 |
| Disk Index | N.A. | 1.04 |
| Video Index | 17.87 | 6.64 |

| NORTON SYSTEM INFO | Ver 4.5 | Ver 7.0 |
| --- | --- | --- |
| Computing Index | 199.6 | 315.4 |
| Disk Index | 9.5 | 6.5 |
| Performance Index | 136.2 | 212.1 |

| QA PLUS | Ver 4.61 | Ver 4.70 | UNITS |
| --- | --- | --- | --- |
| SYSTEM PERF. |  |  |  |
| CPU Speed | 102413 | 102.4 K | Dhrystones |
| Direct Video Speed | 1704127 | 1.7 M | CPS |
| Math Speed | 31630.4 | 29.6 M | Whetstones |
| DISK PERF. |  |  |  |
| Trk-Trk Seek | 4.46 | 4.46 | ms |
| Average Seek | 11.9 | 11.9 | ms |
| Transfer Rate | 655.4 | 455.1 | K/sec |

| CHECKIT | Ver 3.0 | UNITS |
| --- | --- | --- |
| MAIN SYSTEM |  |  |
| CPU Speed | 78419 | Dhrystones |
| Math Speed | 31416.0 | M-Whetstones |
| VIDEO SYSTEM |  |  |
| BIOS Video Speed | 33681 | Char/S |
| Direct Video Speed | 490905 | Char/S |
| HARD DISK |  |  |
| Transfer Speed | 439.5 | K/Second |
| Average Seek Time | 13.6 | ms |
| Track Seek Time | 5.3 | ms |

| POWER METER | Ver 1.7 | Ver 1.81 | UNITS |
| --- | --- | --- | --- |
| SYSTEM TEST |  |  |  |
| Agg. Hardware | 4961.6 | 4688.9 | PMUs |
| Spreadsheet | 3835.7 | 5736.1 | PMUs |
| Word Processing | 4644.1 | 5362.1 | PMUs |
| Database App'n. | 2741.6 | 3126.2 | PMUs |
| CAE / CAD | 133408.9 | 124100.3 | PMUs |
| Throughput Test | 36918.0 | 34414.1 | PMUs |
| CPU MEMORY |  |  |  |
| Agg. CPU/Memory | 79869.3 | 74741.3 | PMUs |
| Clock Speed Test | 186.5 | 186.5 | MHertz |
| MIPS Test | 68.4 | 49.9 | MIPS |
| Dhrystone Test | 87.7 | 87.7 | K-Dhrystones/S |
| Whetstone Test | 19594.4 | 19217.6 | K-Whetstones/S |
| DISK DRIVE |  |  |  |
| Agg. Disk Test | 894.8 | 871.7 | Kb/Sec. |
| Trk-to-Trk Seek Test | 3.9 | 3.9 | MS/Op. |
| Mean Seek Test | 10.2 | 10.3 | MS/Op. |
| Data Transfer Rate | 5505.8 | 5628.1 | MS/Op. |
| File Open/Close | 6.5 | 6.5 | MS/Op. |
| Seq File Rd/Wrt | 0.1 | 0.2 | MS/Op. |
| Ran File Rd/Wrt | 12.4 | 12.8 | MS/Op. |
| VIDEO DISPLAY |  |  |  |
| Agg. Video Test | 10519.1 | 10519.1 | PMUs Md 3 |
| Char Write Test | 0.4 | 0.4 | Sec. Md 3 |
| Line Drawing Test | 2.9 | 2.9 | sec. Md 18 |

The following programs below were tested under Windows 3.1 with the ff. AUTOEXEC.BAT and CONFIG.SYS:

AUTOEXEC.BAT:

PROMPT $P$G

PATH=C:\;C:\DOS

C:\WINDOWS\SMARTDRV.EXE

GMOUSE

CONFIG.SYS:

C:\WINDOWS\HIMEM.SYS

FILES=20

BUFFERS=20

C:\WBIDE\WBIDE.EXE (driver for the Winbond PCI IDE Controller)

Tested using MS-4405 Tseng Labs' ET4000/W32P (Ver. C, 2 MB memory) Windows driver Ver 1.2, 1024 x 768 x 256C resolution.

| WINDOWS TEST |  | UNITS |
| --- | --- | --- |
| SPEEDY | 119.42 | SPEEDMARK |
| WINTACH 1.2 | 70.38 | RPM |
| WINBENCH 3.11 | 46.86 | WINMARK |
| WINBENCH 4.0 | 25.7 | WINMARK |

| WINDSOCK |  |
| --- | --- |
| CPU | 1050 |
| VIDEO | SU ¾ 2110 |
| DISK | GR ¾ 435 |
| MEMORY | GR ¾ 298 |
| WINDOWS (OVERALL) | SU ¾ 975 |

Note:

SU	=	Superb

GR	=	Great

GO 	=	Good

All brand names and trademarks mentioned herein are the properties of their respective owners.