MICRO-STAR INTERNATIONAL CO., LTD.

7F, NO. 4, LANE 497, CHUNG CHENG RD.HSIN TIEN CITY, TAIPEI HSIEN, TAIWAN, R.O.C.TEL:  886-2-218-2292 (REP)FAX:  886-2-218-5552, 218-3361, 218-6049µ §

MS-5103,  MS-5107  AND  MS-5108  PERFORMANCE

COMPARISON  REPORT

Date:  September  14,  1994

DESCRIPTION  OF  TEST  MAINBOARDS:

1.	MS-5103		INTEL  MERCURY  CHIPSET

2.	MS-5107		ALI  M1449 + M1451  CHIPSET

3.	MS-5108		SiS  82C501 + 502 + 503  CHIPSET  with  On-board  						Winbond  W83769F  PCI  IDE  Controller  +  Winbond  					W83787F / W83758F  Super  I/O  Controller

TESTING  CONFIGURATION:

Processor Speed	:	Pentium 66 MHz

VGA Card		:	MS-4405 ET4000/W32P PCI VGA Card (2MB)

IDE Controller 1	:	MS-4805 ARTOP PCI IDE Cache Controller (512KB)

I/O Controller 2	:	UMC ISA Super I/O Controller

System Memory	:	8 MB (Samsung KM416C1200J-7)

Cache Memory	:	256 KB (Winbond W24257AK-15)

Hard Disk drive	:	Conner CFS420A (420MB) HDD

1	For MS-5108, the On-board Winbond W83769F PCI IDE Controller was used for testing.

2	For MS-5108, the On-board Winbond W83787F / W83758F Super I/O Controller was used for testing.

TESTING  DESCRIPTION:

The following diagnostic programs have been used in the performance testing:

1.	PC Magazine Labs Benchmark 7.0 and 8.0

2.	Byte Magazine Labs Benchmark 1.3 and 2.2

3.	Diagsoft Power Meter 1.7 and 1.81

4.	Diagsoft QA PLUS 4.61

5.	Check It 3.0

6.	Speedy

7.	WinTach 1.2

8.	ZD Labs Windows Benchmark 3.11 and 4.0

9.	Windsock 3.1

TEST  RESULTS:

PC Magazine Labs BenchMark 7.0 and 8.0 were tested with the ff. AUTOEXEC.BAT and CONFIG.SYS:

AUTOEXEC.BAT:

PROMPT $P$G

PATH=C:\;C:\DOS

GMOUSE

CONFIG.SYS:

FILES=20

|  | MS-5103 | MS-5107 | MS-5108 | UNITS |
| --- | --- | --- | --- | --- |
| PC LABS BENCHMARK VER 7.0 |  |  |  |  |
| Processor Harmonic | 29380.03 | 27492.92 | 27647.18 |  |
| Video Harmonic | 7426.89 | 12623.47 | 11282.16 |  |
| Memory Harmonic | 7784.59 | 17046.60 | 21882.16 |  |
| PC LABS BENCHMARK VER 8.0 |  |  |  |  |
| Processor Score | 78.96 | 73.24 | 75.34 |  |
| Video Score | 3172.32 | 5953.15 | 5355.02 |  |

All the Diagnostic Programs below were tested with the ff. AUTOEXEC.BAT and CONFIG.SYS:

AUTOEXEC.BAT:

PROMPT $P$G

PATH=C:\;C:\DOS

GMOUSE

CONFIG.SYS:

FILES=20

BUFFERS=20

|  | MS-5103 | MS-5107 | MS-5108 | UNITS |
| --- | --- | --- | --- | --- |
| BYTE BENCHMARK 1.3 |  |  |  |  |
| CPU Index | 14.07 | 15.66 | 17.99 |  |
| FPU Index | 108.88 | 106.66 | 107.94 |  |
| Disk Index | N.A. | N.A. | N.A. |  |
| Video Index | 16.25 | 14.98 | 13.89 |  |
| BYTE BENCHMARK 2.2 |  |  |  |  |
| CPU Index | 2.85 | 2.24 | 2.48 |  |
| FPU Index | 10.23 | 9.43 | 9.43 |  |
| Disk Index | 1.62 | 1.54 | 1.27 |  |
| Video Index | 5.29 | 6.08 | 5.50 |  |
| POWER METER 1.7 |  |  |  |  |
| Agg CPU / Memory Test | 50933.7 | 51077.0 | 51100.3 | PMUs |
| Clock Speed Test | 133.0 | 128.5 | 127.0 | MHertz |
| MIPS Test | 44.8 | 44.8 | 45.9 | MIPS |
| Dhrystone Test | 57.4 | 57.4 | 58.8 | K-Dhrystones/S |
| Whetstone Test | 13148.9 | 12978.1 | 12978.1 | K-Whetstones/S |
| POWER METER 1.81 |  |  |  |  |
| Agg CPU / Memory Test | 48923.7 | 48892.0 | 47270.0 | PMUs |
| Clock Speed Test | 133.3 | 128.5 | 127.8 | MHertz |
| MIPS Test | 30.6 | 30.9 | 32.3 | MIPS |
| Dhrystone Test | 53.7 | 54.3 | 56.8 | K-Dhrystones/S |
| Whetstone Test | 12811.7 | 12811.7 | 12811.7 | K-Whetstones/S |
| QA PLUS 4.61 |  |  |  |  |
| CPU Speed | 68275 | 68275 | 68275 | Dhrystones |
| Math Speed | 21267.0 | 21086.9 | 21086.9 | K-Whetstones |
| CHECK IT 3.0 |  |  |  |  |
| CPU Speed | 52749 | 52749 | 52749 | Dhrystones |
| FPU Speed | 19801.5 | 18827.5 | 18827.5 | M-Whetstones |

The following programs below were tested under Windows 3.1 with the ff. AUTOEXEC.BAT and CONFIG.SYS:

AUTOEXEC.BAT:

PROMPT $P$G

PATH=C:\;C:\DOS

C:\WINDOWS\SMARTDRV.EXE

GMOUSE

CONFIG.SYS:

C:\WINDOWS\HIMEM.SYS

FILES=20

BUFFERS=20

Tested using MS-4405 Tseng Labs' ET4000/W32P (Ver. C, 2 MB memory) Windows driver Ver 1.2, 1024 x 768 x 256C resolution.

|  | MS-5103 | MS-5107 | MS-5108 | UNITS |
| --- | --- | --- | --- | --- |
| SPEEDY | 103.26 | 121.45 | 123.09 | SPEEDMARK |
| WINTACH 1.2 | 66.25 | 66.31 | 66.29 | RPM |
| WINBENCH 3.11 | 49.40 | 54.89 | 53.11 | WINMARK |
| WINBENCH 4.0 | 25 | 23.2 | 23.1 | WINMARK |
| WINDSOCK |  |  |  |  |
| CPU | 692 | 678 | 692 |  |
| Video | SU ¾ 1650 | SU ¾ 1771 | SU ¾ 1748 |  |
| Disk | GR ¾ 281 | GR ¾ 277 | GO ¾ 163 |  |
| Memory | GR ¾ 219 | GR ¾ 230 | GR ¾ 293 |  |
| Windows (Overall) | SU ¾ 700 | SU ¾ 725 | SU ¾ 703 |  |

Note:

SU	=	Superb

GR	=	Great

GO 	=	Good

* All testing were done using the BIOS default settings, except that in MS-5103, the "External Cache" 	was set to Write Back.

* All brand names and trademarks mentioned herein are the properties of their respective owners.