PC-UPROG

UNIVERSAL PROGRAMMER

COPYRIGHT NOTIC

This  documentation  and the  software routines contained in the  PC-UPROG

software diskette are  copyrighted, 1990 - 1995  by  Advantech  Co. , Ltd.

All rights are reserved.  Advantech Co., Ltd. reserves  the right to  make

improvements to the product described in this manual a time without notice

.No part of this manual may be reproduced,copied,translated or transmitted

, in any form, or by any means ,without  the prior written   permission of

Advantech Co., Ltd. Information provided in this manual is  intended to be

accurate and reliable. However, Advantech Co., Ltd. assumes responsibility

for its  use;  nor for any infringements of  rights of third parties which

may result from its use.

ACKNOWLEDGEMENTS

PC-UPROG is a trademark of Advantech Co. Ltd.

IBM and PC are trademarks of Internationa Business Machines Corporation.

MS-DOS is a trademark of Microsoft Corporation.

Rev. A2  In Taiwan   Aug. 1991

1. INSTALLATION

1.1. Changing your CONFIG.SYS file

Since PC-UPROG  opens up to  10 file  during operation,  the system   will

return an error if the FILES parameter of  CONFIG.SYS file is less than 15

( DOS already using 5 files ).

The CONFIG.SYS file may be edited with any ASCII text editor.

1.2. Software Installation

The  PC-UPROG software is  distributed on one 720k diskette.  The diskette

initially contains only two files.

INSTALL.EXE              15K bytes

INSTALL.DAT             306K bytes

First you should make a backup copy of the diskette (the PC UPROG software

is not copy protected).  Keep the master diskette in a  safe  place (there

is a nominal fee for replacement of lost disks).

Boot  your  system  with your standard DOS boot diskette or hard disk.  If

your  system  has a  hard disk, insert PC-UPROG  diskette into your floppy

drive and type INSTALL  C: The install utility will  create the  directory

'\PC-UPROG' on your hard disk, and install the following files.

pc-uprog.dev

pc-uprog.dmp

pc-uprog.pin

pc-uprog.fam

pc-uprog.wav

pc-uprog.hlp

pc-uprog.exe

extuprog.dev

extuprog.dmp

extuprog.pin

extuprog.fam

extuprog.wav

extuprog.hlp

extuprog.exe

The install utility will then test the hardware to verify  the  programmer

unit and PC adapter card are  installed  correctly.  If any  error occurs,

repeat the hardware installation process and try again.

For floppy disk installation, insert PC-UPROG  diskette into drive  A: and

the destination diskette into drive B: Then type INSTALL B:

After you have finished installation, go to directory '\PC-UPROG' and type

'PC-UPROG'.The system will now display the main menu and you may now refer

to 'OPERATION' section of this manual for details on specific  programming

procedures, etc.

After exiting PC-UPROG (extuprog.exe)software, PC-UPROG will have  created

two files.

pc-uprog.opt (extuprog.opt)

pc-uprog.$$$ (extuprog.$$$)

'*.opt' contains  the  software  I/O  port  base  address  an  last device

selected. '*.$$$' is the device data edit buffer.

1.3. Starting PC-UPROG

Power-on your computer.  If  using a  hard  disk,  select the  "\PC-UPROG"

directory.   If using a  floppy  based system,  insert  your  copy  of the

PC-UPROG software into the  floppy disk drive.  Then enter the command:

C:\>PC-UPROG  [XXX]<cr>

[XXX]:      Option for user selecting the I/O port base address, if no

address parameter typing then PC-UPROG will use the I/O

port base address stored in pc-uprog.opt file (or use factory

setting address 300 hex in the first time execution).

The PC-UPROG main menu  will be displayed.

1.4. Ordering Information

PC-UPROG         PC-based universal programmer, including universal

programmer unit, PC adapter card (PCL-560A),

system interface cable

Accessories furnished: System disk

Operations manual

PCL-560A         Additional PC adapter card for PC-UPROG

PLCC-2020-01     20-pin PLCC to 20-pin DIP adapter for GAL or

PAL 16XX series

PLCC-2824-04     28-pin PLCC to 24 pin DIP adapter no connection

pin on (1,8,15,22) for GAL or PAL 20XX series

PLCC-2824-05     28-pin PLCC to 24 pin DIP adapter no connection

pin on (5,8,11,19) for GAL or PAL 20XX series

PLCC-3228-01     32-pin PLCC to 28 pin DIP adapter no connection

pin on (1,12,17,26)

PLCC-3228-11     32-pin PLCC to 28-pin DIP adapter for EPROM

less than 512KB.

PLCC-3232-01     32-pin PLCC to 32-pin DIP adapter

PLCC-3232-11     32-pin PLCC to 32-pin DIP adapter for EPROM

from 1M to 8M

PLCC-4440-01     44-pin PLCC to 40-pin DIP adapter for 16 bit

EPROM such as 27C1024

PLCC-4440-02     44-pin PLCC to 40-pin DIP  adapter for  8751

series.UP NC(1,12,23,34).

PLCC-4440-08     44-pin PLCC to 40 pin DIP adapter for 16 bit

EPROM no connection pin on (1, 13, 23, 33)

PLCC-4428-01     44-pin PLCC adapter for AMD MACH110/210/215

devices

PLCC-4428-02     44-pin PLCC adapter for Lattice pLSI/ispLSI

1016 devices

PLCC-6828-01     PLCC for AMD MACH 120/220

PLCC-6828-02     68-pin PLCC adapter for Lattice pLSI/ispLSI

1024 devices

PLCC-8424-01     PLCC for AMD MACH 130/230/435

PLCC-8428-02     84-pin PLCC adapter for Lattice pLSI/ispLSI

1032 devices

PLCC-EP610       28-pin PLCC socket for EP610,special adapter

for ALtera

PLCC-6840-01     68-pin PLCC to 40 pin DIP adapter for

ALtera EP18XX series

PLCC-ATV2500     PLCC/DIP socket for Atmel ATV2500/ Signetics

PLV2500

AD16C54          DIP for MicroChip 16C54/55/56/57/71/84

AD68705          Motorola 68705 P3/P5, U3/U5, R3/R5 adapter

AD68705C8/C9     Motorola 68705C8/C9 adapter

AD87C751         Signetic 87C750/751/752 adapter

AD86E21          Zilog Z86E21 adapter

AD86E30/40       DIP for Zilog Z86E30/Z86E40

ADEPM5016        Altera EPM5016/32 DIP Adapter

ADEPM7032        Altera EPM7032 DIP Adapter

ADPSD301         44 pin PLCC for PSD3XX Series adapter

ADPSD401         68 pin PLCC for PSD4XX/5XX Series adapter

AT-101/A         UV-ERASER with timer (30 devices maximun)

AT-102/A         Portable UV-ERASER with timer (12 devices

maximun)

PALASM 90        PLD software design package

1.5. DEVICES NEED ADAPTERS

AMD        MACH110/210/215/120/220/130/230/435

Motorola   M68705P3/P5/U3/U5/R3/R5/C8/C9

M68HC11

Philips    87C750/751/752/451/452/552/054/592   PLV2500

Zilog      Z86E21/E30/E40    Z86E08

SGS-Thomas Z86E21

Micro-chip PIC16C54/55/56/57/71/84

PIC17C42

Altera     EP1800/1810/1830/5016/5032/7032  ATV2500/B

TI         EP1810/1830

Intel      5C180,28F002-B/T,28F200-B/T,28F004-B/T,28F400-B/T

Waferscale PSD301/311/311C1,PSD302/312/312C1,PSD303/313/313C1,

PSD401/411A1,PSD401/411A2,PSD402/412A1,PSD402/412A2,

PSD403/413A1,PSD403/413A2,PSD501/511B1,PSD502/512B1,

PSD503/513B1.

Xilinx     XC7236/7236A,XC7272/7272A,XC7336,XC7354.