[   ABOUT    GINSTALL  ]

GINSTALL is designed to automatically install your mouse driver

for you.  To use GINSTALL, put the Genius Mouse driver diskette into

Drive A, close the disk drive door, log onto the A drive by typing

A: <Enter>, and type GINSTALL <Enter>.  Then follow the instructions

on the screen.

The main menu below is the GINSTALL support menu.  You can use

keyboard commands or mouse to select the resolution and install path

for your mouse driver, or you can simply click the left mouse button

on OK or press the letter O on your keyboard to accept the default

values.  Further details are provided below.

�������������������� [ Install Menu ]����������������������

�                                                         �

� ���< Install Mouse Port >����������������������������ͻ �

� �                                                     � �

� �     [ ]  COM1      [ ]  COM2     [ ]  PS/2 PORT     � �

� �                                                     � �

� �     [ ]  COM3      [ ]  COM4     [ ]  BUS MOUSE     � �

� �����������������������������������������������������ͼ �

�                                                         �

� ���< Start-up Resolution >�������������ͻ������������ͻ �

� �                                       �� X-Y LOCK   � �

� �  [A] 50 dpi  [B] 100 dpi  [C] 200 dpi ��            � �

� �                                       �� ��Ŀ       � �

� �  [D] 400 dpi [E] 600 dpi  [F] 800 dpi �� �ON�  OFF  � �

� �                                       �� ����       � �

� ���������������������������������������ͼ������������ͼ �

�                                                         �

� ������������������������������������������������������� �

� �  Install Path :>� C:\GMOUSE                         � �

� ������������������������������������������������������� �

�                                                         �

�               ������ķ            ������ķ              �

�               �  OK  �            � QUIT �              �

�               ������Ľ            ������Ľ              �

�����������������������������������������������������������           #

[ NOTES ]

<A>  The available keyboard commands for the main menu are :

"A" for 50 dpi, "B" for 100 dpi, "C" for 200 dpi, etc .

"X" to turn on or off X-Y Lock function.

"P" for Install Path.  You can edit the path statement to

choose a different path for installation of the

mouse driver.

"O" to accept the settings.

"Q" to Quit the program without installing the mouse driver.

For systems without hard disks, use the Keyboard commands to

install the mouse driver.  After the installation, reboot

your system and the mouse driver will automatically install

itself.

<B>  This program will modify your CONFIG.SYS and AUTOEXEC.BAT

files.  If you do not currently have these files in your system,

GINSTALL will create a CONFIG.SYS file and install the Genius

Mouse Driver in that file.

Your original CONFIG.SYS and AUTOEXEC.BAT files will be saved

under the filenames CONFIG.OLD and AUTOEXEC.OLD.

Note:  Each time you run GINSTALL, it will update your mouse

driver command line in your config.sys, therefore,

replacing your previous mouse driver command line.

<C>  What to do if GINSTALL locks up ?

If GINSTALL locks up, it probably means that a system interrupt

problem exists between your serial ports.  Contact your dealer

if necessary to resolve the problem, then re-run GINSTALL.

<D>  You may load GINSTALL manually if you wish.  Please

review the pertinent section of the Users Manual for details.