PROGRAMMER'S  REFERENCE  FOR  GENIUS  MOUSE  DRIVER

*** 1 : BRIEF DESCRIPTION

The Genius Mouse Driver enables you to use mouse hardware to move an on-screen

cursor and control its movement through a software program.  Various functions

allow you to determine cursor placement, cursor shape, and button status.

In order for you to interface your Genius Mouse with an application program, the

following information on the Genius Driver has been provided.

*** 2 : GRAPHICS AND TEXT CURSORS

GMOUSE Driver supports a hardware text cursor,  a software  text   cursor, and

a graphics cursor. A hardware text cursor is a blinking cursor which moves from

one character to another on-screen. This blinking cursor may take the form of a

block or underscore. A software text cursor makes use of display attributes to

change the visual appearance of a character on-screen.  Movement is from

character to character. A graphics cursor is a shape that moves over on-screen

images.

You can choose any of these three cursors to use on-screen, however, only one

cursor can be displayed at a given time.  Also, within your application program,

you can switch back and forth between cursors.

Display the Graphics Cursor

The cursor appears on-screen or disappears from the screen through the calling

program.  This cursor consists of a block of pixels.  As this block moves

on-screen and affects the pixels beneath it, the cursor shape and background are

created. This interaction is defined by two 16-by-16 bit arrays;

one is the screen mask and the other is the cursor mask. The screen mask

determines what part of the cursor pixel is to be the shape, and what part  is

is to be the background. The cursor mask determines which pixels contribute to

the color of the cursor. Whenever changes are made to the screen which lie

directly  beneath the cursor, the cursor should be concealed so that old values

are not restored to the screen.

Please note that with a high resolution mode, you have a 16-by-16 pixel block;

with a medium resolution (four color) mode, you have a 8-by-16 pixel block; with

a medium resolution (sixteen color) mode, you have a 4-by-16 pixel block.

Refer to function 9.

To create the cursor, the software uses data from the computer's screen memory

which defines the color of each pixel on-screen.  Operations are performed that

affect individual screen bits.  Software ANDs the screen mask defining the

pixels under the cursor and XORs the cursor mask with the result of the AND

operation.

Note the results when:

page 1

Screen Mask Bit is      Cursor Mask Bit is      Resulting Screen Bit is

------------------      ------------------      -----------------------

0                        0                         0

0                        1                         1

1                        0                     unchanged

1                        1                      inverted

With each mouse function, a reference to the graphics cursor location is in

reference to a point on-screen directly beneath the cursor.  This point that the

mouse software uses to determine the cursor coordinates is known as the cursor's

hot spot.

Generally, the upper_left hand corner of the cursor block is designated as the

coordinates for the cursor default value. ((0,0) are the upper_left hand corner

coordinates.)

Software Text Cursor

You can use this text cursor when your computer is in one of the text modes.  By

changing the character attributes beneath the cursor, the appearance of the

character is influenced on-screen.  This effect on the text cursor can be

defined by two 16-bit mask values. These bits can be described as follows:

bit 15 sets the blinking (1) or non-blinking (0) character ; bit 12 - 14 set the

background (1); bits 8 - 10 set the foreground color; and bits 0 - 7 set the

character code. These values in the screen mask and the cursor mask

determine the character's new attributes when the cursor is covering the

character.  The screen mask decides which of the character's attributes are

maintained. The cursor mask decides in what manner the attributes are altered

to produce the cursor.

In creating this cursor, the software works from data which defines each

character on the screen.  The software first ANDs the screen mask and the screen

data bit for the character beneath the cursor.  Next, the software XORs the

cursor mask and the result of the AND operation.

When a function refers to the text cursor location, it gives the coordinates of

the character beneath the cursor.

Refer  to function 10.

Hardware Text Cursor

This cursor is also available when the computer is in one of the text modes.

This cursor is the one seen on-screen when the computer is powered on.  It

consists of 8 pixels wide and 8 to 14 pixels tall.  Software allows you to use

this cursor for your needs. Scan lines determine a cursor's appearance

on-screen. A scan line consists of a horizontal set of pixels.

If a line is on, there will be flashing on the screen. If a line is off, there

is no effect. Scan lines are numbered from 0 to 7, or 0 to 11 depending on the

type of display used. 0 indicates the top scan line.

Refer to function 10.

*** 2.1 : Mouse Buttons

page 2

Mouse functions can give the status of the mouse buttons and the number of times

a certain button has been pressed and released.  The button status is given as

an integer. If a bit is set to 1 the button is down; if a bit is set to 0, the

button is up.

Bit 0 - Left Button Status

Bit 1 - Right Button Status

Bit 2 - Middle Button Status

Each time a mouse button is pressed, a counter records the number of presses and

releases.  The software sets the counter to zero once it has been read or after

a reset.

*** 2.2 : Unit of Distance - Mouse Motion

The motion of the mouse can be expressed in a unit of distance (mouse motion)

and is approximately 1/200 of an inch.

With mouse movement, mouse software determines a horizontal and vertical mouse

motion count.  This count is used by the software to move a cursor a certain

number of pixels on-screen.  Software defines mouse motion sensitivity (the

number of mouse motions needed to move the cursor 8 pixels on-screen) and this

sensitivity determines the rate at which the cursor moves on-screen.

Refer to function 15.

*** 2.3 : Internal Cursor Flag

Mouse software supports an internal flag.  This flag determines when the cursor

should appear on-screen.  If the flag equals 0, the cursor appears on-screen; if

the flag is any other number, the cursor disappears from the screen.

You can call functions 1 and 2 a number of times, however, if you call function

2, you must call function 1 later.  This is necessary to restore the flag's

previous value.

Refer to functions 1 and 2.

*** 3 : CALLING FROM ASSEMBLY LANGUAGE PROGRAMS

To make mouse function calls:

Load the appropriate registers (AX, BX, CX, DX) with the parameter values.

These correspond to G1%, G2%, G3%, and G4% as shown in the BASIC example to

follow.  Then execute software interrupt 51 (33H).  The values given by the

mouse functions will be installed in the registers.

Example:

; * set cursor to location (150,100)

Mov AX,4    ;(function call 4)

Mov CX,150  ;(set horizontal to 150)

Mov DX,100  ;(set vertical to 100)

Int 51(33H) ;(interrupt to mouse)

It is important to note that before using INT 33H, one should verify the

presence of the mouse driver.  Executing an INT 33H will cause uncertain results

if the mouse driver is not loaded.  Assume a mouse driver is present when INT

33H vector is non-zero and the vector does not point to an IRET instruction.

page 3

Note:  When making a mouse call in Assembly Language, expect somewhat of a

different value for the fourth parameter (when compared with calls using a BASIC

program) involving functions 9, 12, and 16.

*** 4 : CALLING FROM BASIC LANGUAGE PROGRAM

To make  mouse function calls:

Set a pair of integer variables in your program for the offset and the segment

of the mouse driver entry point.

In order to obtain the offset and segment values, the following statements

must be inserted into your program before any calls to mouse functions:

10 DEF SEG = 0

15 ' GET GMOUSE ENTRY POINT

20 GMSEG   = PEEK( 51*4 + 2 ) + 256 * PEEK( 51*4 + 3 )   ' GET SEGMENT ENTRY

30 GMOUSE  = 2 + PEEK( 51*4 ) + 256 * PEEK( 51*4 + 1 )   ' GET OFFSET  ENTRY

40 DEF SEG = GMSEG           ' SET SEGMENT REGISTER AS THE SEGMENT OF GMOUSE

To enter the mouse driver, use the CALL statement:

CALL GMOUSE (G1%, G2%, G3%, G4%)

GMOUSE contains the entry offset of the mouse driver.  G1%, G2%, G3%, and G4%

are the integer variables given in the call.  These four must be specified in

the CALL statement even if a value is not assigned.  When a value is assigned,

it must be an integer, that is, a whole number.

Example:

50  ' Find the Activated Mode of Genius Mouse

60  G1% = 0 : G2% = 0

70  CALL GMOUSE ( G1%, G2%, G3%, G4% )

80  IF G2% AND 2 THEN PRINT "Genius Mouse ( 2_Button Mode ) Enable"

90  IF G2% AND 3 THEN PRINT "Genius Mouse ( 3_Button Mode ) Enable"

100 IF NOT G1%   THEN PRINT "Can't Find Genius Mouse"

*** 5 : MOUSE FUNCTIONS

These functions listed apply to the Genius Mouse.  Further descriptions of each

mouse function will be given in the following pages.

Functions                                         Function Number

-----------------------------------------------------------------

Reset Genius Mouse Driver                                 0

Enable Cursor Display                                     1

Disable Cursor Display                                    2

Read Cursor Location & Button State of Genius Mouse       3

Set Cursor Location of Genius Mouse                       4

Read Button Press State of Genius Mouse                   5

Read Button Release State of Genius Mouse                 6

Define Horizontal (X) Range of Cursor Location            7

Define Vertical (Y) Range of Cursor Location              8

Define Graphics Mode Cursor Style                         9

Define Text Mode Cursor Style                            10

Read Genius Mouse Motion Number                          11

page 4

Define Event Handler Entry Location                      12

Enable Light Pen Emulation Function                      13

Disable Light Pen Emulation Function                     14

Define Sensitivity (Mouse Motion/Pixel) of Genius Mouse  15

Disable Cursor Display in Special Range                  16

Define Double-Speed Threshold                            19

Swap Event Handler Entry Location                        20

Get Mouse Driver State Storage Size                      21

Save Mouse Driver state                                  22

Restore Mouse Driver state                               23

Define Alternate User Subroutine                         24

Read Alternate Subroutine Vector                         25

Set Mouse Sensitivity                                    26

Get Mouse Sensitivity                                    27

Set CRT Page Number                                      29

Read CRT Page Number                                     30

Disable Mouse                                            31

Enable Mouse                                             32

Software Reset Genius Mouse Driver                       33

Get Language Byte                                        35

Get Mouse and Driver Information                         36

EGA functions are described in Section *** 7.

*** 6 : DESCRIPTION OF THE MOUSE FUNCTIONS

You'll notice that with the following mouse function descriptions, the

parameters needed to make the calls and the expected outcome (return) for each

is indicated. Also,  any special conditions regarding any of the mouse functions

have been included.  Further, an example of a program has been provided in order

for you to understand how to make the call.

The input and return values are presented for 8086 registers and for BASIC in

the following pages.

It is important to note that each mouse function call needs four parameters.

The Genius Mouse software does not verify any input values, and therefore, if

any incorrect values are given, uncertain results will occur.

Function 0: Reset Genius Mouse Driver

Function 0 gives the current status of the mouse hardware plus the current

status of the mouse software.  The calling program is able to determine the

presence of a mouse driver and/or a serial port.

This function resets the mouse driver to the following default status as

indicated:

Variable                                Value

------------------------------------------------------------------------------

internal cursor flag                    -1 (cursor concealed)

graphics  cursor shape                  horizontal oval

text cursor                             reverse video

user-defined call mask                  all zeroes

light pen emulation mode                enabled

vertical mouse motion/pixel ratio       16 to 8

horizontal mouse motion/pixel ratio     8 to 8

vertical min/max cursor coordinates     0/current display mode y values minus 1

horizontal min/max cursor coordinates   0/current display mode x values minus 1

page 5

8086 Register

Input:  AX = 0

Return: AX = mouse state (-1: installed, 0: not installed)

BX = number of buttons (2 button mode, 3 button mode)

BASIC

Input:  G1% = 0

Return: G1% = mouse state (-1: installed, 0: not installed)

G2% = number of buttons (2 button mode, 3 button mode)

Example:  Used initially to determine if the GMOUSE driver is present and to

reset GMOUSE.

50  ' Find the Actived Mode of Genius Mouse

60  G1% = 0 : G2% = 0

70  CALL GMOUSE ( G1%, G2%, G3%, G4% )

80  IF G2% AND 2 THEN PRINT "Genius Mouse ( 2_Button Mode ) Enable"

90  IF G2% AND 3 THEN PRINT "Genius Mouse ( 3_Button Mode ) Enable"

100 IF NOT G1%   THEN PRINT "Can't Find Genius Mouse"

Function 1: Enable Cursor Display

Function 1 increments the internal cursor flag counter.  If the counter is zero,

the cursor is enabled and appears on-screen.

The default value is -1 which indicates a concealed cursor.  Function 1 must be

called to display the cursor.  In case the internal cursor flag is already zero,

a call to this function produces no effect.

8086 Register

Input:  AX = 1

Return: none

BASIC

Input:  G1% = 1

Return: none

Example:

110  ' Enable Genius Mouse's Cursor

120  G1% = 1

130 CALL GMOUSE ( G1%, G2%, G3%, G4% )

Function 2: Disable Cursor Display

Function 2 disables the cursor by removing it from the screen and decrementing

the internal cursor flag.  Even though the cursor cannot be seen, it still

tracks any motion made with the mouse.

You should use this function before changing any portion of the screen

containing the cursor.  You will avoid the problem of the cursor affecting

screen data.

Keep in mind that whenever your program calls function 2, it must later call

function 1 to return the internal cursor flag to its default value.  In

addition, if your program changes the screen mode, function 2 is called

automatically. Therefore, the cursor's movement is enabled the next time it is

displayed.

Call function 2 at the end of a program in order to conceal the cursor.  This

ensures that nothing remains on-screen.

page 6

8086 Register

Input:  AX = 2

Return: none

BASIC

Input:  G1% = 2

Return: none

Example:

110  ' Disable Genius Mouse's Cursor

120  G1% = 2

130 CALL GMOUSE ( G1%, G2%, G3%, G4% )

Function 3: Read Cursor Location & Button State of Genius Mouse

Function 3 gives the status of mouse buttons, plus cursor location.

Button status consists of a single integer value:

Bit 0 = left button (2 button mode, 3 button mode)

Bit 1 = right button (2 button mode, 3 button mode)

Bit 2 = middle button (3 button mode)

The bit is 1 when the button is pressed.  The bit is 0 when the button is

released.

8086 Register

Input:  AX = 3

Return: BX = button status

CX = horizontal cursor coordinate

DX = vertical cursor coordinate

BASIC

Input:  G1% = 3

Return: G2% = button status

G3% = horizontal cursor coordinate

G4% = vertical cursor coordinate

Example:

110  ' Read Genius Mouse Location & Button State

120  G1% = 3

130 CALL GMOUSE ( G1%, G2%, G3%, G4% )

140 PRINT "Genius Mouse Location : X_Coord=" G3% " Y_Coord=" G4%

150 IF G2% AND 1 THEN PRINT "Left Button"

160 IF G2% AND 2 THEN PRINT "Right Button"

170 IF G2% AND 4 THEN PRINT "Middle Button"

180 PRINT "Pressed"

Function 4: Set Cursor Location of Genius Mouse

Function 4 sets the current cursor location.  Values must be within the

coordinate ranges for the virtual screen and, if necessary, are rounded to the

nearest values allowed for the current screen mode.

page 7

Screen     Display        Virtual         Cell        Bits/Pixel

Mode      Adapter       Screen (XxY)     Size      Graphics Mode

---------  ------------  ---------------  --------   ----------------

0      C, E, 3270     640 x 200        16 x 8          -

1      C, E, 3270     640 x 200        16 x 8          -

2      C, E, 3270     640 x 200         8 x 8          -

3      C, E, 3270     640 x 200         8 x 8          -

4      C, E, 3270     640 x 200         2 x 1          2

5      C, E, 3270     640 x 200         2 x 1          2

6      C, E, 3270     640 x 200         1 x 1          1

7      M, E, 3270     640 x 200         8 x 8          -

D      E              640 x 200        16 x 8          2

E      E              640 x 200         1 x 1          1

F      E              640 x 350         1 x 1          1

10     E              640 x 350         1 x 1          1

30     3270           720 x 350         1 x 1          1

H              720 x 348         1 x 1          1

Display Adapter:

M = IBM Monochrome Display/Printer Adapter

C = IBM Color/Graphics Adapter

E = IBM Enhanced Graphics Adapter

3270 = IBM All Points Addressable Graphics Adapter (3270 PC)

H = Hercules Monochrome Graphics Card

8086 Register

Input:  AX = 4

CX = new horizontal cursor coordinate

DX = new vertical cursor coordinate

Return: none

BASIC

Input:  G1% = 4

G3% = new horizontal cursor coordinate

G4% = new vertical cursor coordinate

Return: none

Example:

110  ' Set Cursor Location at the Upper_Left Corner of Screen

120 G1% = 4

130 G3% = 0 : G4% = 0

140 CALL GMOUSE ( G1%, G2%, G3%, G4% )

Function 5: Read Button Press State of Genius Mouse

Function 5 provides status on the specified button, gives the number of button

presses since the last call, and produces the location of the cursor at last

button press.

Button status consists of a single integer value.  Again, as in function 3:

Bit 0 = left button (2 button mode, 3 button mode)

Bit 1 = right button (2 button mode, 3 button mode)

Bit 2 = middle button (3 button mode)

The bit is 1 when the button is pressed.  The bit is 0 when the button is

released.

page 8

The number of button presses will always fall in the range of 0 to 32767.  There

is no indicator for overflow.  Following this function call, the count is reset

to zero.

8086 Register

Input:  AX = 5

BX = button status (left = 0, right = 1, middle = 2)

Return: AX = button status

BX = number of button presses

CX = horizontal cursor coordinate at last press

DX = vertical cursor coordinate at last press

BASIC

Input:  G1% = 5

G2% = button status (left = 0, right = 1, middle = 2)

Return: G1% = button status

G2% = number of button presses

G3% = horizontal cursor coordinate at last press

G4% = vertical cursor coordinate at last press

Example:

110 ' Read the Left Button Press State of Genius Mouse

120 G1% = 5 : G2% = 2

130 CALL GMOUSE ( G1%, G2%, G3%, G4% )

140 IF G1% AND 2 THEN PRINT "The Middle Button Pressed at X_loc=" G3%

Function 6: Read Button Release State of Genius Mouse

Function 6 provides status on the specified button, gives the number of button

releases since the last call, and provides the location of the cursor at the

last button release.

Button status consists of a single integer value.  Again, as in function 3:

Bit 0 = left button (2 button mode, 3 button mode)

Bit 1 = right button (2 button mode, 3 button mode)

Bit 2 = middle button (3 button mode)

The bit is 1 when the button is pressed.  The bit is 0 when the button is

released.

The number of button releases will always fall in the range of 0 to 32767.

There is no indicator for overflow.  Following this function call, the count is

reset to zero.

8086 Register

Input:  AX = 6

BX = button status (left = 0, right = 1, middle = 2)

Return: AX = button status

BX = number of button releases

CX = horizontal cursor coordinate at last release

DX = vertical cursor coordinate at last release

BASIC

Input:  G1% = 6

G2% = button status (left = 0, right = 1, middle = 2)

page 9

Return: G1% = button status

G2% = number of button releases

G3% = horizontal cursor coordinate at last release

G4% = vertical cursor coordinate at last release

Example:

110 ' Read the Left Button Release State of Genius Mouse

120 G1% = 6 : G2% = 2

130 CALL GMOUSE ( G1%, G2%, G3%, G4% )

140 IF NOT G1% OR &HFFFB THEN PRINT "The Middle Button Released at X_loc=" G3%

Function 7: Define Horizontal (X) Range of Cursor Location

Function 7 defines the horizontal range of the cursor on-screen.  As a result,

cursor movement is limited to this specified area.  If a cursor happens to be

outside of this area when a call is made, the cursor is moved to just inside the

area.

8086 Register

Input:  AX = 7

CX = minimum horizontal cursor coordinate

DX = maximum horizontal cursor coordinate

Return: none

BASIC

Input:  G1% = 7

G3% = minimum horizontal cursor coordinate

G4% = maximum horizontal cursor coordinate

Return: none

Example:

110 ' Enable Cursor in Horizontal Range between 100 to 200

120 G1% = 7

130 G2% = 100 : G3% = 200

140 CALL GMOUSE ( G1%, G2%, G3%, G4% )

Function 8: Define Vertical (Y) Range of Cursor Location

Function 8 defines the vertical range of the cursor on-screen.  As a result,

cursor movement is limited to this specified area.  If a cursor happens to be

outside of this area when a call is made, the cursor is moved to just inside the

area.

8086 Register

Input:  AX = 8

CX = minimum vertical cursor coordinate

DX = maximum vertical cursor coordinate

Return: none

BASIC

Input:  G1% = 8

G3% = minimum vertical cursor coordinate

G4% = maximum vertical cursor coordinate

Return: none

page 10

Example:

110 ' Enable Cursor in Vertical Range between 100 to 200

120 G1% = 8

130 G2% = 100 : G3% = 200

140 CALL GMOUSE ( G1%, G2%, G3%, G4% )

Function 9: Define Graphics Mode Cursor Style

Function 9 defines the style of the cursor in terms of color, shape, and center

for the graphics.  As mentioned before, this cursor is a 16-by-16 pixel block

and is defined by two 16-bit arrays (the screen mask bit and the cursor mask

bit). Cursor coordinates for the hot spot must be in the range of -16 to +16.

8086 Register

Input:  AX = 9

BX = horizontal cursor hot spot

CX = vertical cursor hot spot

DX = pointer to screen and cursor mask

Return: none

BASIC

Input:  G1% = 9

G2% = horizontal cursor hot spot

G3% = vertical cursor hot spot

G4% = pointer to screen and cursor mask

Return: none

Example:

10  ' Define the screen mask

20  '

30    cursor (0,0)  = &HFFFF       '1111111111111111

40    cursor (1,0)  = &HFFFF       '1111111111111111

50    cursor (2,0)  = &HFFFF       '1111111111111111

60    cursor (3,0)  = &HFFFF       '1111111111111111

70    cursor (4,0)  = &HFFFF       '1111111111111111

80    cursor (5,0)  = &HF00F       '1111000000001111

90    cursor (6,0)  = &H0000       '0000000000000000

100   cursor (7,0)  = &H0000       '0000000000000000

110   cursor (8,0)  = &H0000       '0000000000000000

120   cursor (9,0)  = &H0000       '0000000000000000

130   cursor (10,0) = &HF00F       '1111000000001111

140   cursor (11,0) = &HFFFF       '1111111111111111

150   cursor (12,0) = &HFFFF       '1111111111111111

160   cursor (13,0) = &HFFFF       '1111111111111111

170   cursor (14,0) = &HFFFF       '1111111111111111

180   cursor (15,0) = &HFFFF       '1111111111111111

190 '

200 ' Define the cursor mask

210 '

page 11

220   cursor (0,1)  = &H0000       '0000000000000000

230   cursor (1,1)  = &H0000       '0000000000000000

240   cursor (2,1)  = &H0000       '0000000000000000

250   cursor (3,1)  = &H0000       '0000000000000000

260   cursor (4,1)  = &H0000       '0000000000000000

270   cursor (5,1)  = &H0000       '0000000000000000

280   cursor (6,1)  = &H07E0       '0000011111100000

290   cursor (7,1)  = &H7FFE       '0111111111111110

300   cursor (8,1)  = &H7FFE       '0111111111111110

310   cursor (9,1)  = &H07E0       '0000011111100000

320   cursor (10,1) = &H0000       '0000000000000000

330   cursor (11,1) = &H0000       '0000000000000000

340   cursor (12,1) = &H0000       '0000000000000000

350   cursor (13,1) = &H0000       '0000000000000000

360   cursor (14,1) = &H0000       '0000000000000000

370   cursor (15,1) = &H0000       '0000000000000000

380 '

390 ' Set the cursor style and hot spot number of Genius Mouse

400 '

410 '

420   G1% = 9

430   G2% = 6 ' horizontal hot spot

440   G3% = 5 ' vertical hot spot

450   CALL GMOUSE ( G1%, G2%, G3%, cursor (0,0))

Function 10: Define Text Mode Cursor Style

Function 10 chooses the hardware or the software text cursor.

For example, if BX (G2%) is 1, the hardware cursor is selected and the hardware

is set up with the first and last scan lines which define the cursor.

(Values for CX (G3%) and DX (G4%) range from 0 to 7 for the color display and 0

to 11 for the monochrome display.)

If BX (G2%) is 0, the software cursor is selected; and CX (G3%) and DX (G4%)

must specify the screen and cursor masks.  (These masks give the attributes and

character code of the cursor, and their values are dependent on the type of

display in use.)

8086 Register

Input:  AX = 10

BX = select cursor (0: software text, 1: hardware text)

CX = screen mask value/scan line start

DX = cursor mask value/scan line stop

Return: none

BASIC

Input:  G1% = 10

G2% = select cursor (0: software text, 1: hardware text)

G3% = screen mask value/scan line start

G4% = cursor mask value/scan line stop

Return: none

Example:

page 12

110 ' Enable an Inverting Cursor

120 G1% = 10

130 G2% = 0

140 G3% = &HFFFF  :  G4% = &H7700

150 CALL GMOUSE ( G1%, G2%, G3%, G4% )

Function 11: Read Genius Mouse Motion Number

Function 11 gives the mouse motion number since the last call.  A positive

horizontal number indicates rightward movement (negative shows leftward

movement).  A positive vertical number indicates downward movement (negative

shows upward movement).

The number is always in the range of -32768 to 32767.  Overflow is disregarded.

Once the call is completed, the number is set to 0.

8086 Registers

Input:  AX = 11

Return: CX = horizontal number

DX = vertical number

BASIC

Input:  G1% = 11

Return: G3% = horizontal number

G4% = vertical number

Example:

110 ' Read Genius Mouse Motion Number

120 G1% = 11

130 CALL GMOUSE ( G1%, G2%, G3%, G4% )

140 IF G3% > 0 THEN PRINT "Genius Mouse is Moving to Right"

150 IF G4% > 0 THEN PRINT "Genius Mouse is Moving Down"

Function 12: Define Event Handler Entry Location

Function 12 defines the address entry location of an event handler routine which

is called when a certain event (defined by the call mask) occurs.  The program

is temporarily interrupted by the mouse driver. At the end of the event handler

routine  the program continues at the point it was interrupted.

The call mask is a single integer value defining the conditions which will cause

an interrupt.

A specific condition corresponds to a bit in the call mask:

Mask Bit                Condition

--------------------------------------------------

0                    cursor location changed

1                    left button pressed

2                    left button released

3                    right button pressed

4                    right button released

5                    middle button pressed

6                    middle button released

7 - 15               not used

page 13

In order to call the event handler routine, set the mask bit to 1 and put the

mask in at CX (G3%).  To disable, set the mask bit to 0 and put the mask in at

CX (G3%).  Always be sure to set the call mask to 0 before the program finishes.

(Leave the system in the same state upon exit as if was upon entrance.)

8086 Register

Input:  AX = 12

CX = call mask

ES:DX = pointer to event handler routine

Return: none

BASIC

Input:  G1% = 12

G3% = call mask

G4% = pointer to event handler routine

Return: none

Example:

110 ' Active BUTTDOWN Event Handler Routine, When One or More Buttons Pressed

120 G1% = 12

130 G3% = &H002A  :  G4% = BUTTDOWN%

140 CALL GMOUSE ( G1%, G2%, G3%, G4% )

Function 13: Enable Light Pen Emulation Function

Function 13 permits the mouse to act like a light pen.  When in this mode, calls

to the pen function will give the cursor coordinates at the last pen down

location.

Note that the status of "pen down" and "pen off-screen" is controlled by the

mouse buttons:  all buttons up, pen off-screen; one button pressed, pen down.

Light pen emulation is ON after each call to function 0 (Reset Mouse Driver).

8086 Register

Input:  AX = 13

Return: none

BASIC

Input:  G1% = 13

Return: none

Example:

110 ' Enable Light Pen Emulation Function

120 G1% = 13

130 CALL GMOUSE ( G1%, G2%, G3%, G4% )

Function 14: Disable Light Pen Emulation Function

Function 14 turns off the light pen emulation mode.  When disabled, any call to

the pen function will give information only about a real light pen.

8086 Register

Input:  AX = 14

page 14

Return: none

BASIC

Input:  G1% = 14

Return: none

Example:

110 ' Disable Light Pen Emulation Function

120 G1% = 14

130 CALL GMOUSE ( G1%, G2%, G3%, G4% )

Function 15: Define Sensitivity (Mouse Motion/Pixel) of Genius Mouse

Function 15 defines mouse sensitivity as determined by the mouse motion/pixel

ratio.  This is a way of setting the amount of cursor motion wanted for mouse

movement.  These ratios specify mouse motion per 8 pixels.  These values must

be in the range of 1 to 32767.  With a larger ratio, the cursor movement is

shortened for each mouse movement.

Default values:   horizontal ratio - 8  mouse motions to 8 pixels

vertical ratio   - 16 mouse motions to 8 pixels

Note: 1 mouse motion = 1/200 of an inch increment

8086 Register

Input:  AX = 15

CX = horizontal mouse motion counts to pixel ratio

DX = vertical mouse motion counts to pixel ratio

Return: none

BASIC

Input:  G1% = 15

G3% = horizontal mouse motion counts to pixel ratio

G4% = vertical mouse motion counts to pixel ratio

Return: none

Example:

110 ' Define Horizontal Sensitivity as 8

120 ' Define Vertical Sensitivity as 16

130 G1% = 15

140 G3% =  8

150 G4% = 16

160 CALL GMOUSE ( G1%, G2%, G3%, G4% )

Function 16: Disable Cursor Display in Special Range

Function 16 sets up a special range on-screen.  If the cursor moves to this area

or is in this area, it will be disabled.  After a call is made to this function,

it is necessary to call function 1 to enable the cursor again.

Define the special range with screen location values using four components:

page 15

Components          Values

--------------------------------------------------------

1               Left horizontal screen location

2               Upper vertical screen location

3               Right horizontal screen location

4               Lower vertical screen location

8086 Register

Input:  AX = 16

ES:DX = pointer to special range

Return: none

BASIC

Input:  G1% = 16

G4% = pointer to special range

Return: none

Example:

110 ' Disable Cursor Display in (0,0) to (100,100) Range

120 G1% = 16

130 RANGE%(1) = 0   : RANGE%(2) = 0

140 RANGE%(3) = 100 : RANGE%(4) = 100

150 CALL GMOUSE ( G1%, G2%, G3%, RANGE%(0) )

.

.

.

500 ' Enable Cursor Display Again

510 G1% = 1

520 CALL GMOUSE ( G1%, G2%, G3%, G4% )

Function 19: Define Double-Speed Threshold

Function 19 defines the threshold value (mouse motion per second) for doubling

the cursor's motion.  Should the mouse move faster than the DX (G4%) value, the

cursor motion doubles.  The default value is 64 mouse motions per second.

If you should want to disable double-speed, just set the threshold to 32767

(7FFFH) mouse motions/second.

8086 Register

Input:  AX = 19

DX = threshold speed in mouse motions/second

Return: none

BASIC

Input:  G1% = 19

G4% = threshold speed in mouse motions/second

Return: none

Example:

110 ' Define Double-Speed Threshold as 20 Mouse Motions/Second

page 16

120 G1% = 19

130 G4% = 20

140 CALL GMOUSE ( G1%, G2%, G3%, G4% )

.

.

.

500 ' Disable Double-Speed Threshold Function

510 G1% = 19

520 G4% = 256 'MAX. VALUE

530 CALL GMOUSE ( G1%, G2%, G3%, G4% )

Function 20: Swap Event Handler Entry Location

Function 20 sets new values for the call mask and event handler routine

address  for mouse  hardware interrupts and return the values that were

previously specified.

For detail information to reference Function 12 description.

8086 Register

Input:  AX = 20

CX = new call mask

ES:DX = new pointer to event handler routine

Return: CX = old call mask

ES:DX = old pointer to event handler routine

BASIC

Input:  G1% = 20

G3% = call mask

G4% = pointer to event handler routine

Return: G3% = old call mask

G4% = old pointer to event handler routine

Example:

100 ' Swap Event Handler Entry Location

110 ' Active BUTTDOWN Event Handler Routine, When One or More Buttons Pressed

120 G1% = 20

130 G3% = &H002A  :  G4% = BUTTDOWN%

140 CALL MOUSE ( G1%, G2%, G3%, G4% )

Function 21: Get Mouse Driver State Storage Size

Function 21 returns the size of the buffer required to store the current state

of the  mouse  driver.  It is used with  functions 22  and 23 when you want to

temporarily  interrupt a program  that is using the mouse  and execute another

that also uses the mouse.

8086 Register

Input:  AX = 21

Return: BX = buffer size required for mouse driver state

BASIC

Input:  G1% = 21

page 17

Return: G2% = buffer size required for mouse driver state

Example:

110 ' Get Mouse Driver State Storage Size

120 G1% = 21

130 CALL MOUSE ( G1%, G2%, G3%, G4% )

140 STATESIZE% = G2%

Function 22: Save Mouse Driver state

Function 22 saves the current mouse driver state in a buffer allocated by your

program.   It is used with  functions 21  and 23  when you want to temporarily

interrupt a program  that is using the mouse  and execute another program that

also uses the mouse.

Before your program  calls function 22, the program should call function 21 to

determine  the buffer size required  for saving the  mouse driver state,  then

allocate the appropriate amount of memory.

8086 Register

Input:  AX = 22

ES:DX = pointer to the buffer

Return: None

BASIC

Input:  G1% = 22

G4% = pointer to the buffer

Return: None

Example:

110 ' Save The Mouse Driver State

120 G1% = 22

130 G4% =  BUFPTR

140 ' Assume BUFPTR contains the address of the buffer

150 CALL MOUSE ( G1%, G2%, G3%, G4% )

Function 23: Restore Mouse Driver State

Function 23 restores the last  mouse driver state saved by function 22.  It is

used with functions 21 and 22 when you want to temporarily interrupt a program

that is  using the mouse  and execute another program that also uses the mouse.

To restore the mouse driver state saved by function 22, call function 23 at the

end of the interrupt program.

8086 Register

Input:  AX = 23

ES:DX = pointer to the buffer

Return: None

BASIC

Input:  G1% = 23

G4% = pointer to the buffer

Return: None

Example:

page 18

110 ' Restore The Mouse Driver State

120 G1% = 23

130 G4% =  BUFPTR

140 ' Assume BUFPTR contains the address of the buffer

150 CALL MOUSE ( G1%, G2%, G3%, G4% )

Function 29: Set CRT Page Number

Function 29 specifies the CRT page on which the mouse cursor will be displayed.

For information on the number of CRT pages available in each display mode your

adapter supports, see the documentation that came with the graphics adapter.

8086 Register

Input:  AX = 29

BX = CRT page for mouse cursor display

Return: none

BASIC

Input:  G1% = 29

G2% = CRT page for mouse cursor display

Return: none

Example:

110 ; Set CRT page 2 for display mouse cursor

120 G1% = 29

130 G2% = 2

140 CALL MOUSE ( G1%, G2%, G3%, G4% )

.

.

.

500 ; Enable Cursor Display Again

510 G1% = 1

520 CALL MOUSE ( G1%, G2%, G3%, G4% )

Function 30: Read CRT Page Number

Function 30 returns the number of the CRT page on which the mouse cursor is

displayed.

8086 Register

Input:  AX = 30

Return: BX = CRT page number of current cursor display

BASIC

Input:  G1% = 30

Return: G2% = CRT page number of current cursor display

Example:

110 ; Read CRT page number

120 G1% = 30

130 CALL MOUSE ( G1%, G2%, G3%, G4% )

page 19

*** 7 : USING GENIUS MOUSE WITH IBM ENHANCED GRAPHICS ADAPTER

Within the Genius Mouse driver, you'll find nine EGA functions.  These functions

permit your program to write to and read from write-only registers.

The cursor in use is defined as a monochrome cursor with one bit per pixel.  The

bit masks are determined by function 9 and apply to all active planes.

In order to make an EGA function call from an Assembly-Language program, first

load the AX, BX, CX, DX, and ES registers with the values indicated for the

parameters.  Note that five values must be given for a high level language

program.  Next, execute software interrupt 16 (10h). The values that are

returned are intalled in the registers by EGA functions.

Upon start with DOS, PC BIOS will verify if the EGA BIOS exists.  When this is

verified, the PC will execute the EGA BIOS, booting up the program to write the

INT 10h entry vector to the address of the INT 42h vector.  Now, EGA BIOS

address will be written to INT 10h. Following this, you are able to call EGA

BIOS (by using INT 10h) and PC video BIOS (by using INT 42h).

There are twenty functions in EGA BIOS.  (PC BIOS has only 16.) The EGA BIOS

routines only intercept the BIOS ROM video routines (INT 10h, AH = 13h or less).

The following indicates nine EGA functions and the corresponding function

number:

Function                                             Number (HEX)

-----------------------------------------------------------------

Retrieve Single Data                                       F0

Save Single Data                                           F1

Retrieve Registers on a Specified Port                     F2

Save Registers on a Specified Port                         F3

Retrieve Several Registers Data                            F4

Save Several Registers Data                                F5

Reset All Registers as Initial Values                      F6

Set Initial Values                                         F7

Get Version Number of Genius Mouse Driver                  FA

In the above functions, the EGA I/O port number and address are as follows:

Port No.  Register Name   No. of Registers  Index No.  Address Select Register

------------------------------------------------------------------------------

00H      CRT Controller         25           0 - 24            3x4H

08H      Sequencer               5           0 - 4             3C4H

10H      Graphics Controller     9           0 - 8             3CEH

18H      Attribute Controlle    20           0 - 19            3C0H

Singular Registers

20H      Miscellaneous Output    1           ignored           3C2H

28H      Feature Control         1           ignored           3xAH

30H      Graphics 1 Position     1           ignored           3CCH

38H      Graphics 2 Position     1           ignored           3CAH

Note: x = B or D depending on the base I/O address;

determined by Miscellaneous Output Register bit 1.

page 20

Function F0: Retrieve Single Data

This function retrieves data from a single register.

Input:  AH = F0H

BX = Index number

DX = Port number

Return: BL = Retrieved data from EGA register

Example:

FUN_F0     EQU     0f0H         ; Function F0

;

GR_CONTR   EQU     010H         ; Graphics Controller

MODE_REG   EQU     005H         ; Mode Regisiter

;

GR1_PORT   EQU     030H         ; Graphics 1 Position Register

GR2_PORT   EQU     038H         ; Graphics 2 Position Register

;

VIDEO      EQU     010H         ; BIOS ROM Video Routine Entry

; Retrieve the Mode Register in Graphics Controller

MODE_REG   DB      00

;

MOV     DX, GR_CONTR

MOV     BX, MODE_REG

MOV     AH, FUN_F0

INT     VIDEO

MOV     MODE_REG, BL

; Retrieve Graphics 1 Position Data

GR1_POS    DB      00

;

MOV     DX, GR1_POS

MOV     AH, FUN_F0

INT     VIDEO

MOV     GR1_POS, NL

Function F1: Save Single Data

This function saves data to an EGA register.  Upon finishing a call to this

function, the BH and DX values are altered.

Input:  AH = F1H

BL = Index number (Non-single register only)

= Data (Single register only)

BH = Data (Non-single register only)

= Disregard (Single register only)

DX = Port number

Return: None

Example:

page 21

FUN_F1     EQU     0f1H         ; Function F1

;

SEQUENCE   EQU     008H         ; Sequencer

MASK_REG   EQU     002H         ; Map Mask Register

;

FEAT_PORT  EQU     028H         ; Feature Control Register

;

VIDEO      EQU     010H         ; BIOS ROM Video Routine Entry

; Save Map Mask Register of Sequencer

MAP_MASK   EQU     03H

;

MOV     DX, SEQUENCE

MOV     BL, MASK_REG

MOV     BH, MAP_MASK

MOV     AH, FUN_F1

INT     VIDEO

MOV     MAP_MASK, BL

; Save Feature Control Register

FEATURE    DB      02H

;

MOV     DX, FEAT_PORT

MOV     BL, FEATURE

MOV     AH, FUN_F1

INT     VIDEO

MOV     FEATURE, BL

Function F2: Retrieve Registers on a Specified Port

This function retrieves data from registers on a specifi� port.  Upon finishing

a call to this function, the CX value is altered.

Input:  AH = F3H

CH = Starting index number

CL = Number of registers

DX = Port number

ES:BX = Destination of returned data

Return: Returned data to destination address

Example:

FUN_F2     EQU     0f2H         ; Function F2

;

GR_CONTR   EQU     010H         ; Graphics Controller

;

VIDEO      EQU     010H         ; BIOS ROM Video Routine Entry

; Retrieve Four Registers Data from Graphics Controller

GRAPH_POOL DB      04   DUP (0)

;

MOV     DX, DS

MOV     ES, DX

;

page 22

MOV     DX, GR_CONTR

MOV     BX, OFFSET GRAPH_POOL

MOV     CX, 04H

MOV     AH, FUN_F2

INT     VIDEO

Function F3: Save Registers on a Specified Port

This function saves data from registers on a specifi� port.  Upon finishing a

call to this function, the BX, CX, and DX values are altered.

Input:  AH = F3H

CH = Starting index number

CL = Number of register

DX = Port number

ES:BX = Address source of incoming data

Return: None

Example:

FUN_F3     EQU     0f3H         ; Function F3

;

ATTR_CONTR EQU     018H         ; Attribute Controller

;

VIDEO      EQU     010H         ; BIOS ROM Video Routine Entry

; Save Four Registers Data into Attribute Controller

PALET_DATA DB      1, 2, 4, 3

;

MOV     DX, DS

MOV     ES, DX

;

MOV     DX, ATTR_CONTR

MOV     BX, OFFSET PALET_DATA

MOV     CX, 08

MOV     AH, FUN_F3

INT     VIDEO

Function F4: Retrieve Several Registers Data At The Same Time

This function retrieves data from several registers at the same time.  Upon

finishing a call to this function, the CX value is altered.

Input:  AH = F4H

CX = Number of registers (more than 1)

ES:BX = Address of register packet (each consists of 4 bytes;

port  address,  byte 1-2;  index number,  byte 3;

returned data, byte 4)

Return: Returned data is saved into byte 4

Example:

FUN_F4     EQU     0f4H         ; Function F4

;

VIDEO      EQU     010H         ; BIOS ROM Video Routine Entry

page 23

; Retrieve Follow  Registers Data

TABLE       DW     030H         ; Graphics 1 Position Register

DB      00          ; Single Register

DB      00          ; Retrieved Data

;

DW     010H         ; Graphics Controller

DB      05          ; Mode Register

DB      00          ; Retrieved Data

;

;

MOV    DX, DS

MOV    ES, DX

;

MOV    BX, OFFSET TABLE

MOV    CX, 02

MOV    AH, FUN_F4

INT    VIDEO

Function F5: Save Several Registers Data At The Same Time

This function saves data from several registers at the same time.  Upon

finishing a call to this function, the CX value is altered.

Input:  AH = F5H

CX = Number of registers (more than 1)

ES:BX = Address of register packet (each consists of 4 bytes;

port  number, byte 1-2;  index number,  byte 3;

output data, byte 4)

Return: None

Example:

FUN_F5     EQU     0f5H         ; Function F5

;

VIDEO      EQU     010H         ; BIOS ROM Video Routine Entry

; Save Follow Registers Data

TABLE       DW      20H         ; Miscellaneous

DB      00          ; Single Register

DB      01          ; Data

;

DW      18H         ; Attribute Controller

DB      12H         ; Color Plane Enable

DB      07H         ; Data

;

;

MOV    DX, DS

MOV    ES, DX

;

MOV    BX, OFFSET TABLE

MOV    CX, 02

MOV    AH, FUN_F5

INT    VIDEO

page 24

Function F6: Reset All Registers as Initial Values

This function resets all values to default values for the specific registers.

Function 7 sets the default values.

Input:  AH = F6H

Return: None

Example:

FUN_F6     EQU     0f6H         ; Function F6h

;

VIDEO      EQU     010H         ; BIOS ROM Video Routine Entry

MOV     AH, FUN_F6

INT     VIDEO

Function F7: Set Initial Values

This function sets the initial default values.  Upon finishing a call to this

function, the BX and DX values are altered.

Input:  AH = F7H

DX = Port number

ES:BX = Table of output data

Return: None

Example:

FUN_F7     EQU     0f7H         ; Function F7

;

ATTR_CONTR EQU     018H         ; Attribute Controller

;

VIDEO      EQU     010H         ; BIOS ROM Video Routine Entry

; Setting Initial Values for the Attribute Controller

ATTR_DATA  DB      1,  2,  4,  3,  5,  6,  0,  7

DB      0,  0,  0,  0,  0,  0,  0,  0

DB      0,  0, 0fh, 0

;

MOV     DX, DS

MOV     ES, DX

;

MOV     DX, ATTR_CONTR

MOV     BX, OFFSET ATTR_DATA

MOV     AH, FUN_F7

INT     VIDEO

Function FA: Get Version Number of Genius Mouse Driver

This function will give the Genius Mouse driver version number.

Input:  AH = FAH

BX = 00H

Return: ES:BX = Pointer to Genius Mouse driver version number.

page 25