+-------------------------------------------------------+

|     Genius HiPoint Release Notes (version 1.10)       |

|     Copyright (C) KYE Systems Corp. 1992              |

+-------------------------------------------------------+

This document contains information that supplements the

"Genius HiPoint User's Guide" and accompanying software.

CONTENTS

(1). Notes on Setup for DOS and Windows

PATH Modifications

CONFIG Modifications

Operation from Floppy Disk Drive

Using COM3 or COM4 in you system

(2) Modifying POINT.INI

Acceleration Profiles

Orientation

(3). Miscellaneous Items

Loading POINT.INI Settings

=========================================================================

(1). Notes on Setup for DOS and Windows

**********************

* PATH Modifications *

**********************

If you have an older version of the Genius mouse driver or other mouse

driver in your system, delete it from your system and use a text editor

to remove the old directory from the PATH statement in your AUTOEXEC.BAT

file.

************************

* CONFIG Modifications *

************************

If you have an older version of the Genius mouse driver - GMOUSE.SYS -

in your CONFIG.SYS file, the HiPoint Setup program will eliminate it

and save the original CONFIG.SYS file as CONFIG.BAK.

************************************

* Operation from Floppy Disk Drive *

************************************

If you don't have a hard disk, you can load the DOS HiPoint mouse driver

and DOS HiPoint Control Panel from the Floppy Disk Drive into the memory

and operate from there. A minimum of 256K is necessary.

*************************************

* Using COM3 or COM4 in your system *

*************************************

If you connect your mouse on COM3 or COM4, there may be some problems

while running under Windows. The Windows HiPoint driver doesn't fully

support these 2 communication ports, so for some PCs you are not able

to use HiPoint in Windows under enhanced mode. The DOS driver runs fine

through COM3 or COM4.

We therefore suggest connecting the mouse through COM1 or COM2 when using

HiPoint.

=========================================================================

(2). Modifying POINT.INI

The POINT.INI file contains the program information and the default

settings of the Genius HiPoint driver program. Any changes to it will affect

the operation of your mouse. It is recommended that only advanced users

make changes directly to this file. Before making any changes to the file,

always first save it as a backup file in case of mistakes, i.e. POINT.BAK.

*************************

* Acceleration Profiles *

*************************

The acceleration parameters determine the relative speed of cursor movement

for set distances of ball movement within a standard amount of time. On the

existing four acceleration profiles, speed is slowed for shorter distances

and increased for longer distances. Thus, when operating your HiPoint, you

get more precision in short distances, but are still able to move the cursor

across the whole acreen with a quick, short turn.

Acceleration curves work by establishing multipliers for different distances

covered in a single unit of time. The multiplier adjusts the speed set under

the Sensitivity setting. Thus the multiplier may be 0.5 for a short distance

to slow the movement speed by one-half, but for a long distance the

multiplier may be 2.0 thus doubling the movement speed. Here is a sample

profile.

[AccelerationProfile2]

Label=Cruising

Movement=1    26    42     61    80

Factor=1.00  1.25  1.50   1.75  1.50

You may make changes directly to the existing four acceleration curves. In

each set of figures the upper line contains the number of "mickeys", which

for input devices is unit of distance per unit of time. The number of mickeys

cannot exceed 127. Below each mickey designation is the multiplier. Be sure

to leave at least one space between each number. Each line must have less

than 80 characters. Do not use tabs, use the space key.

***************

* Orientation *

***************

If you wish to directly input the angle of operation for HiPoint, key

in a figure from 0 to 359 next to "RotationAngle=". 0 would be a natural setting for left-handed users.

=========================================================================

(3). Miscellaneous Items

******************************

* Loading POINT.INI settings *

******************************

The settings in the POINT.INI file are read by both the DOS and Windows

HiPoint drivers only when the drivers are loaded. Similarly, the Windows

HiPoint Control Panel reads the settings in POINT.INI only when it is

started. The DOS HiPoint utilities Orientation and Control Panel get

their initial settings from those resident in HiPoint-driver memory.

Saving changes permanently from any of the HiPoint utilities updates both

the settings in driver memory and the settings in the POINT.INI file.

However, if you edit the POINT.INI file manually, your new settings

will not be recognized by either mouse driver until it is reloaded.

To make your changes to POINT.INI settings take effect in DOS:

1. Save the edited POINT.INI file.

2. Restart your computer.

Or,

Remove the HiPoint driver from memory by typing POINT U;

then reload the HiPoint driver by typing POINT.

To make your changes to POINT.INI settings take effect in Windows:

1. Save the edited POINT.INI file.

2. Exit Windows.

3. Restart Windows.

=========================================================================