AST LAN ADAPTER RELEASE DISK Rel. 1.10

Copyright (c) AST Research, Inc. 1988-89

All Rights Reserved

Netware Directory

This directory and the directories below it contain configuration software

and run-time drivers for using your AST local area network adapter with a

NetWare environment.  Please consult your AST LAN Adapter Installation

Guide for instructions on configuring the adapter, and your AST-Advanced

NetWare or other NetWare documentation for instructions on installing

NetWare.

Before you can configure and install NetWare with your AST adapter, you

must first place the correct drivers on your NetWare diskettes.  If you are

using AST-Advanced NetWare, these drivers should already be on your NetWare

master diskettes.  If you purchased NetWare from another vendor or if the

driver you need is not on your NetWare masters, please follow the following

instructions.

This diskette contains drivers which are certified for use with NetWare

versions 2.0a and 2.12.  They will also run with any other version of

NetWare which is compatible with either of these two releases.  If you are

using NetWare 2.0X (including 2.0a, 2.0a++ and 2.01), proceed to Step 1.

If you are using NetWare 2.1X (including 2.1 and 2.12), proceed to Step 9.

STEPS 1-8 ARE FOR NETWARE 2.0X ONLY

===================================

1.   Make backup copies of your NetWare 2.0X  GENOS-1 and GENSH-1

diskettes, and put the originals safely back in their box.

2.   Insert your copy of GENOS-1 into a floppy disk drive and use the DIR

command to view the contents of the root  directory.

Typically, you will see something like this:

Volume in drive A is GENOS-1

Directory of  A:\

GENOS    BAT     1220   4-21-86  11:36a

IBM$RUN  OVL     2016   1-21-86   1:50p

$RUN     OVL     2016  11-16-85   2:35p

SYS$MSG  DAT    14225   5-02-86  10:36a

SYS$ERR  DAT     4508   4-23-86   3:58p

SELOS    EXE    65536   4-23-87  10:23a

NULL         <DIR>      1-02-87  12:58a

TRN          <DIR>      1-02-87  12:58a

SMC          <DIR>      1-02-87  12:58a

ASYNC        <DIR>      1-02-87  12:58a

GNET         <DIR>      1-02-87  12:58a

ETHERNET     <DIR>      1-02-87  12:58a

3.   Select a directory containing NetWare drivers for hardware you are not

using.  If you don't use G-NET LAN  adapters, for example, you might

select the GNET subdirectory.  Please delete the files in that

directory, followed by the directory itself.

4.   Create a new directory on GENOS-1 called ASTDRVR.

5.   Use the following table to determine which directory  on your AST

diskette contains NetWare 2.0X drivers for your particular adapter:

Adapter                         AST Diskette Directory

------------------------------------------------------

AST-Ethernet                   \NETWARE\ETHERNET\NW20X

AST-EtherNode                  \NETWARE\ETHRNODE\NW20X

AST-Resource Sharing Network   \NETWARE\RSN\NW20X

AST-Star System                \NETWARE\STAR\NW20X

6.   Copy the files in the GENOS1 subdirectory of your adapter driver

directory (for example, \NETWARE\ETHERNET\NW20X\GENOS1\*.*) into the

\ASTDRVR directory you just created on GENOS-1.  If there is

insufficient disk space on GENOS-1, repeat step 3 and delete

additional unused directories until you can successfully complete this

step.

7.   Repeat steps 2-6 but using your copy of the GENSH-1  diskette and

copying files from the GENSH1 subdirectory of your adapter driver

directory (for example, \NETWARE\ETHERNET\NW20X\GENSH1\*.*).  Be

careful to remove the same directory from GENSH-1 as you did from

GENOS-1.

8.   Using your new GENOS-1 and GENSH-1 diskettes, install NetWare 2.0X in

the usual manner.

STEPS 9-13 ARE FOR NETWARE 2.1X ONLY

====================================

9.   Make backup copies of the following NetWare 2.1X diskettes, and put

the originals safely back in their box:

SHGEN-1

LAN_DRVR_002

AUXGEN

Be sure that the copies are not write protected, since you will need

to write to each of them during the installation process.

10.  Use the following table to determine which directory on your AST

diskette contains NetWare 2.1X drivers for your particular adapter:

Adapter                         AST Diskette Directory

------------------------------------------------------

AST-Ethernet                   \NETWARE\ETHERNET\NW21X

AST-EtherNode                  \NETWARE\ETHRNODE\NW21X

AST-Resource Sharing Network   \NETWARE\RSN\NW21X

AST-Star System                \NETWARE\STAR\NW21X

11.  Copy *.LAN from your adapter driver directory (for example,

\NETWARE\ETHERNET\NW21X\*.LAN) to the root directories of your

SHGEN-1, AUXGEN and LAN_DRV_002 diskettes.

12.  Copy *.OBJ from your adapter driver directory (for example,

\NETWARE\ETHERNET\NW21X\*.OBJ) to the root directory of your

LAN_DRV_002 diskette.

13.  Using your new SHGEN-1, LAN_DRV_002 and AUXGEN diskettes, install

NetWare 2.1X in the usual manner.

#