Cabletron Systems, Inc.

FOR USE WITH MICROSOFT OS/2 LAN MANAGER

INSTALLATION GUIDE

for

DOS NDIS VERSION 01.00.00

These drivers are designed to be used with 3Com 3+Open, the

Microsoft OS/2 LAN Manager OEM distribution release 1.0 or later,

and other compatible OEM versions of LAN Manager.  The drivers

named NDISPD conform to the Network Driver Interface

Specification (NDIS) defined by Microsoft/3Com. This release

supports only the DOS workstation under 3Com 3+Open.

Files Included:

README.DOC               This File.

NDISPD.DOS               The DOS NDIS MAC driver

PROTOCOL.INI             A sample PROTOCOL.INI file

OVERVIEW

1) Install your Cabletron Network Adapter card.

Use diagnostics to verify correct installation (Optional).

2) Install 3Com 3+Open LAN MANAGER at this time if not previously

installed.

3) Install Cabletron files from Release disk.

4) Modify CONFIG.SYS to reference Cabletrons packet driver file

(CSIPD_E.EXE).

5) Modify CONFIG.SYS to reference Cabletrons NDIS MAC driver

(NDISPD.DOS).

6) Modify PROTOCOL.INI file to reflect use of Cabletrons adapter

card.

7) Modify PROTOCOL.INI file to reflect "BINDINGS".

8) Reboot system and follow LAN MANAGER vendor instructions.

Packet driver installation

INSTALLING ON DOS WORKSTATIONS

1) Install the Cabletron adapter card.  Proper installation can

be verified by running the diagnostic program under DOS.

Please refer to the DNI adapter card documentation and the

information in the DIAGS subdirectory.

2) Install 3Com 3+Open LAN MANAGER or continue to next step if

previously installed.  The adapter selected does not matter

because the references to it will be replaced once the

installation is complete. The examples below assume the

3Com EtherLink II has been selected with 3+Open.

Do not start LAN Manager.

3) Copy the DOS NDISPD.DOS driver from the Cabletron DNI Drivers

diskette to the driver directory used by LAN Manager.  For

example, installing a 3+Open DOS workstation using the normal

directory structure type:

COPY A:\LANMAN\NDISPD.DOS C:\3OPEN\DOSWKSTA\LANMAN\DRIVERS

Copy the packet driver software from the Cabletron DNI Drivers

diskette to the driver directory used by the LAN Manager.

For example,installing a 3+Open DOS workstation using the

normal directory structure type:

COPY A:\CSIPD\CSIPD_E.EXE C:\3OPEN\DOSWKSTA\LANMAN\DRIVERS

4) Modify the config.sys file to reference the Cabletron Packet

driver file. Use a text editor to add the following line

before the line which specifies the PROTMAN.DOS file.

Add the line:

DEVICE=\3OPEN\DOSWKSTA\LANMAN\DRIVERS\CSIPD_E.EXE

The LAN Manager installation will add several drivers to the

CONFIG.SYS file. Only the MAC driver referring to a particular

board should be replaced.

The line with PROTMAN.DOS should not be modified.

5) Modify the config.sys file to reference the Cabletron

adapter rather than the adapter selected during LAN Manager

installation.  For example, if the 3Com EtherLink II was

selected during a workstation installation, use a text editor

to change the line:

DEVICE=\3OPEN\DOSWKSTA\LANMAN\DRIVERS\ELNNKII.DOS

to the line:

DEVICE=\3OPEN\DOSWKSTA\LANMAN\DRIVERS\NDISPD.DOS

The LAN Manager installation will add several drivers to the

CONFIG.SYS file. Only the MAC driver referring to a particular

board should be replaced.  The line with PROTMAN.DOS should not

be modified.

6) Modify the PROTOCOL.INI file to include an entry for the

Cabletron driver.  The PROTOCOL.INI file contains sections

defining parameters for all the protocol drivers used by LAN

Manager.  To add a section for this driver, use a text editor

to add the following lines to the file:

[CSI8023]

drivername = CSINET$

The line containing "[CSI8023]" defines the name for the

driver and will be used in the bindings command described below.

The drivername "CSINET$" is used by the protocol manager.

Other lines can be defined for the driver and can be modified

appropriately depending on how the adapter and PC are configured.

The values shown correspond to the current factory defaults for

the supported adapters.  A more complete explanation for each of

these parameters and its appropriate values is given at the end

of this document.

7) Modify the PROTOCOL.INI file to change the bindings of the

protocol stack to reference this driver in place of the driver

selected during installation.  Use a text editor to modify the

"bindings =" line to reference the "CSI8023" name defined in

the section added in step 3 rather than the name set during

installation.

For example: under 3+Open with the DLC/NETBEUI protocol stack and

3Com EtherLink II selected during installation, in the

PROTOCOL.INI file section labeled "[DLC]", the line:

BINDINGS = ETHERLINKII

must be changed to:

BINDINGS = CSI8023

8) The installation is complete.  Reboot and bring up LAN

Manager using the instructions supplied by the LAN Manager

vendor.

PROTOCOL.INI PARAMETERS

Several parameters are typically specified in the CSI8023 section

of the PROTOCOL.INI file.  The only parameter that must be

specified is the "drivername".  All the others have default

values supplied by the driver and can be left out of the

PROTOCOL.INI file if the installed adapter matches the default

configuration.