September 1992

LANsmart Installation

for

D-Link Ethernet Pocket LAN Adapter

==================================

This installation procedure applies to the following D-Link products:

DE-620,DE-620CT,DE-620TP,DE-620AUI,

Files        Contents

-----------  -------------------------------------------------

DE-620.DRV   Driver program for the above-mentioned adapters

DE-620.EXE   Driver program for the above-mentioned adapters

1. RUN LANsmart v.1.26 and v.1.30

==================================

Follow the procedure below to use the new LANsmart driver program on this

diskette with LANsmart v.1.26 and v.1.30.

1) Replace the LANsmart program LANMA05.DRV with the new driver program.

Example:

> COPY A:\LANSMART\DE-620.DRV  C:\LANSMART\LANSMA05.DRV

2) Now, you can use both the menu mode and the command mode to log onto

the network.  The LOGON command syntax is as follows:

> LOGON <username> <SRV|WKS> /NET:5 [other parameters]

2. RUN LANsmart version 2.xx & v3.xx

=====================================

You may use both the menu and the command mode to logon to the network.

The LOGON command syntax is as follows:

> LOGON <username> <SRV|WKS> /NET:DE-620 [other parameters]

The driver program DE-620.DRV should first be copied to the directory

that contains LOGON.EXE program. /NET:DE-620 specifies to LANsmart your

network adapter card.