December 1991

D-Link Ethernet Pocket LAN Adapter

3COM 3+ Share Driver Installation

==================================

The D-Link 3COM 3+ Share Driver Program lets you run 3Com 3+ Share

version 1.5 on your PC with the D-Link Ethernet Pocket LAN

Adapter.

The following files will be used in this installation:

Files	       Size (Bytes)   Date     Time    Contents

-----------    ------------   -------  ------  -------------------------

DE600PD.SYS    7721	     12-06-91	       DE-600 Driver Program

ETH.SYS        9094	     12-03-91	       3Com 3+ Share Driver

1. Follow the installation procedure for 3Com EtherLink board.

2. Copy the program ETH.SYS from the Installation Diskette for

the D-Link Ethernet Pocket LAN Adapter to the boot disk/diskette.

This program will replace the program of the same file name

(ETH.SYS) when you perform the installation procedure in step 1

above.

3. In the CONFIG.SYS file on the boot disk/diskette, make sure

you have the following two statements.	The order of these two

statement must be as shown below.

DEVICE = DE600PD.SYS 99

DEVICE = ETH.SYS

99 is the interrupt vector. The interrupt vector

can be from 96 to 128.

You may now boot and start your 3COM workstation.