December 1991

D-Link NDIS Driver Program For DOS

==================================

The NDIS driver program DE600.DOS is for use with the following

D-Link products:

DE-600          DE-600-E

DE-600AUI       DE-600AUI-E

DE-600TP        DE-600TP-E

The D-Link NDIS driver programs for DOS are developed following

Microsoft's NDIS specification version 2.0.1. This installation

procedure includes following network operating systems.

MS LAN Manager v2.0

DECnet PathWork v4.0

WIN/TCP PathWay Access for DOS v1.1

3COM 3+ Open v1.1

Banyan VINES v4.0

Installation Guide

==================

1. How to install DE-600.DOS on MS LAN Manager.

Microsoft LAN Manager provides a SETUP.EXE program on the

LAN Manager Setup disk for users to install LAN Manager.

To install the NDIS device driver program on a DOS workstation,

refer to "LAN Manager Installation Guide" for details.

A. Suppose you install MS LAN Manager on a DOS

workstation for the first time.

aa. Prepare a diskette with volume name " DOS_DRV_DLK " .

The diskette's directory structure is as follows:

MALANMAN.DOS  (root)

|

DRIVERS

________|_________

|                  |

ETHERNET            NIF

|

-------------

|           |

DE600

bb. Copy the following files to the following directories.

DE600.DOS      to      \MSLANMAN.DOS\DRIVERS\ETHERNET\DE600

PROTOCOL.INI   to      \MSLANMAN.DOS\DRIVERS\ETHERNET\DE600

DE600.NIF      to      \MSLANMAN.DOS\DRIVERS\NIF

cc. Execute the SETUP.EXE program that is provided by LAN Manager.

Upon completion of retrieval of the last workstation disk's

contents, you will be asked if you want to import additional

network drivers.  Answer "YES".

dd. Insert the prepared diskette into the floppy drive.

ee. Follow the instructions on the screen.  SETUP.EXE

will retrieve all NDIS driver programs from the diskette

and put them on the hardisk and directory where

LAN Manager resides.

ff. After completion of the setup procedure, your CONFIG.SYS file

will contain statements for the D-Link NDIS driver programs.

Reboot your computer and follow NDIS binding procedure to use

the D-Link NDIS driver programs.

B. If you have installed LAN Manager in your hardisk.

aa. Prepare a diskette with volume name " DOS_DRV_DLK " .

The diskette's directory structure is as follows:

MALANMAN.DOS  (root)

|

DRIVERS

_________|_________

|                  |

ETHERNET            NIF

|

-------------

|           |

DE600

bb. Copy following files to the following directories:

DE600.DOS       to      \MSLANMAN.DOS\DRIVERS\ETHERNET\DE600

PROTOCOL.INI    to      \MSLANMAN.DOS\DRIVERS\ETHERNET\DE600

DE600.NIF       to      \MSLANMAN.DOS\DRIVERS\NIF

cc. Execute the SETUP.EXE program.

dd. On the screen, select 'Action' function and import additional

network device drivers.

ee. Insert the D-Link NDIS driver program disk into the floppy drive.

ff. Follow the instructions on the screen.  SETUP.EXE

will retrieve all NDIS driver programs from the D-Link NDIS

driver disk and put them on the hardisk and directory where

LAN Manager resides.

gg. After completion of the setup procedure, your CONFIG.SYS file

will contain statements for the D-Link NDIS driver programs.

Reboot your computer and follow NDIS binding procedure to use

the D-Link NDIS driver programs.

2. How to install DE600.DOS on WIN/TCP PathWay Access for DOS

a. Copy DE600.DOS to directory C:\LANMAN.

b. In the directory C:\LANMAN, use a text editor to create a

PROTOCOL.INI file. The file content is as follows:

[DE600]

printport = 1

interrupt = 7

drivername = DE600$

c. In CONFIG.SYS file, add the following statement:

device=c:\lanman\de600.dos

d. In AUTOEXEC.BAT batch file, add the following statement:

ndis -d:1

winbind

wintcp

3. How to install DE600.DOS on DECnet PathWork v4.0

You can use the DECnet PathWork provided NETSET.EXE to install

D-Link NDIS driver program. Please refer to DECnet installation

manual for detail information.

Here, we suppose that you have installed the DECnet PathWork

with 3Com's Etherlink II card.

a. In CONFIG.SYS file, change statement

device = c:\decnet\ELNKII.SYS

to

device = c:\decnet\DE600.DOS

b. Modify PROTOCOL.INI file

from:

---------------------------------

[ELNKMACII]

drivername = ELNKII$

ioaddress = 0x300

interrupt = 3

xmitbufs = 2

[IPX4MAC]

:

bindings = ELNKMACII

:

[DATALINK]

:

bindings = ELNKMACII

:

to:

-----------------------------------

[DE600_NIF]

drivername = DE600$

printport = 1

interrupt = 7

[IPX4MAC]

:

bindings = DE600_NIF

:

[DATALINK]

:

bindings = DE600_NIF

:

4. How to install DE600.DOS on 3Com 3+ Open

Suppose you have installed the 3Com 3+ Open with 3Com's 3C501

card.

a. Copy DE600.DOS to directory C:\3OPEN\DOSWKSTA\LANMAN\DRIVERS.

b. Modify the device driver statement for 3C501 card in the

CONFIG.SYS file to

device = c:\3open\doswksta\lanmam\driver\DE600.DOS

c. Modify PROTOCOL.INI file to

[ETHERLINK]

:

drivername = DE600$

printport = 1

interrupt = 7

:

5. How to install DE600.DOS on Banyan VINES v4.0

a. Create a DOS 3.1 or above bootable diskette.

Put this boot diskette in your floppy drive.

b. Login to your VINES network.

c. Run PCCOPY.

Select 'NDIS Ethernet" from PCCOPY menu. PCCOPY will copy

the NDIS-specific files to the bootable diskette.

d. Copy DE600.DOS files from D-Link driver diskette to

this bootable diskette.

e. Create a CONFIG.SYS file to include the following statement.

device = protman.dos

device = de600.dos

f. Create a PROTOCOL.INI file as follows:

[PROTOCOL MANAGER]

drivername = PROTMAN$

[VINES_XIF]

drivername = NDISBAN$

bindings = "DE600_NIF"

[DE600_NIF]

printport = 1

interrupt = 7

drivername = DE600$

g. Run PCCONFIG then select Network Card Setting.

h. Choose 'NDIS Ethernet' from the menu. Then enter the correct

hardware settings.

i. Save the new value and exit from PCCONFIG.

j. Reboot your computer.

================================================================================

D-Link NDIS PROTOCOL.INI User Guide

This guide specifies the PROTOCOL.INI keywords used for the

D-Link NDIS device driver.

IOADDRESS - This keyword specifies the I/O base address used by

the D-Link NIC. This value must match the base I/O

address jumper setting on your NIC.

INTERRUPT - This keyword specifies the interrupt used by the

D-Link NIC. This value must match the IRQ jumper

setting on your NIC.

DRIVERNAME - This keyword specifies the NIC driver's name.

PRINTPORT - This keyword specifies the printer port used by

the D-Link Pocket LAN Adapter.

BASEMEMORY - This keyword specifies the expanded memory address

used by the D-Link NIC. This value must match the

base memory jumper setting on your NIC.

ADAPTERID -  This keyword specifies the adapter ID used by D-Link

MCA bus card. This value must match the card ID jumper

setting on your MCA adapter.

NETADDRESS - This keyword specifies the node address (hex) used by

your network interface card.  However, you can specify

a node address different from the one burned in your NIC,

in which case, the specified address will be used

instead of the NIC's actual node address.

Following is an example PROTOCOL.INI for D-Link Ethernet product:

[DE600]

Drivername = DE600$

Printport = 1

Interrupt = 7