December 1991

D-Link Ethernet Pocket LAN Adapter

PC-NFS Driver Installation

==================================

The D-Link PC-NFS Driver Program lets you run Sun MicroSystems PC-NFS

on your PC with the D-Link Ethernet Pocket LAN Adapter.

The following files will be used in this installation:

Files	       Size (Bytes)  Date      Time   Contents

-----------    ------------  --------  -----  -------------------------

PDNFS.SYS      4160	     12-06-91         PC-NFS Driver Program

DE600PD.SYS    7721	     12-06-91         DE-600 Driver Program

DE600PD.COM    7858	     12-03-91         DE-600 Driver Program

Two methods can be used to install:

METHOD 1:

1. Install the PC-NFS program according to the procedure

described in the PC-NFS manual.  On the screen of the PC-NFS

install program, select 3C501 (the 3Com EtherLink card) as your

network interface card.  When the screens prompt you, select 3 for the

interrupt level (IRQ), and select 300h for the base I/O address.

2. Copy the following files to the \NFS directory of your hard

disk:

PDNFS.SYS

DE600PD.SYS

DE600PD.COM

4. Modify the CONFIG.SYS file as follows:

DEVICE=C:\ANSI.SYS

DEVICE=C:\NFS\PCNFS.SYS

DEVICE=C:\NFS\SOCKDRV.SYS

DEVICE=C:\NFS\DE600PD.SYS 98

DEVICE=C:\NFS\PDNFS.SYS

LASTDRIVE=V

AUTOEXEC.BAT file:

SET PATH = C:\; C:\NFS

SET TZ=EST5EDT

NET YPDOMAIN ypserver

NET START RDR

NET YPSET homer

NET NAME john

NET USER F: \\user\john

We use the name "john" here as an example.

98 is the interrupt vector. The interrupt vector

can be from 96 to 128.

5. Reboot the PC to start PC-NFS.

You may now execute the PC-NFS commands to communicate

other computers through the TCP/IP protocol.

METHOD 2:

1. Install the PC-NFS program according to the procedure

described in the PC-NFS manual.  On the screen of the PC-NFS

install program, select 3C501 (the 3Com EtherLink card) as your

network interface card.  When the screens prompt you, select 3 for the

interrupt level (IRQ), and select 300h for the base I/O address.

2. Copy the following files to the \NFS directory of your hard

disk:

PDNFS.SYS

DE600PD.SYS

DE600PD.COM

4. Modify the CONFIG.SYS file as follows:

DEVICE=C:\ANSI.SYS

DEVICE=C:\NFS\SOCKDRV.SYS

DEVICE=C:\NFS\PCNFS.SYS

DEVICE=C:\NFS\PDNFS.SYS

LASTDRIVE=V

AUTOEXEC.BAT file:

SET PATH = C:\; C:\NFS

SET TZ=EST5EDT

NET YPDOMAIN ypserver

NET START RDR

NET YPSET homer

NET NAME john

NET USER F: \\user\john

DE600PD 98

We use the name "john" here as an example.

98 is the interrupt vector. The interrupt vector

can be from 96 to 128.

5. Reboot the PC to start PC-NFS.

You may now execute the PC-NFS commands to communicate

other computers through the TCP/IP protocol.