July 1993

LANsmart Installation

for

D-Link DE-621 Duplex and DE-620 Ethernet Pocket LAN Adapter

===========================================================

Thank you for purchasing the D-Link Duplex for desktop, laptop

and notebook PCs to retain DE-620 and printer or key card connection.

Before you installing driver program, please refer to DE-620 or DE-621

manual to get more detail installation. A simple installation is as the

following figure.

����

/	/�

/    /  �

/    /	*�<------- connect to DE-620 only

����Ŀ  *  �

connect     |\ 	      �    �*	*�<------- connect to printer or

to pc	     |.|	  �����    �  *  /	   key card

printer <-- |.|���������ͳ DE-621 �*  /

port	     |.|	  �	   � /

|/ 	  ����������

This installation procedure applies to the following D-Link products:

DE-620,DE-620CT,DE-620TP,DE-620AUI,

Files	     Contents

-----------  -------------------------------------------------

DE-620.DRV   Driver program for the above-mentioned adapters

DE-620.EXE   Driver program for the above-mentioned adapters

1. RUN LANsmart v.1.26 and v.1.30

==================================

Follow the procedure below to use the new LANsmart driver program on this

diskette with LANsmart v.1.26 and v.1.30.

1) Replace the LANsmart program LANMA05.DRV with the new driver program.

Example:

> COPY A:\LANSMART\DE-620.DRV  C:\LANSMART\LANSMA05.DRV

2) Now, you can use both the menu mode and the command mode to log onto

the network.  The LOGON command syntax is as follows:

> LOGON <username> <SRV|WKS> /NET:5 [other parameters]

2. RUN LANsmart version 2.xx & v3.xx

=====================================

You may use both the menu and the command mode to logon to the network.

The LOGON command syntax is as follows:

> LOGON <username> <SRV|WKS> /NET:DE-620 [other parameters]

The driver program DE-620.DRV should first be copied to the directory

that contains LOGON.EXE program. /NET:DE-620 specifies to LANsmart your

network adapter card.