July 1993

Ethernet Card Setup/Diagnostic program

for

DE-621 Duplex and DE-620 Ethernet Pocket LAN Adapter

====================================================

Thank you for purchasing the D-Link Duplex for desktop, laptop

and notebook PCs to retain DE-620 and printer or key card connection.

Before you installing driver program, please refer to DE-620 or DE-621

manual to get more detail installation. A simple installation is as the

following figure.

����

/	/�

/    /  �

/    /	*�<------- connect to DE-620 only

����Ŀ  *  �

connect     |\ 	      �    �*	*�<------- connect to printer or

to pc	     |.|	  �����    �  *  /	   key card

printer <-- |.|���������ͳ DE-621 �*  /

port	     |.|	  �	   � /

|/ 	  ����������

The SETUP.EXE program (Setup/Diagnostic) lets you configure the DE-620 hardware

settings, and test the state and cable connection of the card.

To run the program, insert the DE-620 driver program diskette in drive 'A' and

do the following:

> cd \UTILITY	     --change directory to UTILITY

> SETUP 	     --run the SETUP.EXE program

or

> cd \UTILITY	     --change directory to UTILITY

> SETUP /F	     --run the SETUP.EXE with parameter /F

if the data in your EEPROM has been corrupted

or you cannot configure your card through normal

Please run "SETUP /F".

Menu Options

============

1.  SETUP lets you configure the hardware settings of DE-620.

You have the option to view/change :

<1> Node ID

<2> Printer Port IRQ

<3> H/W Model

<4> Media Type

<5> Printer Mode

<6> RAM Size

<7> Write Delay Factor

<8> Read Delay Factor

<9> Interrupt Trigger

<10> Auto Detecting Setting

2.  DIAGNOSTIC lets you diagnose the state and cable connections of DE-620.

3.  DIAGNOSTIC lets you diagnose if DE-621 Duplex work with DE-620 is fine.