DEC  1994

3Com 3+ Share Driver Installation

for

D-Link DE-650 PCMCIA Ethernet Adapter

=====================================

*********************************** NOTE ***********************************

Each DE-650 series adapter comes with a driver diskette of its own.  If you

have more then one model of DE-650 adapter, make sure that you use the driver

diskette that was included with the DE-650 adapter you are using.

****************************************************************************

The D-Link 3COM 3+ Share Driver Program lets you run 3Com 3+ Share on your PC

with the D-Link DE-650 PCMCIA Ethernet Adapter.

The following files will be used in this installation:

Files		 Contents

------------ 	 ------------------------------------------------------------

DE65NPD.SYS      DE-650 Driver Program.

ETH.SYS 	 3Com 3+ Share Driver.

1. Follow the installation procedure for 3Com EtherLink board.

2. Copy the program ETH.SYS from the Installation Diskette for the D-Link

DE-650 PCMCIA Ethernet Adapter to the boot disk/diskette.

This program will replace the program of the same file name (ETH.SYS) from

the installation procedure in step 1 above.

3. In the CONFIG.SYS file on the boot disk/diskette, make sure you have the

following statements.  The order of these statements must be as shown below.

*** PC installed Socket and Card services ***

DEVICE = DE65NPD.SYS [-d] [-w] <sw_int> <irq> <io_addr>

DEVICE = ETH.SYS

OR

*** PC installed Socket service or Databook Cardtalk***

*** PC's PCMCIA controller is Intel 82365SL ***

*** PC is Toshiba T3300SL ***

DEVICE = DCD16.EXE

DEVICE = DE65NPD.SYS [-d] [-w] <sw_int> <irq> <io_addr>

DEVICE = ETH.SYS

Please refer to \IBMPC\INSTALL\README.DOC for more detail information.

You may now boot and start your 3COM workstation.

Notes:

1. Parameters for DE65NPD.SYS

[-d]		Delay Initial - The driver program will not initialize the

network interface card until the packet driver program is

accessed by its upper layer program first time.

[-w]		The -w parameter is for use with the Windows environment only.

<sw_int>	Is the software interrupt vector number used by the packet

driver program, which can be from 96 to 128.

<irq>		Is the hardware interrupt (IRQ) line set on your network

interface card. The default is 5 if this parameter is not

specified.

<io_addr>	Is base I/O address set on your network interface card.

The default address of 0x300 is used, if this parameter is not

specified.

2. Parameters for INSTALL utility:

DCD16.EXE

DCD16.EXE is an integration version of open adapter utility.

DCD16.EXE supports Intel 82365SL, Toshiba T3300SL's ICCNT, and Socket

Services v1.01, v2.0, v2.01, v2.10 Card Services v2.0, v2.01, v2.10.

DEVICE = DCD16.EXE [/iop:P0] [/irq:P1] [/slt:P2]

Parameter description:

/iop   :I/O base

/irq   :IRQ value

/slt   :Slot number

P0 :The selectable window I/O base with 0x20 range.

The default value is 0x300.

P1 :The selectable irq value.

The default value is 0x5.

P2 :The selectable slot number.

The default value is 0. You must set 0 when you use slot 1.

Set slt:1 when slot 2 is used,etc.