DEC  1994

AT&T UNIX Driver Installation

for

DE-650 PCMCIA Ethernet Card

===============================

*********************************** NOTE ***********************************

Each DE-650 series adapter comes with a driver diskette of its own.  If you

have more then one model of DE-650 adapter, make sure that you use the driver

diskette that was included with the DE-650 adapter you are using.

****************************************************************************

The following files will be used to in this installation:

Files         Contents

---------     -------------------------------------------

DE65N.O       DE-650 Unix driver object program.

DLK           D-Link Unix adapter setup script

STRCF         For Unix streams

Installation and configuration consist of the following steps:

1.  Login as the root user.

2.  Installing TCP/IP software first.

3.1 If the install diskette is UNIX tar format.

Copying D-Link driver through the tar utility.

#tar xvnof /dev/fd048ds9

./usr/lib/tcprt/dlk/de65n.o

./usr/lib/tcprt/dlk/strcf

./usr/lib/tcprt/dlk/dlk

./usr/lib/tcprt/dlk/readme.doc

3.2 If the install diskette is DOS format.

Copying D-Link driver through the doscp utility.

#mkdir /usr/lib/tcprt/dlk

#doscp a:/ibmpc/at\&tunix/de65n.o  /usr/lib/tcprt/dlk/de65n.o

#doscp a:/ibmpc/at\&tunix/dlk  /usr/lib/tcprt/dlk/dlk

#doscp a:/ibmpc/at\&tunix/strcf  /usr/lib/tcprt/dlk/strcf

#doscp a:/ibmpc/at\&tunix/readme  /usr/lib/tcprt/dlk/readme.doc

#chmod +x /usr/lib/tcprt/dlk/dlk

4.  Installing D-Link AT&T UNIX driver.

#cd /usr/lib/tcprt/dlk

#./dlk

Do you wish to install or delete the dlk driver?  Type i to install

A display of information about driver.		  Type y to continue

Select interrupt vector number [5]:		  Type value

Select I/O base address [300]:	        	  Type hexadecimal value

Select memory base address [d0000]:		  Type hexadecimal value

Select PCMCIA slot number [0]:			  Type value

Do you wish to relink the kernel now?		  Type y to relink

5.  Reboot UNIX.

#cd /

#shutdown -y -g0 -i6