Dec. 1994

Windows NT v3.5 Driver Installation

for

D-Link DE-650 PCMCIA Ethernet Adapter

=======================================

*********************************** NOTE ***********************************

Each DE-650 series adapter comes with a driver diskette of its own.  If you

have more then one model of DE-650 adapter, make sure that you use the driver

diskette that was included with the DE-650 adapter you are using.

****************************************************************************

The DE-650 Ethernet LAN drivers of DE-650 cards for Windows NT contained in this

diskette are as follows:

Files              Contents

--------------     ----------------------------------------------------

DE65N.SYS          NDIS 3.0 driver for Windows NT.

DE65N.DLL          Installation manual screen for Windows NT.

OEMSETNT.INF       OEMSETUP file for Windows NT.

\DISK1

******************************************

*  DE-650 Driver in PCMCIA Environments  *

******************************************

DE-650 NT driver and DE-650 WFW 3.11 driver comply to NDIS 3.0 specs. and are

supported only for Intel 82365SL PCMCIA adapter and there is no need to run

DCD16.EXE.

**********************************************

*  DE-650 Driver Setting in Windows NT V3.5  *

**********************************************

Because DE-650 is a PCMCIA driver and PCMCIA needs the extra settings of slot

number and memory base, these settings are needed to be chosen. That is, users

have to choose the settings of IO Base, Interrupt Number, Mem Base, and PCMCIA

Slot Number.

**********************************

*  Windows NT V3.5 Installation  *

**********************************

Installation Procedure:

(1)  When LAN drivers is installed, Windows NT will present a list of all

available drivers.  Since there will be no D-Link DE-650 drivers initially,

select OTHER for 3rd party adapters not in the driver list.

(2)  Windows NT will ask for the drive/path containing the DE-650 Windows NT

drivers.  Insert the D-Link driver diskette in drive A: and respond with

the following.

A:\IBMPC\NT35

(3)  Windows NT will attempt to locate an OEMSETNT.INF file in the specified

path. If you have entered the path name correctly, Windows NT should copy

the appropriate drivers to the Windows NT system.

(4)  At this point, the driver would have been properly copied and the

installation would proceed to the next step.  Please refer to your Windows

NT installation guide for correct procedures.

(5)  After the initial installation, the DE-650 drivers will be permanently

copied to the Windows NT system.  Subsequent network driver lists will

already contain D-Link DE-650 driver.