February 1993

NetWare Driver Installation

===========================

The NetWare driver programs on this diskette are for use with the

D-Link DE-650 PCMCIA Ethernet Card.

This diskette is labeled as LAN_DRV_DLK and the files are organized as follows:

Directory	      Contents

---------	      --------------------------------------------------------

\NETWARE	      A pre-generated IPXDE650.COM

\NETWARE\V215B	      NetWare v2.15b workstation drivers

\NETWARE\V215C	      NetWare v2.15c, v2.2 or NetWare 386 workstation drivers

\NETWARE\ODI.DOS      ODI workstation driver for DOS

\NETWARE\NET386       Server driver for Netware

*********************************

*  QUICK SETUP FOR WORKSTATION	*

*********************************

A pre-generated workstation driver IPXDE650.COM is included on

this diskette.	You can simply run the IPXDE650 on a workstation

to access the Novell network:

>IPXDE650	  --Supports NetWare Lite, 2.1x, 2.2 or NetWare 386.

>NETx	  --This program is provided by Novell.

***********************

*  WORKSTATION SETUP  *

***********************

If you want to generate your own workstation driver instead of using the

provided IPXDE650.COM, please do the following:

1)  If you make a backup of this diskette, make sure that it is labeled as

LAN_DRV_DLK.  You can use the DOS LABEL command as follows:

>LABEL A:LAN_DRV_DLK

2)  Copy the appropriate NetWare driver files to the root directory of

this diskette (or backup):

>Copy A:\NETWARE\V215c\*.*  A:\   --For NetWare 386 workstation

3)  Execute Novell's SHGEN or WSGEN program, which resides on the SHGEN-1 or

WSGEN diskette.  When you are asked to select a network driver, press

the INS key.  You will be asked to insert a LAN_DRV_??? diskette.  Insert

this diskette (or backup) in drive A.  Upon completion, a new IPX.COM file

will be created in your SHGEN-1 or WSGEN diskette.

The default values for all parameters of IPXDE650.COM are as follows:

I/O port #1	 300

IRQ #1		 5

Memory #1	 d000

Note : if you generate IPX.COM by "1:Driver Configurable by JUMPERS

Utility", the I/O PORT # will be None, you must execute the JUMPERS

Utility offered by NetWare, the procedure is as follows:

a. >JUMPERS

b. Enter the file IPX.COM to modify.

c. The screen displays the following:

DE-650 Options

I/O PORT #1     None

d. Press Enter

(window assistance)

d1.	 I/O PORT#

=========

200

220

240

260

280

300

.

.

.

380

d2. Select one port#.

d3. Save the port# that you selected.

d4.	IRQ #1		 None

IRQ  #

======

2

3

4

.

.

15

d5. Select one IRQ#

d6. Save the port# that you selected

d7. Memory #1	None

Memory #

========

C800

.

.

.

d8. Select Memery #

d9. Save the Memory # that you selected

************************

*  NETWARE LITE USERS  *

************************

1)  Follow the normal installation procedure as instructed by NetWare Lite.

When asked to select a network driver, simply choose "NOVELL NE2000".

2)  After the installation is completed, modify the STARTNET.BAT file as

follows:

From:

---------------------------

LSL

NE2000

IPXODI A

CLIENT

To:

---------------- or -------------- or ----------------  or --------------

AE 	      PCIC	       ICCNT		   PCMDLINK *

LSL	      LSL	       LSL		   LSL

DE650ODI	      DE650ODI	       DE650ODI 	   DE650ODI

IPXODI A	      IPXODI A	       IPXODI A 	   IPXODI A

CLIENT	      CLIENT	       CLIENT		   CLIENT

AE.EXE : is used to enable the PCMCIA card on which the Socket Service has

been installed.

PCIC.EXE : is used to enable the Intel 82365SL PC Card Interface Controller

(PCIC) chipset.

ICCNET.EXE : is used to enable the ICCNT (I/O Card Controller) chipset in

TOSHIBA T3300SL Model Notebook.

PCMDLINK.COM : is used to enable Phoenix PCMCCU and to allocate resources

that I/O base: 320, IRQ: 5, and MEM base: D000

Only PCMCCU v1.3 need PCMDLINK.EXE to enable PCMCCU.

For more detail information, please refer to \open_adp\readme.doc or

\install.doc.

3) Copy \NETWARE\ODI.DOS\NET.CFG file to NWLITE sub-directory which was

created when you installed NetWare Lite.

4) You may now run STARTNET to access the NetWare network.

**********************

*  ODI DRIVER SETUP  *

**********************

ODI Workstation for DOS

=======================

D-Link ODI Driver program allows you to run multiple protocl stacks.  It also

allows you to install up to two network adapters in a workstation. D-Link ODI

driver supports "port, node address, protocol and frame" options.  You can use

these options to create your own NET.CFG file.	For detailed info on Keywords,

please refer to Novell's manual "NetWare ODI shell for DOS - NET.CFG options".

1)  Copy the DE650ODI.COM file (from this diskette) to the directory which

contains Novell's LSL.COM, IPXODI.COM and NETx.COM files.

2)  Follow Novell's instructions on using the ODI programs. You can substitute

the DE650ODI.COM whenever a reference is made to the Hardware Specific

Module.

3)  A typical startup for ODI workstation is as follows:

>AE		--Enable PCMCIA Adapter

>LSL		--Link Support Layer Module provided by Novell

>DE650ODI		--Hardware Specific Module provided by D-Link

>IPXODI		--IPX Protocol Stack Module provided by Novell

>NETx		--DOS Support Module provided by Novell

>LOGIN

or

>PCMDLINK		--Enable Phoenix PCMCCU

>LSL		--Link Support Layer Module provided by Novell

>DE650ODI		--Hardware Specific Module provided by D-Link

>IPXODI		--IPX Protocol Stack Module provided by Novell

>NETx		--DOS Support Module provided by Novell

>LOGIN

or

>PCIC		--Enable PCMCIA Adapter

>LSL		--Link Support Layer Module provided by Novell

>DE650ODI		--Hardware Specific Module provided by D-Link

>IPXODI		--IPX Protocol Stack Module provided by Novell

>NETx		--DOS Support Module provided by Novell

>LOGIN

or

>ICCNT		--Enable PCMCIA Adapter

>LSL		--Link Support Layer Module provided by Novell

>DE650ODI		--Hardware Specific Module provided by D-Link

>IPXODI		--IPX Protocol Stack Module provided by Novell

>NETx		--DOS Support Module provided by Novell

>LOGIN

AE.EXE : is used to enable the PCMCIA card on which Socket Service has

been installed.

PCIC.EXE : is used to enable the Intel 82365SL PC Card Interface Controller

(PCIC) chipset.

ICCNET.EXE : is used to enable the ICCNT (I/O Card Controller) chipset in

TOSHIBA T3300SL Model Notebook.

PCMDLINK.COM

PCMDLINK.COM is PCM client driver and only Phoenix PCMCCU v1.3 need.

For more detail information, please refer to \open_adp\readme.doc or

\install.doc.

Parameter of AE.EXE, PCIC.EXE, ICCNT.EXE, PCMDLINK.COM

Example:

>AE	 [/iop:P0] [/mem:P1] [/irq:P2] [/slt:P3]

>PCIC [/iop:P0] [/mem:P1] [/irq:P2] [/slt:P4] [/grp:P4]

>ICCNT [/iop:P0] [/mem:P1] [/irq:P2]

>PCMDLINK (resources fixed on irq:5, mem:d600, io base:320)

Parameter description:

/iop   :I/O base

/mem   :Attribute memory base

/irq   :IRQ value

/slt   :Slot type

/grp   :Group value

P0 :The selectable window I/O base with 0x20 range.

The default value is 0x300.

P1 :The selectable window memory base.

The default value is 0xd000.

P2 :The selectable irq value.

The default value is 0x5.

P3 :The selectable slot number.

The default number is 1.

P4 :The selectable grp value.

The default value is 0.

Group 0 means Index IO is 3E0h, Data IO is 3E1h, Base is 00h

Group 1 means Index IO is 3E0h, Data IO is 3E1h, Base is 80h

Group 2 means Index IO is 3E2h, Data IO is 3E3h, Base is 00h

Group 3 means Index IO is 3E2h, Data IO is 3E3h, Base is 80h

*****************************

*  SERVER AND BRIDGE SETUP  *

*****************************

Example :  NetWare 386 Server/Bridge Setup

===========================================

1)  Copy the appropriate NetWare driver files (from this diskette) to the

root directory of your Novell diskette labeled SYSTEM-1.

>Copy  A:\NETWARE\NET386\*.*  B:\	--SYSTEM-1 disk must be in drive B

2)  Execute Novell's SERVER program, which resides in the SYSTEM-1 diskette.

After loading the hard disk driver, you can load the network driver as

follows:

:Load  A:\386DE650.LAN	--Select your network card settings

:Bind ipx to 386DE650

:load install		--Follow the NetWare installation instructions