July 1993

D-Link PCMCIA Ethernet Card

==============================

\OPEN_ADP\PCIC.EXE

PCIC.EXE is used to enable Intel 82365SL PC Card Interface Controller

(PCIC) chipset for DOS environment.

\OPEN_ADP\ICCNT.EXE

ICCNET.EXE is used to enable ICCNT (I/O Card Controller) chipset in

TOSHIBA T3300SL Model Notebook.

\OPEN_ADP\AE.EXE

AE.EXE is used to enable PCMCIA Ethernet Card on which Socket Service

or DATABOOK Socket and CardTalk has been installed.

\OPEN_ADP\PCMDLINK.COM

PCMDLINK.COM is PCM client driver for Phoenix PCMCCU v1.3.

\OPEN_ADP\DSCD.EXE

DSCD.EXE is a integration beta version of apen adapter utility.

DSCD.EXE supports Intel 82365SL, Toshiba T3300SL's ICCNT, and Socket

service v1.01, v2.0, v2.01.

For more detail information, please refer to \INSTALL.DOC.

**********************

SOCKET SERVICE: AE.EXE

**********************

The AE.EXE program is used to open I/O window mapping, memory window

mapping, and IRQ of PCMCIA Ethernet Card on which socket service or

DATABOOK Socket and Cardtalk has been installed.

To run DE-650's driver programs, AE.EXE must first be executed.

The parameters of the DE-650 driver programs must match the hardware

parameters settings of AE.EXE.  These parameters can be I/O base, attribute

memory base, and IRQ number.

Note:

IRQ parameter uses 5 or other except IRQ 9 because DATABOOK doesn't support

IRQ 9.

Two methods can be used to execute AE.EXE:

Method 1

--------

From DOS prompt,

A>ae [/iop:P0] [/mem:P1] [/irq:P2] [/slt:P3]

Method 2

--------

In DOS config.sys

device=ae.exe [/iop:P0] [/mem:P1] [/irq:P2] [/slt:P3]

Parameters are options.  Their descriptions:

/iop   :I/O base

/mem   :Attribute memory base

/irq   :IRQ number

/slt   :Slot no

P0 :The selectable window I/O base with 0x20 range.

The default value is 0x300.

P1 :The selectable window memory base.

The default value is 0xd000.

P2 :The selectable irq value.

The default value is 0x5.

P3 :The selectable slot number.

The default number is 1.

Example:

A>ae /iop:320 /mem:d000 /irq:a /slt:1

***************

INTEL: PCIC.EXE

***************

This PCIC.EXE program is used to open I/O window mapping, memory window

mapping, and IRQ of PCIC (82365SL) adapter for PCMCIA card.

To run drivers of DE-650, PCIC.EXE must be executed first.

The given parameters of DE-650 drivers must match the hardware parameters

settings of PCIC.EXE.  The parameters can be I/O base, attribute

memory base, and IRQ value.

Two methods could be used to execute PCIC.EXE.

Method 1

--------

From a DOS prompt:

A>pcic [/iop:P0] [/mem:P1] [/irq:P2] [/slt:P3] [/grp:P4]

Method 2

--------

From DOS config.sys:

device=pcic.exe [/iop:P0] [/mem:P1] [/irq:P2] [/slt:P3] [/grp:P4]

Parameter description:

/iop   :I/O base

/mem   :Attribute memory base

/irq   :IRQ value

/slt   :Slot type

/grp   :Group value

P0 :The selectable window I/O base with 0x20 range.

The default value is 0x300.

P1 :The selectable window memory base.

The default value is 0xd000.

P2 :The selectable irq value.

The default value is 0x5.

P3 :The selectable slot number.

The default number is 1.

P4 :The selectable grp value.

The default value is 0.

Group 0 means Index IO is 3E0h, Data IO is 3E1h, Base is 00h

Group 1 means Index IO is 3E0h, Data IO is 3E1h, Base is 80h

Group 2 means Index IO is 3E2h, Data IO is 3E3h, Base is 00h

Group 3 means Index IO is 3E2h, Data IO is 3E3h, Base is 80h

Example:

A>pcic /iop:320 /mem:d000 /irq:a /slt:1 /grp:0

******************

TOSHIBA: ICCNT.EXE

******************

The ICCNT.EXE program is used to open I/O window mapping, memory window

mapping, and IRQ of ICCNT adapter for the PCMCIA Ethernet Card.

To run drivers for DE-650, ICCNT.EXE must be executed first.

The parameters of DE-650 drivers must match the hardware parameters

settings of ICCNT.EXE.  The parameters could be I/O base, attribute

memory base, and IRQ value.

Two methods can be used to execute ICCNT.EXE:

Method 1

--------

From a DOS prompt:

A>iccnt [/iop:P0] [/mem:P1] [/irq:P2]

Method 2

--------

From DOS config.sys:

device=iccnt.exe [/iop:P0] [/mem:P1] [/irq:P2]

Parameter description:

P0 :The selectable window I/O base with 0x20 range.

The default value is 0x300.

P1 :The selectable window memory base.

The default value is 0xd000.

P2 :The selectable irq value.

The default value is 0x5.

Example :

A>iccnt /iop:320 /mem:d000 /irq:a

***************************************

Phoenix PCM Client driver: PCMDLINK.EXE

***************************************

Phoenix PCM software for Intel 82365SL chipset and other.

PCMSS.EXE    03-20-93 or above ;ExCA Socket Service driver

PCMCCU.EXE   03-25-93 or above ;PCMCIA Card Configuration Utility

PCMDLINK.COM   8421  01-07-93  ;PCMCIA card Client driver

* This file was provided by Phoenix,

it only support one fixed parameter:

IO: 320h, IRQ: 5, MEM: D6000h.

Only PCMCCU v1.3 need PCMDLINK.EXE.

Please use Jumpers to adject resource.

Note: If notebook is made by Chaplet, then PCMCCU will need parameter for

it. It is as the following

PCMCCU /CIRQ:5

Example:

1. IPX driver:

Content of CONFIG.SYS:

device=pcmss.exe

device=pcmccu.exe

Content of AUTOEXEC.BAT:

pcmdlink <-----  Only PCMCCU v1.3 need to execute

IPX	       PCMDLINK.EXE.

netx

f:

login supervisor

2. ODI driver:

Content of CONFIG.SYS:

device=pcmss.exe

device=pcmccu.exe

Content of AUTOEXEC.BAT:

pcmdlink <-----  Only PCMCCU v1.3 need to execute

lsl	       PCMDLINK.EXE.

de650odi

ipxodi

netx

f:

login supervisor

***************************

SystemSoft installation

With DE-650

***************************

Testing DE-650 with SystemSoft PCMCIA drivers (Intel 82365SL controller)

------------------------------------------------------------------------

[Preface]

The SystemSoft is a company that develope the PCMCIA drivers, like

Socket Service, Card Service and Card Installation Client drivers.

Now, they had provided drivers that support the Intel 82365SL PCMCIA and

Databook TCIC-2/N controller chips,,,etc.

SystemSoft's drivers for Intel 82365SL controller chip:

SS386SL.EXE 2-25-93  or above	 ;Socket Service driver 2.0 v1.14

CS.EXE	    3-09-93  or above	 ;Card Service driver 2.00 v4.01

CIS.EXE     3-05-93  or above	 ;Card Installation Client driver

v2.01

For example with IPX driver:

The content of the CONFIG.SYS file:

:

DEVICE=SS386SL.EXE

DEVICE=CS.EXE

DEVICE=CIS.EXE	/LIRQ 5 /NWBAS D8

The content of the AUTOEXEC.BAT file:

:

DE650IPX	;Using IRQ 5, I/O 300h, MEMORY D800

NETX

F:

LOGIN JONES

LANTEST

WIN LANTEST

[Parameters Explanation]

The following parameters explanation for your reference. If you need more

(whole) detail parameters explanation, please contact with me.

SS365SL.EXE

/PORT hhh	Specifies the base port address of the first of uo to two PCIC

Adapter boards. SS386SL assumes the each Adapter responds to

two consecutive I/O ports for its address/data ports and that

all of four ports are consecutive if two Adapters are present.

The default port address are 3E0h/3E1h for the first Adapter

and 3E2h/3E3h for second Adapter (/PORT 3E0).

/SKT n	Specifies the number of supported sockets (1-4). The default

is 2 (/SKT 2).

CS.EXE

/IRQ h	Specifies the IRQ number to be used for the card-status-change

interrupt. The default value is 0Fh (/IRQ F). The allowable

range is 0h through 0Fh.

CIC.EXE

/BAS hh	Specifies the base address in system memory of the memory

window used to access Attribute and Common Memory space during

card recognition and initialization. The value must be states

in 4K units. The default value is D0h (/BAS D0) which specifie

a window string at D0000:0h. The base address specified must b

in the range C0 through EFh.

/SIZ hh	Specified the size of the system memory window. The size is

stated in 4K units (where 1=4K, 2=8K, 3=12K, etc.). The defaul

is 4h (/SIZ 4) which signifies a 16K window.

/CSBAS hh	Specifies the base segment address of a window area given to

Card Services with an AdjustResoureInfo function call. The

value must be in 4K units. The default value is D4h (/CSBAS D4

, which specifies a window starting at D400:0h. The base

address specified must be in the range C0h to EFh.

/CSSIZ hh	Specifies the size of a window area given to Card Services wit

an AdjustResourceInfo function call. The size is stated in 4K

units (where 1=4K, 2=8K, 3=12K, etc.) with 64K being the

maximum size. The default value is 4h (/CSBAS 4), which

signifies a 16K window. If the specified value is 0, the

AdjustResourceInfo function call is not made.

/NWBAS hh	Specifies the base segment address of a attribute or common

memory tobe opened. The value must be in 4K units. The default

value is D8h (/NWBAS D8), which specifies a window starting

at D800:0h. The base address specified must be in the range

C0h through EFh. This switch applies to network cards that

require an attribute or common memory window to be left open

after configuration.

/NWSIZ hh	Specifies the size of memory window to be left open. The size

is stated in 4K units (where 1=4K, 2=8K, 3=12K, etc.) with 64K

being the maximum size. The default value is 4h (/CSBAS 4),

which signifies a 16K window. The applies to network cards tha

require an attribute or common memory window to be left open

after configuration.

/LIRQ hh	Specifies the IRQ that is assigned to LAN cards. The default

is 5h (/LIRQ 5). The allowable range is 0h through 0Fh.

[Note]

*  Make sure that the memory windows (/BAS, /CSBAS, /NWBAS) do not overlap.

The default values for these switches prevent a memory window conflict.

Use caution if changing the default values.

The other hand, if you use some memory management software, please avoid

the memory conflict with above memory windows. For example: if you use

the Microsoft's EMM386.EXE, please attach with "X=D000-DBff" parameter

(for default values of /BAS, /CSBAS, /NWBAS).

*  By the way, Older SystemSoft drivers is also work fine with DE-650.

SS386SLA.EXE  11-17-92 or above   ;Socket A Service driver v2.0

SS386SLB.EXE  11-17-92 or above   ;Socket B Service driver v2.0

CS.EXE	      11-24-92 or above   ;Card Service driver v2.0

CIS.EXE       11-24-92 or above   ;Card Installation Client driver

v1.00

[Summary]

If you want to adjust the IRQ, I/O or MEMORY for DE-650 driver, you only

need to adjust the parameters of the CIC.EXE file.