8003/8013 FAMILY LAN ADAPTER DRIVER

FOR USE WITH DEC's DECnet-DOS and PCSA/PC

INSTALLATION GUIDE

The DLL.EXE driver contained on this diskette is for use with the

8003/8013 family LAN adapters for Ethernet, StarLAN, and Ethernet for

twisted pair wiring.  This driver operates with DEC's DECnet-DOS

or PCSA/PC software.

This document contains information on how to install an 8003/8013

family LAN adapter and the DLL DECnet driver on DEC's DECnet network.

1. Jumper the adapter according to your system configuration.

The DLL device driver is installed at PC boot time according to

the command line specified in the AUTOEXEC.BAT file (described

below).  The user must check that the software settings given on

the device driver command line match the jumper settings on the

adapter.

Jumper settings and configuration information for some adapters

are described in the JUMPERS.DOC file.  For more information,

refer to the "User Installation Guide" that comes with each adapter.

For Micro Channel adapters, the interrupt, I/O base address,

and RAM base address are established by the system configuration

procedure when the adapter is installed in the machine.

2. Insert the adapter in your PC.

3. Power up your PC.

4. Install DEC's software

Follow the appropriate instructions for either DECnet-DOS or

PCSA/PC from Digital Equipment, selecting the "3COM"  option

whenever asked to choose an adapter type.  Specifics for the

hardware are defined below.

Usage of DLL parameters is as follows:

XT/AT(*) bus adapters:

dll [/irq:x] [/io:xxx] [/sm:xxxx] [/size:xx] [database_dir]

Micro Channel adapters:

dll [/slot:x] [database_dir]

All adapters:

dll /u

irq:x	Specifies IRQ level in decimal

(default 3)

io:xxx	Specifies hex I/O base address

(default 280h)

sm:xxxx	Specifies hex Shared Memory segment

(default D000h)

size:xx	Specifies Shared Memory size in decimal

Kilobytes (default 8)

slot:x	Specifies slot number of adapter (defaults

to slot number LAN adapter is installed in

if only one is installed in machine)

database_dir

Pathname of directory where you can find

DECPARM.DAT file (defaults to \DECNET\

on the current drive)

/u	Unload a previously running copy of dll

4A. For DECnet-DOS: Install the DECnet-DOS network software

using the DIP.EXE installation program supplied on the

DECnet-DOS diskette.  Select ETHERNET communications during the

installation.

Next, copy the 8003/8013 family driver DLL.EXE from this diskette

to the \DECNET directory of your system drive.  For example,

COPY A:\DECNET\DLL.EXE C:\DECNET\DLL.EXE

Next, edit your AUTOEXEC.BAT file to load the DLL driver as

described in step 5.

4B. For PCSA/PC: Have your system administrator create a KEY

diskette for your system.  The KEY diskette should be created

with your DECnet address and your personal startup files.

Next, copy the 8003/8013 family driver DLL.EXE from this diskette

to the appropriate directory of your KEY diskette.  For example,

with your KEY diskette in drive A: and this diskette in drive B:,

type:

COPY B:\DECNET\DLL.EXE A:\xxxxxx\DLL.EXE

Next, edit file MSNET.INI on your KEY diskette as described in

step 5A or 5B.  (Make sure to change ALL occurrences of DLL.)

5A. XT/AT bus adapters: You need to edit your AUTOEXEC.BAT or

MSNET.INI files to load the DLL.EXE driver and to reflect the

jumper or configuration selections of step 1.  Use the format:

C:\DECNET\DLL /IRQ:irq /IO:base /SM:page /SIZE:size (AUTOEXEC.BAT)

or

DLL /IRQ:irq /IO:base /SM:page /SIZE:size (MSNET.INI)

where,

irq- IRQ level to receive adapter interrupts in decimal (3 is

factory default)

base- I/O base address in hexadecimal, jumper W1 (280 is factory

default)

page- 4-digit hexadecimal memory segment byte address of the

shared buffer memory (not set by jumpers) (D000 is

default)

size- size of the shared memory on the adapter in decimal (8 is

default)

The actual DLL.EXE defaults are the same as the adapter defaults

shown in parentheses.

For example, the following statement corresponds to the default

factory configuration of the adapter for base I/O and IRQ, and an

8K buffer memory base address of DE000 (which is not the default):

DLL /IRQ:3 /IO:280 /SM:DE00 /SIZE:8

To change the settings from the command line for version 1 of

DECnet-DOS, you may use the version 1 format using "=" instead of

the version 2 format using ":" as the delimiter.  For example,

DLL /IRQ=3 /IO=280 /SM=DE00 /SIZE=8

or

DLL /IRQ:3 /IO:280 /SM:DE00 /SIZE:8

Your "irq" options for the /IRQ Interrupt parameter are: 2, 3, 4,

5, 6 (on some adapters) or 7 for both 8003 and 8013 series adapters,

as well as 10, 11, or 15 for 8013 series adapters.

Your "base" options for the /IO I/O base address parameter

are: 3E0, 3C0, 3A0, 380, 360, 340, 320, 300, 2E0, 2C0, 2A0, 280,

260, 240, 220, or 200.  (Beware: Conflicts may occur above 360.)

All I/O base addresses supported by the driver may not be supported

by every 8003/8013 family adapter.  Refer to the adapter's user

installation guide for I/O base addresses supported by the adapter.

Your "page" options for the /SM shared memory parameter

are: 8000 - F000, on 8K-byte boundaries (increments of 200 hex).

EGA and VGA can create problems below the D000 and D400 address

respectively.  (The DLL.EXE driver defaults to D000; there are no

jumpers to change this value on the adapter.)

Your "size" options for the /SIZE shared memory size

parameter are: 8, 16, or 32.  (This is the size of the shared

memory in kilobytes).  The default is 8.  Refer to your adapter's

user installation guide to determine the size of your adapter's

shared RAM buffer.

5B. Micro Channel Bus adapters: You need to edit your AUTOEXEC.BAT

or MSNET.INI files to load the DLL.EXE driver. Check that the

automated installation procedure has not added command line options

which are not needed for the Micro Channel version.  Use the format:

C:\DECNET\DLL        (AUTOEXEC.BAT)

or

DLL                  (MSNET.INI)

For Micro Channel adapters, IRQ, IO, SM, and SIZE should not be

specified (and will be ignored, except an error message will be

generated).  The actual values used are those set by the system

configuration procedure for the adapter.

If you have more than one 8003/8013 family Micro Channel LAN

adapter installed in your system, you must tell DLL which one to

use.  The adapter is referenced by the slot in which it is installed,

using the /SLOT parameter.

For example, if you have an Ethernet adapter in slot 5, and a

StarLAN adapter in slot 3, and you wish to run DECnet using the

Ethernet adapter, use:

DLL /SLOT:5

Even though you can specify SLOT when a single adapter is installed,

you should let SLOT default.  Then if you later move the adapter to

another slot, you will not need to change AUTOEXEC.BAT or MSNET.INI.

6. Reboot the system.

Licensed by Standard Microsystems for use by its customers only.

Standard Microsystems is a registered trademark of Standard

Microsystems Corporation. Other trademarks mentioned herein belong

to their respective companies.

Copyright (c) 1991 Standard Microsystems Corporation

All Rights Reserved