8003/8013 FAMILY LAN ADAPTER BOARD DRIVER

FOR USE WITH DEC's DECnet-DOS and PCSA/PC

RELEASE INFORMATION

Version 4.01                     November 14, 1991

This directory contains the DECnet-DOS (*) and PCSA/PC driver

version 4.00 for the 8003/8013 family LAN adapters.

Refer to Installation materials from Digital Equipment

Corporation for information on the installation of the DEC

software.  Additional information on installation of 8003/8013

adapters can be found in the file INSTALL.DOC in this disk

directory.  Adapter configuration information can be found in

the User Installation Guide for the 8003/8013 LAN adapter

you are using.

MODIFICATION HISTORY

-- Version 4.01 (November 14, 1991) --

Fix a bug which could cause adapters with National 8390 NIC

chips to lock up when recieving large files.

-- Version 4.00 (October 14, 1991) --

Changed the sign-on message to Standard Microsystems Corp.

-- Version 3.04 (August 9, 1991) --

1. Added support for 8013EP/A and 8013WP/A.

2. Fixed the problem that hangs the machine when running Microsoft Windows

across the network if HIMEM.SYS is loaded in config.sys.

-- Version 3.02 (January 24, 1991) --

Fixed the problem in ver. 3.01 with 8013EBT card.

-- Version 3.01 (October 26, 1990) --

Fixed a problem running a 16-bit Ethernet adapter with a

16-bit VGA video adapter.  This version has been tested by

Western Digital.

This version contains modifications for the WD83C690 Network

Interface Controller.  This version or later versions must

be used with adapters which contain the WD83C690.  It is

recommended that the latest version of the driver be used

in all applications.

** NOTE!! **

Use of adapters which contain the WD83C690 with previous

versions of this driver may lead to data integrity problems.

-- Version 3.00 (August 16, 1990) --

This driver has been qualified by Digital Equipment

Corporation.  This is the initial release of the DLL

written based on DEC's OEM developer's kit for DLL developers

including a library of routines designed to maintain compati-

bility across releases of DECnet-DOS.  The main advantage of

this library is compatibility with the extended memory features

of DECnet-DOS 3.0.

This driver is only loosely based upon code from previous

versions of DLL for the 8003/8013 LAN adapters, so fitness of a

previous version of DLL for a particular purpose should

not imply that this version will also be suitable.

(*) DEC and DECnet are trademarks of Digital Equipment Corporation.

AT and PS/2 are registered trademarks and Micro Channel is a

trademark of International Business Machines Corporation.

Copyright (c) 1991 Standard Microsystems Corporation

All Rights Reserved