December 1991

D-Link Ethernet Pocket LAN Adapter

FTP PC/TCP Installation

==================================

The following files will be used to in this installation:

Files	       Size (Bytes)  Date     Time    Contents

-----------    ------------  -------  ------  -------------------------

DE600PD.SYS    7721	    12-06-91          DE-600 Packet Driver

DE600PD.COM    7858	    12-06-91          DE-600 Packet Driver

The D-Link driver programs above let you run FTP's PC/TCP

with the D-Link Ethernet Pocket LAN Adapter.  Two methods can be

used:

METHOD 1:

If you use the DE600PD.SYS program, the following statement must

be included in your CONFIG.SYS file:

DEVICE = DE600PD.SYS 98

This statement may be placed anywhere in the CONFIG.SYS file.

After you boot up your PC, at DOS prompt, execute the ETHDRV.EXE

device driver program provided by FTP.

METHOD 2:

If you use the DE600PD.COM program, follow the procedure below:

1. Execute the DE600PD.COM as follows:

>DE600PD 98

where 98 is an interrupt vector number, which can be from 96

to 128.

2. Execute the ETHDRV.EXE device driver program (provided by

FTP).

You are now ready to use FTP's PC/TCP.