__________________________________________________________  AUTO BUSMASTER FAMILY ODI NETWARE LAN DRIVER, VERSION 1.34

About This File  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  2

Supported Platforms  . . . . . . . . . . . . . . . . . . . . . . . . . .  2

MPCO LAN Driver Installation Procedure   . . . . . . . . . . . . . . . .  3

NetWare Server   . . . . . . . . . . . . . . . . . . . . . . . . . . .  3

Configurable MPCO LAN Driver Parameters  . . . . . . . . . . . . . . . .  5

Sample MPCO.CFG Configuration File for NetWare Server  . . . . . . . . .  8

MPCO LAN Driver Performance Tuning   . . . . . . . . . . . . . . . . . .  9

NetWare Server   . . . . . . . . . . . . . . . . . . . . . . . . . . .  9

16/4 Auto Busmaster Family Adapter Custom Statistic Counters   . . . .   10

Messages for the MPCO LAN Driver   . . . . . . . . . . . . . . . . . .   14

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  1

ABOUT THIS FILE

This file describes the use of the Auto Busmaster Family ODI NetWare LAN

Driver (MPCO.LAN) with any of the 16/4 Auto Busmaster Family Adapters listed

below:

16/4 Auto Busmaster PCI Adapter

16/4 Auto Busmaster MC 32 Adapter

The term "16/4 Auto Busmaster Family Adapter" is used in this document to

refer to any of the adapters listed above.  This file is an extension of the

16/4 AUTO BUSMASTER FAMILY ADAPTER INSTALLATION AND TESTING INSTRUCTIONS that

are shipped with the respective adapter.

This file describes the installation and configuration of the 16/4 Auto

Busmaster Family Adapter Open Datalink Interface (ODI) NetWare MPCO LAN

driver.  It contains information about the following topics:

o   Supported platforms

o   Software installation procedures

o   Configurable MPCO LAN driver parameters

o   Performance tuning the MPCO LAN driver

o   16/4 Auto Busmaster Family Adapter Custom Statistic Counters

o   Messages for the MPCO LAN driver

SUPPORTED PLATFORMS

The MPCO LAN driver is supported in any of the hardware environments

supported by the 16/4 Auto Busmaster Family Adapters.  See the 16/4 AUTO

BUSMASTER FAMILY ADAPTER INSTALLATION AND TESTING INSTRUCTIONS for

information about supported hardware.

The following operating system environments are supported:

o   Novell NetWare 3.1x

o   Novell NetWare 4.0

o   Novell Netware for OS/2

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  2

MPCO LAN DRIVER INSTALLATION PROCEDURE

This section describes how to install the MPCO LAN driver on the NetWare

Server platform.

See the 16/4 AUTO BUSMASTER FAMILY ADAPTER INSTALLATION AND TESTING

INSTRUCTIONS that is shipped with the adapter for instructions on how to

install the 16/4 Auto Busmaster Family Adapter.

NetWare Server

This section describes how to install the MPCO LAN driver on a NetWare

Server.

To install the MPCO LAN driver on a NetWare Server, perform the following

steps:

1.  Choose a target directory path on the diskette or the fixed disk (for

example, C:\NW) for installation of the MPCO LAN driver and its

associated files.  All files (in step 2, below) should be copied to this

target directory.

2.  Copy the following files to the target directory:

o   MSM.NLM

o   TOKENTSM.NLM

o   ETHERTSM.NLM

o   MPCO.LAN

If you are installing on NetWare 3.11, copy the following NetWare 3.11

specific modules to the target directory:

o   PATCHMAN.NLM

o   LSLENH.NLM

o   MSM31X.NLM

3.  Add the following lines to the STARTUP.NCF file:

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  3

Set Minimum Packet Receive Buffers = 48

NOTES:

1.  If an 16/4 Auto Busmaster Family Adapter is installed in a 16-bit

slot in your computer or the adapter is a 16-bit adapter, add

Set Reserved Buffers Below 16 Meg = 48

2.  If more than one 16/4 Auto Busmaster Family Adapter is installed

in your computer, increase the value for the

above parameters by 48 times the number of

installed 16/4 Auto Busmaster Family Adapters.

4.  Add the following lines to the AUTOEXEC.NCF file:

Add the following two lines for NetWare 3.11 only:

LOAD C:\NW\PATCHMAN.NLM

LOAD C:\NW\LSLENH.NLM

LOAD C:\NW\MPCO.LAN @MPCO.CFG

NOTES:

a.  @MPCO.CFG is an optional command line parameter (see step 5, below).

b.  MPCO.LAN automatically loads the MSM.NLM (MSM31X.NLM if running

Novell NetWare 3.1x) TOKENTSM.NLM and ETHERTSM.NLM modules.

See NOVELL NETWARE SYSTEM ADMINISTRATION for more information about the

AUTOEXEC.NCF and STARTUP.NCF files.

5.  If you need to change any MPCO LAN driver parameter values from their

defaults, these parameters can be specified on the command line when the

MPCO LAN driver is loaded, or they can be specified by creating an

MPCO.CFG configuration file.  If the optional @MPCO.CFG command line

parameter is used to specify an MPCO.CFG configuration file, then the

configuration file must be located in the same directory as the MPCO.LAN

file.  You can use any text editor to create or edit the MPCO.CFG file.

See "Sample MPCO.CFG Configuration File for NetWare Server" for a sample

MPCO.CFG file.

6.  For your convenience, a copy of the LSLENH.NLM, PATCHMAN, MSM.NLM,

MSM31X.NLM, TOKENTSM.NLM, ETHERTSM.NLM, and MONITOR.NLM have been shipped

on the adapter options diskette.  PLEASE CONTACT NOVELL TO ENSURE YOU

HAVE THE LATEST COPY OF THESE FILES.

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  4

CONFIGURABLE MPCO LAN DRIVER PARAMETERS

The following parameters can be configured at the NetWare Server by setting

keywords on the command line or in the MPCO.CFG file.  A sample MPCO.CFG file

is shown in "Sample MPCO.CFG Configuration File for NetWare Server."

SLOT=

This keyword defines the slot number of the corresponding 16/4 Auto

Busmaster Family Adapter in the computer.  To avoid being prompted to

enter a slot number when the MPCO LAN driver is loaded, specify this

keyword in the AUTOEXEC.NCF file.  This keyword must be within the

range of 1-802.

o   1-8

For  16/4 Auto Busmaster MC 32 Adapter

o   17-24

For  16/4 Auto Busmaster PCI Adapter config m1 off

FRAME=

This keyword defines the frame type to be used with the 16/4 Auto

Busmaster Family Adapter when loading the MPCO LAN driver.

For token-ring 16/4 Auto Busmaster Family Adapters the default value

for this keyword is TOKEN-RING.  The value for this keyword must be

one of the following ascii strings:

o   TOKEN-RING

o   TOKEN-RING_SNAP

For ethernet 16/4 Auto Busmaster Family Adapters the default value for

this keyword is is ETHERNET_802.2 on versions of NetWare later than

3.11 and ETHERNET_802.3 for NetWare 3.11 and earlier.  The value for

this keyword must be one of the following ascii strings:

o   ETHERNET_802.2

o   ETHERNET_802.3

o   ETHERNET_II

o   ETHERNET_SNAP

NODE=

This keyword is used to set a locally administered address for the

16/4 Auto Busmaster Family Adapter.  A local address must contain 12

hexadecimal characters and can be specified in either MSB (Most

Significant Bit) or LSB (Least Significant Bit) format.

To specify a MSB node address the letter "M" immediately follows the

last digit of the node address.  If an "M" or "L" is not specified

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  5

after the last digit of the node address, the default format for the

address is MSB for token-ring and LSB for ethernet 16/4 Auto Busmaster

Family Adapters.  This keyword is specified on the command line or in

the MPCO.CFG as NODE for the NetWare Server.  See "Sample MPCO.CFG

Configuration File for NetWare Server" for a sample MPCO.CFG file.

If a node address is specified in MSB format, the characters must be

within the range 400000000000-7FFFFFFFFFFF (for example,

400000000001M).  This same address specified in LSB format would be

020000000080L with an "L" optionally appended to the end.

NOTES:

1.  Do not assign the same local address to 2 or more stations.

Severe station communication or network failures may occur when 2

or more stations are assigned the same address.

2.  If you do not assign a locally administered address, the adapter

uses the universally administered address permanently embedded on

the adapter.

RXBUFFERS=

This keyword defines the number of receive buffers used by the MPCO

LAN driver.

The value for this keyword must be: 4, 8, 16, 32, or 64; the default

is 32.  (Receive buffer size is defined by the server SET command.

See NOVELL NETWARE SYSTEM ADMINISTRATION for more information.)

TXBUFFERS=

This keyword defines the number of transmit buffers used by the MPCO

LAN driver.

The value for this keyword must be: 1, 2, 4, 8, or 16; the default is

16. (Maximum transmit frame size is defined by the server SET command.

See NOVELL NETWARE SYSTEM ADMINISTRATION for more information.)

TXINTSOFF

This keyword will reduce CPU utilization in server environments when

using the standard IPX protocol.  Reducing CPU utilization (shown on

the monitor screen) may improve performance if CPU utilization is very

high (above 80% utilized).  This option should not be used if Novell's

packet burst protocol (PBURST.NLM) is loaded, as performance on packet

burst links may be significantly degraded.  Use of this option is

recommended only for servers using the standard IPX protocol.

DATARATE

This keyword will set the Data Rate for the 16/4 Auto Busmaster PCI

Adapter only. It does not affect the adapter configuration for any

other 16/4 Auto Busmaster Family Adapter.

The value for this keyword must be: A, 16, 4 (AutoSense, 16MBPS,

4MBPS); the default is A.

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  6

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  7

SAMPLE MPCO.CFG CONFIGURATION FILE FOR NETWARE SERVER

; The following keywords are generic ODI keywords for MPCO

;  when using the 16/4 Auto Busmaster Family Adapter:

;

NODE=400000002222M

FRAME=TOKEN-RING_SNAP

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  8

MPCO LAN DRIVER PERFORMANCE TUNING

This section describes how to get the best performance from the 16/4 Auto

Busmaster Family Adapter and MPCO LAN driver.

NetWare Server

The maximum frame size that can be sent or received by a token-ring 16/4 Auto

Busmaster Family Adapter configured at a 4-Mbps data rate is 4496 bytes and

at a 16-Mbps data rate is 17996 bytes.  The maximum frame size that can be

sent or received by an ethernet 16/4 Auto Busmaster Family Adapter is 1518.

Since frame sizes are typically negotiated between the NetWare server and

workstations, it is best to configure receive buffer sizes at the server that

equal the maximum transmit buffer size at workstations.  For this reason,

increasing your receive buffer size to a value that is too big only increases

the amount of memory that is used by NetWare and may not increase the

performance of the 16/4 Auto Busmaster Family Adapter or NetWare.

Since the MPCO.LAN driver uses NetWare receive buffers to receive frames, the

size of receive buffers used by the MPCO.LAN driver is configurable by

changing the size of the NetWare receive buffers.  To obtain the best

performance from the 16/4 Auto Busmaster Family Adapter edit the STARTUP.NCF

file and modify the following NetWare parameter according to your

configuration as indicated below:

If a token-ring and/or ethernet 16/4 Auto Busmaster Family Adapter

is installed in your NetWare Server increase the following

parameter as shown

below:

Set Maximum Physical Receive Packet Size = 4202

If only an ethernet 16/4 Auto Busmaster Family Adapter is installed

in your NetWare Server increase the following parameter

as shown below:

Set Maximum Physical Receive Packet Size = 1518

If the above parameter is not found in the STARTUP.NCF, use the NetWare set

command to view system parameters and ensure that the default for this

parameter is greater than the values approriate for your configuration.  If

it is less, specify the above parameter in the STARTUP.NCF.  See

"Configurable MPCO LAN Driver Parameters" regarding the TXINTSOFF keyword

that may improve performance of the NetWare server.

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  9

16/4 AUTO BUSMASTER FAMILY ADAPTER CUSTOM STATISTIC COUNTERS

The custom statistic counters give the user information that is specific to

the 16/4 Auto Busmaster Family Adapter and MPCO LAN driver.  There follow

descriptions of the custom statistic counters and what action, if any, you

should take based on these counters.

Token-ring Utilization Percentage:

DESCRIPTION:  This counter indicates the percentage of the token-ring

bandwidth that is used.  This counter is updated every 30

seconds.

CAUSE/ACTION: None.

MAC Frame Count:

DESCRIPTION:  This counter indicates the number of token-ring Media Access

Control (MAC) frames sent by the MPCO driver.

CAUSE/ACTION: None.  These MAC frames were built and requested to be sent

by a protocol stack.  This counter is only valid for

token-ring 16/4 Auto Busmaster Family Adapters.

Receive Collisions:

DESCRIPTION:  This counter indicates the number of frames that experienced

a collision while being received by an ethernet 16/4 Auto

Busmaster Family Adapter.

CAUSE/ACTION: If this counter steadily increases, it may indicate a

problem with your ethernet 16/4 Auto Busmaster Family

Adapter or your ethernet network.  Run the adapter

diagnostic tests located on the Option Diskette that were

shipped with the 16/4 Auto Busmaster Family Adapter.  If

there are no problems with the adapter, reload the MPCO LAN

driver.  If this problem persists, contact your network

administrator.

Receive Frames Discarded:

DESCRIPTION:  This counter indicates the number of frames that were

received by the 16/4 Auto Busmaster Family Adapter but were

discarded by the MPCO LAN driver because they were received

with an error.

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  10

CAUSE/ACTION: If this counter steadily increases, it may indicate a

problem with your computer or the 16/4 Auto Busmaster Family

Adapter.  Run the adapter diagnostic tests located on the

Option Diskette that were shipped with the 16/4 Auto

Busmaster Family Adapter.  If there are no problems with the

adapter, reload the MPCO LAN driver.  If this problem

persists, contact your network administrator.

Transmit Frames Discarded:

DESCRIPTION:  This counter indicates the number of frames that could not

be transmitted by the 16/4 Auto Busmaster Family Adapter and

were discarded by the MPCO LAN driver.

CAUSE/ACTION: Possible explanations for this problem are as follows:

The 16/4 Auto Busmaster Family Adapter is not responding.

Run the adapter diagnostic tests located on the Option

Diskette that was shipped with the 16/4 Auto Busmaster

Family Adapter.  If there are no problems with the adapter,

reload the MPCO driver.  If a problem was found, follow the

instructions supplied with the diagnostic tests to correct

the problem.

If your 16/4 Auto Busmaster Family Adapter is connected to a

token-ring network, the token-ring network may be

experiencing problems and, therefore, the 16/4 Auto

Busmaster Family Adapter could not transmit these frames.

If this problem persists, contact your network

administrator.

Adapter Hardware Errors:

DESCRIPTION:  This counter indicates the number of hardware errors

reported by the 16/4 Auto Busmaster Family Adapter.

CAUSE/ACTION: The 16/4 Auto Busmaster Family Adapter and MPCO LAN driver

may recover from these errors.

If this counter continues to increase, Run the adapter

diagnostic tests located on the Option Diskette that was

shipped with the 16/4 Auto Busmaster Family Adapter.  If

there are no problems with the adapter, reload the MPCO LAN

driver.  If a problem was found, follow the instructions

supplied with the diagnostic tests to correct the problem.

Adapter Detected Parity Errors:

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  11

DESCRIPTION:  This counter is incremented when the 16/4 Auto Busmaster

Family Adapter detects a parity error while data is being

transferred between the 16/4 Auto Busmaster Family Adapter

and the computer.

CAUSE/ACTION: If this counter continues to increase, it may indicate a

problem with your computer or the 16/4 Auto Busmaster Family

Adapter.  Run the adapter diagnostic tests located on the

Option Diskette that was shipped with the 16/4 Auto

Busmaster Family Adapter.  If there are no problems with the

adapter reload the MPCO LAN driver.  If a problem was found,

follow the instructions supplied with the diagnostic tests

to correct the problem.

Adapter RAM Access Errors:

DESCRIPTION:  This counter is incremented when the 16/4 Auto Busmaster

Family Adapter reports a RAM access violation.

CAUSE/ACTION: If this counter continues to increase, and you are

experiencing problems with the 16/4 Auto Busmaster Family

Adapter, run the adapter diagnostic tests located on the

Option Diskette that was shipped with the 16/4 Auto

Busmaster Family Adapter.  If there are no problems with the

adapter, reload the MPCO LAN driver.  If this problem still

exists, contact your network administrator.

System Bus Errors:

DESCRIPTION:  This counter indicates the number of System Bus errors that

were detected.

CAUSE/ACTION: The 16/4 Auto Busmaster Family Adapter detected an error

with the System bus while accessing or attempting to access

the bus.

If this counter continues to increase, run the diagnostics

on the reference diskette that came with your computer to

test for problems with the Micro Channel bus.

Receive Hardware Mismatch Errors:

DESCRIPTION:  This counter is incremented when the MPCO LAN driver detects

a difference between the hardware reported frame length and

the frame length field in the IPX, IP, ARP or RARP protocol

header.

CAUSE/ACTION: If this counter continues to increase, contact your network

administrator.

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  12

Receive Overrun Errors:

DESCRIPTION:  This counter is incremented when the 16/4 Auto Busmaster

Family Adapter receives data from the network faster than it

can be moved into computer memory.

CAUSE/ACTION: If this counter continues to increase, run the diagnostics

on the reference diskette that came with your computer to

test for problems with the system bus.  If this problem

still persists, contact your network administrator.

Transmit Underrun Errors:

DESCRIPTION:  This counter is incremented when the 16/4 Auto Busmaster

Family Adapter attempts to transmit a frame to the network

but cannot move the data fast enough from computer memory to

the 16/4 Auto Busmaster Family Adapter.

CAUSE/ACTION: If this counter continues to increase, run the diagnostics

on the reference diskette that came with your computer to

test for problems with the system bus.  If this problem

still persists, contact your network administrator.

Transmit Checksum Errors:

DESCRIPTION:  This counter is incremented when the transmission of a frame

completes and the checksum value for the frame is incorrect.

CAUSE/ACTION: If this counter continues to increase, contact your network

administrator.

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  13

MESSAGES FOR THE MPCO LAN DRIVER

MPCO-xx-071: The matching virtual adapter could not be found.

Cause:  An error occurred while trying to load another copy

of the MPCO LAN driver to support an additional

frame type (FRAME= parameter) on an

16/4 Auto Busmaster Family Adapter.  The MSM could not find

a previous copy of the MPCO LAN driver required

to load the specified frame type.

Action: Obtain the most current copy of the MSM.NLM or

MSM31X.NLM (if running NetWare version 3.1x), and then

reload the MPCO LAN driver for the desired frame

types.  If this error persists, contact your

network administrator.  Refer to the

NOVELL NETWARE SYSTEM ADMINISTRATION manual

for information about loading and unloading LAN

drivers.

MPCO-xx-072: A resource tag is unavailable.

Cause:  The NetWare operating system was unable to allocate

memory for a resource tag.  The MPCO LAN driver

did not load because required internal resources

were not available.  There may be too many NetWare

Loadable Modules (NLMs) running in the NetWare Server.

Action: Unload the NLMs that are loaded but are not being used,

and then load the MPCO LAN driver.  If this error

persists, down, exit and then restart the server.  Load

the MPCO LAN driver before loading NLMs.  This may

require you to edit the AUTOEXEC.NCF file before restarting

the server.  Refer to NOVELL NETWARE SYSTEM ADMINISTRATION

for information about the AUTOEXEC.NCF file.

MPCO-xx-073: Unable to allocate memory.

Cause:  The NetWare operating system was unable to allocate

memory for the MPCO LAN driver.  The MPCO

LAN driver did not load.

Action: Unload the NetWare Loadable Modules (NLMs) that are

loaded but are not being used, and then load the MPCO

LAN driver.  If this error persists, down, exit and then

restart the server.  Load the MPCO LAN driver before

loading NLMs.  This may require you to edit the

AUTOEXEC.NCF file before restarting the server.  Refer to

NOVELL NETWARE SYSTEM ADMINISTRATION for information

about the AUTOEXEC.NCF file.

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  14

MPCO-xx-074: The hardware interrupt cannot be set.

Cause:  The NetWare operating system was unable to set the

hardware interrupt level requested by the MPCO

LAN driver.  The MPCO LAN driver did not load.

Action: Try to load the MPCO LAN driver again.  If the error

persists, contact your network administrator.

MPCO-xx-075: The MLID cannot be registered with the LSL.

Cause:  The MPCO LAN driver could not be registered with the

Link Support Layer (LSL) module.  The maximum number of

LAN drivers have already been loaded or there was not

enough memory to load the MPCO LAN driver.  The MPCO

LAN driver did not load.

Action: Unload LAN drivers that are not being used and then

load the MPCO LAN driver.  If this error persists,

down, exit and then restart the server.  Load the MPCO

LAN driver before loading other LAN drivers.  This may

require you to edit the AUTOEXEC.NCF file before restarting

the server.  Refer to NOVELL NETWARE SYSTEM ADMINISTRATION

for information about the AUTOEXEC.NCF file.

MPCO-xx-076: The polling procedure cannot be added.

Cause:  Too many polling processes are registered with

the NetWare operating system.  The MPCO LAN

driver did not load.

Action: Unload LAN drivers or NetWare Loadable Modules that

are loaded but are not being used.  Try to load the

MPCO LAN driver again.  If this error persists,

contact your network administrator.

MPCO-xx-077: The event notification routine cannot be registered.

Cause:  The NetWare operating system was unable to allocate

memory to register the MPCO LAN driver event

notification routine.  The MPCO LAN driver did

not load.

Action: Unload LAN drivers or NetWare Loadable Modules that

are loaded but are not being used.  Try to load the

MPCO LAN driver again.  If this error persists,

contact your network administrator.

MPCO-xx-078: The firmware file cannot be read.

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  15

Cause:  The Netware operating system was unable to load the

MPCO LAN driver.  The MPCO LAN driver was not

loaded.

Action: Obtain another copy of the MPCO LAN driver from

the original distribution diskette.  Reload the MPCO

LAN driver.  If this error persists, contact your network

administrator.

MPCO-xx-084: Unable to allocate memory below 16 Megabyte boundary.

Cause:  The Netware operating system ran out of memory buffers

below 16 megabytes.  The MPCO LAN driver was not

loaded.

Action: Set the Reserved Buffers Below 16 Meg parameter in the

STARTUP.NCF to a larger number.

MPCO-xx-086: The driver parameter block is too small.

Cause:  The MPCO.LAN file is designed to run with an older

version of the MSM.NLM.  The MPCO LAN driver was

not loaded.

Action: Obtain an older version of the MSM.NLM or a newer

version of the MPCO.LAN file.

MPCO-xx-087: The media parameter block is too small.

Cause:  The TOKENTSM.NLM NetWare Loadable Module (NLM) is

designed to run with an older version of the MSM.NLM

or MSM31X.NLM if running NetWare Version 3.1x.

Action: Obtain an older version of the MSM.NLM (MSM31X.NLM

if running NetWare Version 3.1x) or a new version of

the TOKENTSM.NLM.

MPCO-xx-091: The hardware configuration conflicts.

Cause:  The MPCO LAN driver requested use of a hardware

resource that was already being used by another

hardware device.  The MPCO LAN driver did not load.

Action: Change the hardware configuration of the

16/4 Auto Busmaster Family Adapter or the hardware

configuration of the conflicting hardware device.

MPCO-xx-092: Cannot schedule AES without an HSM routine.

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  16

Cause:  The MPCO.LAN file is damaged.

Action: Obtain another copy of the MPCO LAN driver from

the original distribution diskette.  Reload the MPCO

LAN driver.  If this error persists, contact your network

administrator.

MPCO-xx-093: Cannot schedule interrupt time call back without an

HSM routine.

Cause:  The MPCO.LAN file is damaged.

Action: Obtain another copy of the MPCO LAN driver from

the original distribution diskette.  Reload the MPCO

LAN driver.  If this error persists, contact your network

administrator.

MPCO-xx-094: Cannot set hardware interrupt without an HSM routine.

Cause:  The MPCO.LAN file is damaged.

Action: Obtain another copy of the MPCO LAN driver from

the original distribution diskette.  Reload the MPCO

LAN driver.  If this error persists, contact you network

network administrator.

MPCO-xx-095: Cannot add polling without an HSM routine.

Cause:  The MPCO.LAN file is damaged.

Action: Obtain another copy of the MPCO LAN driver from

the original distribution diskette.  Reload the MPCO

LAN driver.  If this error persists, contact your network

administrator.

MPCO-xx-126: The group bit in the node address override was cleared.

Cause:  The address specified for the node address parameter

is an incorrect adapter node address.  The group

address bit for the node address specified had been

set.  The group address bit was cleared by the MPCO

LAN driver.  The MPCO LAN driver was loaded.

Action: Correct the node address parameter to specify a

locally administered address according to the

IBM TOKEN-RING NETWORK ARCHITECTURE REFERENCE.

On a DOS or OS/2 workstation, change the node address

parameter in the NET.CFG file. On a NetWare Server,

change the node address parameter specified on the

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  17

command line when the MPCO LAN driver was loaded,

or change the node address specified in the

MPCO.CFG configuration file.

MPCO-xx-127: The local bit in the node address override was set.

Cause:  The address specified for the node address parameter

is an incorrect adapter node address.  The node address

specified does not have the locally administered address

bit set.  The locally administered address bit was set

by the MPCO LAN driver.  The MPCO LAN driver was

loaded.

Action: Correct the node address parameter to specify a

locally administered address according to the

IBM TOKEN-RING NETWORK ARCHITECTURE REFERENCE.

On a DOS or OS/2 workstation, change the node address

parameter in the NET.CFG file. On a NetWare server

change the node address parameter specified on the

command line when the MPCO LAN driver was loaded,

or change the node address specified in the

MPCO.CFG configuration file.

MPCO-xx-200: Initializing the 16/4 Auto Busmaster Family Adapter ... please

wait.

CAUSE: The 16/4 Auto Busmaster Family Adapter initialization process

has started.

ACTION: None.

MPCO-xx-201: Connecting the 16/4 Auto Busmaster Family Adapter to the

network.

CAUSE: The 16/4 Auto Busmaster Family Adapter is connecting to an

ethernet or token-ring network.

ACTION: None.

MPCO-xx-202: The 16/4 Auto Busmaster Family Adapter is closed.

CAUSE: The MPCO LAN driver could not connect the 16/4 Auto Busmaster

Family Adapter to the network due to previously reported

hardware problems.  The MPCO LAN driver has closed the

adapter and has generated an alert after unsuccesfully trying

to connect the adapter to the network after several times.

ACTION: Follow the recommended action for correcting any error

messages displayed by the MPCO LAN driver before this one.

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  18

MPCO-xx-204: The 16/4 Auto Busmaster Family Adapter requires a computer with

a

Micro Channel or Industry Standard Architecture (ISA) bus.

CAUSE: The 16/4 Auto Busmaster Family Adapter will work only in a

computer that supports the IBM Micro Channel Architecture or

the Industry Standard Architecture.

ACTION: Install the 16/4 Auto Busmaster Family Adapter in a computer

that supports the IBM Micro Channel Architecture or the

Industry Standard Architecture.

MPCO-xx-205: The MPCO LAN driver was not loaded

because of an incorrect keyword value.

CAUSE: The value for one of the keywords specified on the command

line or in the MPCO.CFG file was not within the allowable

range.  The MPCO LAN driver did not load.

ACTION: Correct the value of the keyword displayed in the message

that was displayed above this error message, and reload the

MPCO LAN driver.

MPCO-xx-207: The 16/4 Auto Busmaster Family Adapter could not be reset.

CAUSE: The MPCO LAN driver timed out while waiting for the 16/4 Auto

Busmaster Family Adapter to complete a hardware reset.  The

MPCO LAN driver did not load.

ACTION: Restart your computer and try to load the MPCO LAN driver

again.  If this error persists, contact your network

administrator.

MPCO-xx-209: The MPCO LAN driver failed to initialize the 16/4 Auto Busmaster

Family Adapter.

CAUSE: There was a problem while initializing the 16/4 Auto

Busmaster Family Adapter.  There may be previous error

messages above this one or the 16/4 Auto Busmaster Family

Adapter may not be responding.

ACTION:

Correct any problems with error

messages displayed above this error

message, and then reload the MPCO LAN

driver.

If this is the only error message that is displayed, restart

your computer and check the adapter by running the 16/4 Auto

Busmaster Family Adapter diagnostic tests packaged with the

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  19

adapterx.  Then, reload the MPCO LAN driver.  If this error

persists, contact your network administrator.

MPCO-xx-210: Could not find an 16/4 Auto Busmaster Family Adapter in this

computer.

CAUSE: There is no 16/4 Auto Busmaster Family Adapter installed in

this computer or the PCI configuration for the system has

disabled the 16/4 Auto Busmaster PCI Adapter .  The MPCO LAN

driver did not load.

ACTION: Install an 16/4 Auto Busmaster Family Adapter in this

computer if it has a 80386 or higher microprocessor or

reconfigure the PCI configuration , and then reload the MPCO

LAN driver.

MPCO-xx-211: The 16/4 Auto Busmaster Family Adapter was not found in the

specified slot(MC) or specified DMA Channel(ISA).

CAUSE: There is no 16/4 Auto Busmaster Family Adapter in the

slot(MC) or DMA(ISA) specified by the SLOT(MC) or DMA(ISA)

keyword.

ACTION: Correct the SLOT(MC) or DMA(ISA) keyword so that it indicates

a slot(MC) number or DMA(ISA) value in which an 16/4 Auto

Busmaster Family Adapter is located, and then reload the MPCO

LAN driver.

If there is an 16/4 Auto Busmaster Family Adapter in the

specified slot(MC) or DMA(ISA), contact your network

administrator.

MPCO-xx-212: Insufficient memory available for configured transmit and

receive buffers.

CAUSE: During initialization of the MPCO LAN driver, there was

insufficient memory available for the configured number of

transmit and receive buffers.

ACTION: Increase the number of buffers that the server operating

system can allocate by editing the STARTUP.NCF file on the

server.  Increase the Minimum Packet Receive Buffers server

parameter in the STARTUP.NCF file by the total number of

transmit and receive buffers that the MPCO LAN driver is

using.  Down, exit, and then restart the NetWare Server.

If the 16/4 Auto Busmaster Family Adapter is installed in a

16-bit slot in your computer add or increase the following

parameter in the STARTUP.NCF file:

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  20

Set the Reserved Buffers Below 16 Meg = total transmit and

receive buffers used by the MPCO LAN driver.

If this error still persists, unload the LAN drivers and

NetWare Loadable Modules (NLMs) that are loaded but are not

being used at the NetWare Server.

If this error still persists, decrease the number of buffers

that are being used by the MPCO LAN driver by specifying the

transmit and receive keywords for the MPCO LAN driver.  See

"Configurable MPCO LAN Driver Parameters" for information

about specifying keywords for the operating system that you

are using.

MPCO-xx-213: The slot(MC) or DMA(ISA) to use for the 16/4 Auto Busmaster

Family Adapter

was not specified.

CAUSE: The MPCO LAN driver prompted the user to select a slot(MC)

number or DMA(ISA) value that has an 16/4 Auto Busmaster

Family Adapter installed but no choice was made.

ACTION: None.  The MPCO LAN driver was not loaded.

MPCO-xx-215: The value specified for RXBUFFERS is not valid.

The value for RXBUFFERS has been changed to xx.

VARIABLE DATA: Valid RXBUFFERS values (xx) are 4, 8, 16, 32 or 64.

CAUSE: An incorrect value was specified for RXBUFFERS either on the

command line or in the MPCO.CFG file for the server.  The

MPCO LAN driver changed the value for RXBUFFERS to the value

given in this error message.

ACTION: None.  However, to keep this message from being displayed

when the MPCO LAN driver is loaded, change the value

specified by the RXBUFFERS keyword to one of the supported

values displayed above.  See "Configurable MPCO LAN Driver

Parameters" for the supported values.

MPCO-xx-216: The value specified for TXBUFFERS is not valid.

The value for TXBUFFERS has been changed to xx.

VARIABLE DATA: Valid TXBUFFERS values (xx) are 1, 2, 4, 8, 16.

CAUSE: An incorrect value was specified for TXBUFFERS either on the

command line or in the MPCO.CFG file for the server.  The

MPCO LAN driver changed the value for TXBUFFERS to the value

given in this error message.

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  21

ACTION: None.  However, to keep this message from being displayed

when the MPCO LAN driver is loaded, change the value

specified by the TXBUFFERS keyword to one of the supported

values for your operating system.  See "Configurable MPCO LAN

Driver Parameters" for the supported values.

MPCO-xx-219: The 16/4 Auto Busmaster Family Adapter reported an internal

error

when it was opened.

CAUSE: The open adapter command issued by the MPCO LAN driver to

connect to the netork failed.  An alert was generated by the

MPCO LAN driver.

ACTION: Restart the computer and try to load the MPCO LAN driver

again.  If this message continues to occur, check the adapter

by running the adapter diagnostic tests located on the Option

Diskette that was shipped with the 16/4 Auto Busmaster Family

Adapter.  If there is a problem with the adapter hardware,

contact your network administrator.  If there are no problems

with the adapter hardware, report this problem to your

support center.

MPCO-xx-221: The 16/4 Auto Busmaster Family Adapter received a disconnect

frame

from another adapter on the network.

CAUSE: The 16/4 Auto Busmaster Family Adapter received a disconnect

frame request that was sent by another adapter at a remote

station.  The 16/4 Auto Busmaster Family Adapter installed in

this computer disconnected from the network.

ACTION: To connect the 16/4 Auto Busmaster Family Adapter back to the

network, unload the MPCO LAN driver and then reload it.  If

this error persists, contact your network administrator.

MPCO-xx-223: The 16/4 Auto Busmaster Family Adapter failed to connect to the

network; another adapter is using the same node address.

CAUSE: The 16/4 Auto Busmaster Family Adapter could not connect to

the network because another station is using the same value

that was specified for the node address parameter at your

station.

ACTION: Change the node address parameter value specified for your

station.  This parameter is specified as NODE= at the NetWare

Server.  See "Configurable MPCO LAN Driver Parameters" for

more information about configuration parameters.

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  22

MPCO-xx-224: The 16/4 Auto Busmaster Family Adapter detected a wire fault

error

and closed the adapter.

CAUSE: The connection between the 16/4 Auto Busmaster Family Adapter

and the LAN has been broken.  The adapter or the adapter

cable may have developed a problem.

ACTION: Check the connection between the adapter and cable by running

the adapter diagnostic tests located on the Option Diskette

that was shipped with the 16/4 Auto Busmaster Family Adapter.

MPCO-xx-225: The token-ring network is beaconing.

CAUSE: The token-ring LAN segment to which the 16/4 Auto Busmaster

Family Adapter is connected is beaconing.  The ring is

experiencing an error condition detected by the adapter when

there is either a signal loss (possibly caused by a broken

line) or no token circulating the ring within the predefined

time limit.

ACTION: Contact your network administrator.

MPCO-xx-228: A Micro Channel error occurred; the adapter was reset.

CAUSE: An error was detected with the Micro Channel bus while data

was being transferred between the 16/4 Auto Busmaster Family

Adapter and the computer.  This condition is reported as an

alert.

ACTION: The adapter will retry the data transfer, and the MPCO LAN

driver Micro Channel Error statistics counter will be

incremented.

If this error persists, check the adapter by running the

adapter diagnostic tests located on the Option Diskette that

was shipped with the 16/4 Auto Busmaster Family Adapter, and

check your computer by running the hardware diagnostic tests.

If this error still persists, contact your support center or

place of purchase of the 16/4 Auto Busmaster Family Adapter.

MPCO-xx-229: The 16/4 Auto Busmaster Family Adapter reported a hardware

error.

CAUSE: The 16/4 Auto Busmaster Family Adapter reported an adapter

hardware error.  The adapter was closed.

ACTION: The MPCO LAN driver will increment the Adapter Hardware Error

counter, issue an alert, and try to reset the adapter.  The

MPCO LAN driver may recover from this error.  If not, run the

16/4 Auto Busmaster Family Adapter diagnostic tests packaged

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  23

with the adapter.  If there are no problems with the adapter,

reload the MPCO LAN driver.  If a problem was found, follow

the instructions supplied with the diagnostic tests to

correct the problem.

MPCO-xx-231: The alert condition was corrected.

CAUSE: The 16/4 Auto Busmaster Family Adapter, MPCO LAN driver, or

network has recovered from a previously reported alert

condition.

ACTION: None.

MPCO-xx-232: The 16/4 Auto Busmaster Family Adapter detected a parity error.

CAUSE: The 16/4 Auto Busmaster Family Adapter detected a parity

error while data was being transferred between the 16/4 Auto

Busmaster Family Adapter and the computer.

ACTION: The MPCO LAN driver will increment the Adapter Detected

Parity Error counter, and this condition is reported as an

alert.  The 16/4 Auto Busmaster Family Adapter and MPCO LAN

driver may recover from this error.  If not, run the 16/4

Auto Busmaster Family Adapter diagnostic tests packaged with

the adapter.  If there are no problems with the adapter,

reload the MPCO LAN driver.  If a problem was found, follow

the instructions supplied with the diagnostic tests to

correct the problem.

MPCO-xx-233: The 16/4 Auto Busmaster Family Adapter reported a RAM access

violation.

CAUSE: The 16/4 Auto Busmaster Family Adapter reported a RAM access

violation.  The MPCO LAN driver may have tried to access RAM

on the 16/4 Auto Busmaster Family Adapter that was not

accessible.

ACTION: The MPCO LAN driver will increment the Adapter RAM Access

Error counter, and this condition is reported as an alert.

The 16/4 Auto Busmaster Family Adapter and MPCO LAN driver

may recover from this error.  If not, run the 16/4 Auto

Busmaster Family Adapter diagnostic tests packaged with the

adapter.  If there are no problems with the adapter, reload

the MPCO LAN driver.  If a problem was found, follow the

instructions supplied with the diagnostic tests to correct

the problem.

MPCO-xx-234: The 16/4 ISA Busmaster Adapter did not complete POS setup.

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  24

CAUSE: The MPCO LAN driver timed out while waiting for the 16/4 ISA

Busmaster Adapter to complete POS initialization.  The MPCO

LAN driver did not load.

ACTION: Restart your computer and try to load the MPCO LAN driver

again.  If this error persists, contact your network

administrator.

MPCO-xx-235: The 16/4 ISA Busmaster Adapter needs 64K of I/O ports to run in

I/O mode. This system contains only 1K of I/O ports.  Set the 16/4 ISA

Busmaster Adapter to memory-mapped mode.

CAUSE: The 16/4 ISA Busmaster Adapter needs more than 1K of I/O

ports because all of the selectable I/O ports are above the

1K boundary.

ACTION: Either, set switch 7 of the 16/4 ISA Busmaster Adapter's

switch block to respond to memory mapped addresses or install

this adapter in a system with 64K of I/O ports.

MPCO-xx-236: Two 16/4 ISA Busmaster Adapters were found with identical DMA

channel switch settings.

CAUSE: Two 16/4 ISA Busmaster Adapters with duplicate DMA channel

values were found in the computer.

ACTION: Verify the settings of switches 9 and 10 for each

16/4 ISA Busmaster Adapter in the computer.

Each 16/4 ISA Busmaster Adapter must have a

unique DMA channel value.

MPCO-xx-237: Check the connection of the adapter cable to the

16/4 Auto Busmaster Family Adapter.

CAUSE: The 16/4 Auto Busmaster Family Adapter detected an error when

connecting to the network  This error can occur if there is

no adapter cable connected to the 16/4 Auto Busmaster Family

Adapter or there is an error in the network cabling.

ACTION: Check the connection of the adapter cable to the adapter and

ethernet network.  If the 16/4 Auto Busmaster Family Adapter

is connected to the network by a coaxial cable, check with

your network administrator to make sure the network cabling

is properly terminated.

MPCO-xx-238: Check the connection of the UTP adapter cable to the

16/4 Auto Busmaster Family Adapter.

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  25

CAUSE: The 16/4 Auto Busmaster Family Adapter is configured to use

the unsheilded twisted pair (UTP) connector on the adapter.

A UTP cable is not connected to the adapter UTP connector or

the UTP cable is not connected to the network.

ACTION: If there is no UTP cable connected to the 16/4 Auto Busmaster

Family Adapter UTP connector, connect one and then reload the

MPCO LAN driver.  If there is already a UTP cable connected

to the 16/4 Auto Busmaster Family Adapter, check with your

network administrator to make sure that the UTP cable is

properly connected to the network.

MPCO-xx-239: Check the connection of the STP adapter cable to the

16/4 Auto Busmaster Family Adapter.

CAUSE: The 16/4 Auto Busmaster Family Adapter is configured to use

the sheilded twisted pair (STP) connector on the adapter. A

STP cable is not connected to the adapter STP connector or

the STP cable is not connected to the network.

ACTION: If there is no STP cable connected to the 16/4 Auto Busmaster

Family Adapter STP connector, connect one and then reload the

MPCO LAN driver.  If there is already a STP cable connected

to the 16/4 Auto Busmaster Family Adapter, check with your

network administrator to make sure that the STP cable is

properly connected to the network.

MPCO-xx-240: Check the connection of the AUI adapter cable to the

16/4 Auto Busmaster Family Adapter.

CAUSE: The 16/4 Auto Busmaster Family Adapter is erx., check with

your network administrator to make sure that the STP cable is

properly connected to the network.

MPCO-xx-241:

Check the connection of the coax adapter cable to the 16/4

Auto Busmaster Family Adapter.

CAUSE: The 16/4 Auto Busmaster Family Adapter is configured to use

the coax connector on the adapter.  A coax adapter cable is

not connected to the adapter coax connector or the coax cable

is not connected to the network.

ACTION: If there is no coax cable connected to the 16/4 Auto

Busmaster Family Adapter coax connector, connect one and then

reload the MPCO LAN driver.  If there is already a coax cable

connected to the 16/4 Auto Busmaster Family Adapter, check

with your network administrator to make sure that the coax

cable is properly connected to the network.

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  26

MPCO-xx-242: 16/4 Auto Busmaster Family Adapter is configured for Token-ring

but is

connected to an ethernet network.

CAUSE: The adapter is incorrectly connected.

ACTION: Connect the adapter to a Token-Ring Network.

MPCO-xx-243: The IBM Streamer Family Adapter is configured for

Ethernet but is connected to a token-ring network.

CAUSE: The adapter is incorrectly connected.

ACTION: Connect the adapter to an Ethernet Network.

MPCO-xx-244: The configured ring speed for the IBM Streamer Family

Adapter is incorrect.  Change the ring speed to xx Mbps.

CAUSE: The adapter is configured to the wrong ring speed.

ACTION: Configure the adapter to the correct (xx) speed.

MPCO-xx-245: The IBM Streamer Family Adapter ring speed was temporarily

changed to xx Mbps to match the token-ring network.

CAUSE: The adapter is connected to an intelligent hub and was opened

at the wrong speed. The driver automatically changed the ring

speed to match.

ACTION: Configure the adapter for xx Ring Speed.

MPCO-xx-248: The IBM Streamer Family Adapter detected a parity error.

CAUSE: The 16/4 Auto Busmaster Family Adapter detected a parity

error while data was being received from the network by the

16/4 Auto Busmaster Family Adapter.

ACTION: The MPCO LAN driver will increment the Adapter Detected

Parity Error counter, and this condition is reported as an

alert.  The 16/4 Auto Busmaster Family Adapter and MPCO LAN

driver may recover from this error.  If not, run the 16/4

Auto Busmaster Family Adapter diagnostic tests packaged with

the adapter.  If there are no problems with the adapter,

reload the MPCO LAN driver.  If a problem was found, follow

the instructions supplied with the diagnostic tests to

correct the problem.

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  27

MPCO-xx-249: The value specified for DATARATE is not valid.

The Data Rate will default to AutoSense.

CAUSE: An incorrect value was specified for DATARATE either on the

command line or in the MPCO.CFG file for the server. The MPCO

LAN driver changed the value for DATARATE to the value given

in this error message.

ACTION: None.  However, to keep this message from being displayed

when the MPCO LAN driver is loaded, change the value

specified by the DATARATE keyword to one of the supported

values for your operating system.  See "Configurable MPCO LAN

Driver Parameters" for the supported values.

MPCO-xx-250: Invalid configuration detected, Interrupt 0 not allowed.

CAUSE: The MPCO LAN driver detected an invalid interrupt level. The

interrupt level is assigned by the system.

ACTION: Reconfigure the system PCI Slot which contains the 16/4 Auto

Busmaster PCI Adapter to a valid interrupt level (1-15).

MPCO-xx-251: Unable to read PCI PCR register.

CAUSE: The MPCO LAN driver cannot access the 16/4 Auto Busmaster PCI

Adapter

ACTION: Check the adapter by running the 16/4 Auto Busmaster Family

Adapter diagnostic tests packaged with the adapter.  If there

is a problem with the adapter hardware, contact your network

administrator.  If there are no problems with the adapter

hardware, report this problem to your support center.

MPCO-xx-252: Unable to write PCI PCR register.

CAUSE: The MPCO LAN driver cannot access the 16/4 Auto Busmaster PCI

Adapter

ACTION: Check the adapter by running the 16/4 Auto Busmaster Family

Adapter diagnostic tests packaged with the adapter.  If there

is a problem with the adapter hardware, contact your network

administrator.  If there are no problems with the adapter

hardware, report this problem to your support center.

MPCO-xx-253: Unable to read PCI IO Address register.

CAUSE: The MPCO LAN driver cannot access the 16/4 Auto Busmaster PCI

Adapter

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  28

ACTION: Check the adapter by running the 16/4 Auto Busmaster Family

Adapter diagnostic tests packaged with the adapter.  If there

is a problem with the adapter hardware, contact your network

administrator.  If there are no problems with the adapter

hardware, report this problem to your support center.

MPCO-xx-254: Unable to setup PCI routines for this instance.

CAUSE: The MPCO LAN driver cannot access the 16/4 Auto Busmaster PCI

Adapter

ACTION: Check the adapter by running the 16/4 Auto Busmaster Family

Adapter diagnostic tests packaged with the adapter.  If there

is a problem with the adapter hardware, contact your network

administrator.  If there are no problems with the adapter

hardware, report this problem to your support center.

MPCO-xx-255: Unable to read PCI GPR register.

CAUSE: The MPCO LAN driver cannot access the 16/4 Auto Busmaster PCI

Adapter

ACTION: Check the adapter by running the 16/4 Auto Busmaster Family

Adapter diagnostic tests packaged with the adapter.  If there

is a problem with the adapter hardware, contact your network

administrator.  If there are no problems with the adapter

hardware, report this problem to your support center.

MPCO-xx-256: Unable to read PCI GPR register.

CAUSE: The MPCO LAN driver cannot access the 16/4 Auto Busmaster PCI

Adapter

ACTION: Check the adapter by running the 16/4 Auto Busmaster Family

Adapter diagnostic tests packaged with the adapter.  If there

is a problem with the adapter hardware, contact your network

administrator.  If there are no problems with the adapter

hardware, report this problem to your support center.

MPCO-xx-257: Unable to read PCI ILR register.

CAUSE: The MPCO LAN driver cannot access the 16/4 Auto Busmaster PCI

Adapter

ACTION: Check the adapter by running the 16/4 Auto Busmaster Family

Adapter diagnostic tests packaged with the adapter.  If there

is a problem with the adapter hardware, contact your network

administrator.  If there are no problems with the adapter

hardware, report this problem to your support center.

Auto Busmaster Family ODI NetWare LAN Driver, Version 1.34  29