THE DOS ODI2NDI MODULE

______________________

OVERVIEW

________

The DOS ODI2NDI Module was developed to provide Novell

DOS Workstation support for IBM NDIS MAC Driver Adapters.

ODI2NDI is a transparent link between the Netware Link

Support Layer (LSL) and the NDIS MAC Driver. That is,

to the NDIS MAC Driver, ODI2NDI is an Network Device

Interface Specification (NDIS) Protocol Stack, and

to the Netware LSL, ODI2NDI is an Open Data-Link

Interface (ODI) MAC Driver. ODI2NDI allows coexistence

for Netware and other NDIS Protocol stacks.

ODI2NDI CONFIGURATION

_____________________

ODI2NDI is a Terminate and Stay Resident (TSR) program.

ODI2NDI is sensitive to the order in which Netware and

NDIS modules are loaded.  NDIS modules (with the excep-

tion of NETBIND) are device drivers that are loaded

during config.sys processing. The Netware Requestor

modules are TSR programs that are loaded either during

autoexec.bat processing or following autoexec.bat

processing. Therefore, the NDIS modules are loaded

prior to ODI2NDI (again, with the exception of

NETBIND) and the Netware Workstation modules.

LOAD ORDER

ODI2NDI has a REQUIRED load order with respect to the

Netware Requestor modules and the NDIS NETBIND. It must

be loaded after the Netware LSL.COM, and before the NDIS

NETBIND, ROUTE.COM and IPXODI.COM. In addition, the

NETBIND must be loaded prior to the IPXODI.COM.

For example:

LSL.COM

ODI2NDI.COM

NETBIND.COM

ROUTE.COM    (For Token Ring Source Routing)

IPXODI.COM

NETX.COM

CONFIGURATION PARAMETERS

ODI2NDI has no command line parameters but supports three

NET.CFG keywords and five PROTOCOL.INI keywords. NET.CFG

is the configuration file for the Netware Requestor envi-

ronment and PROTOCOL.INI is the configuration file for

the NDIS environment.

The supported NET.CFG keywords are:

o   NODE ADDRESS

o   FRAME

o   PROTOCOL

The supported PROTOCOL.INI keywords are:

o   DRIVERNAME

o   BINDINGS

o   NETADDRESS

o   TOKEN-RING

o   TOKEN-RING_SNAP

ODI2NDI reads the NET.CFG file first, and then reads the

PROTOCOL.INI file. The keywords in the PROTOCOL.INI will

override the keywords in NET.CFG. The NET.CFG file and

keywords are optional. The PROTOCOL.INI file is required.

There must be an ODI2NDI section, a DRIVERNAME and a

BINDINGS keyword. The instructions that follow will detail

the necessary configuration information.

INSTALLATION AND CONFIGURATION PROCEDURE

The following steps explain how to configure DOS ODI2NDI

NetWare Workstation support for IBM Adapters.

Step 1 - Install the IBM adapter hardware

o       Use the hardware installation documentation

provided with the adapter to install the

adapter hardware.

Step 2 - Install the NDIS MAC Driver

This can be accomplished in one of two ways:

1.  Install the IBM adapter hardware and use the LAN

Support Program V1.3x Installation Aid (DXMAID)

to install the NDIS MAC Driver for your adapter.

The LSP V1.3x Installation Aid will modify the

CONFIG.SYS, PROTOCOL.INI and AUTOEXEC.BAT files to

while installing the NDIS MAC Driver.

NOTE:  If this method of installation is used for the

NDIS MAC Driver,  Step 5 "Install Protocol Manager"

step can be skipped.

2.  Install the IBM adapter and NDIS MAC Driver using the

manual installation instructions provided with the

adapter.

Step 3 - Install the Netware Workstation Software

o   Install and configure NetWare Workstation Software on

the workstation. See the NetWare documentation for

instructions.

o   Use TOKEN.COM for the adapter ODI MAC Driver.

NOTE:  If you access a NetWare server through an IBM

Token-Ring Network bridge, be sure to install the

source routing module ROUTE.COM. IBM Token-Ring Networks

use source routing to move information between LANs.

********************************************************

Step 4 - Install ODI2NDI

o   Create an ODI2NDI directory.

o   Copy all files from the ODI2NDI\DOS directory on

the adapter device driver diskette to the ODI2NDI

directory.

o   Edit the AUTOEXEC.BAT file that was modified during

the Netware DOS Requestor installation.

Replace the line containing TOKEN.COM with the

following two lines:

*:\ODI2NDI\ODI2NDI.COM

*:\ODI2NDI\NETBIND.COM

These two statements will load ODI2NDI and bind

ODI2NDI to the NDIS MAC Driver.

NOTE:  If you installed the NDIS MAC Driver with the

LSP Installation Aid during Step 2 "Install the NDIS

MAC Driver", the install inserted a NETBIND invoca-

tion at the beginning of the autoexec.bat. Comment out

the NETBIND invocation from the autoexec.bat. The

NETBIND must occur after ODI2NDI has been loaded and

before IPXODI and NETX are loaded.

**************************************************

Step 5 - Install Protocol Manager

NOTE:  If the NDIS MAC Driver was installed using the IBM

LAN Support Program Installation Aid, or Protocol Manager

is already installed, this step can be skipped. Please

proceed to Step 6 "Modify the PROTOCOL.INI" file.

1.  Edit the CONFIG.SYS file on the root directory of

your boot drive.

a.  Insert a line at the top of the file to load

Protocol Manager. The Protocol Manager must be

the first device driver loaded. The inserted

line should be as follows:

DEVICE=*:\ODI2NDI\PROTMAN.DOS /i:*:\ODI2NDI

b.  The /i option indicates to the protocol manager to

search for the PROTOCOL.INI file in the *:\ODI2NDI

directory.

Step 6 - Modify the PROTOCOL.INI file

NOTE:  If the Protocol Manager was installed in Step 5

"Install the Protocol Manager", the sample PROTOCOL.INI

can be used. If the Protocol Manager was not installed in

Step 5, Use the example in the sample PROTOCOL.INI file

as a guide to update the existing PROTOCOL.INI file.

o   Add a protocol stack section for ODI2NDI with the

following lines:

[ODI2NDI_MOD]

DRIVERNAME=ODI2NDI$

The first is the Protocol stack section header, and

the second provides the protocol stack name. These two

lines are required.

o   There are four ODI2NDI PROTOCOL.INI keywords. The

following keywords should be placed following the

Protocol stack section header. The BINDINGS keyword

is required in the PROTOCOL.INI file.

-   BINDINGS=MACNAME

Add a BINDING keyword in the ODI2NDI section in

the PROTOCOL.INI file. This must be present or

the ODI2NDI can not function. A example is

included in the sample PROTOCOL.INI. Replace

MACNAME in the example with the name of the IBM

adapter NDIS MAC Driver. This name can be found

in the NDIS MAC Driver documentation for the IBM

adapter.

NOTE:  ODI2NDI is only capable of binding to one

NDIS MAC Driver at a time.

-   TOKEN-RING="yes" or TOKEN-RING="no"

TOKEN-RING_SNAP="yes" or TOKEN-RING_SNAP="no"

ODI2NDI supports two frame types. Frame Type can

be configured with a keyword entry in the

PROTOCOL.INI file.

Frame type can also be configured via a NET.CFG

file keyword. Step 7 "Modify the NET.CFG File"

covers modifying the NET.CFG file. ODI2NDI

defaults to frame type Token-Ring.

-   NETADDRESS="4000xxxxxxxx"

ODI2NDI requires that the NET ADDRESS be config-

ured to the same address in all NDIS protocol

stacks that will be binding to the NDIS MAC

Driver. Node Address can be configured with a

NETADDRESS keyword entry in the PROTOCOL.INI

file. The default NETADDRESS is the Universally

Administered Address.

Net Address can also be configured via a NET.CFG

file keyword. Step 7 "Modify the NET.CFG File"

covers modifying the NET.CFG file.

Step 7 - Modify the NET.CFG file

Using the example in the sample NET.CFG file on the

option diskette as a guide, add a LINK DRIVER section for

ODI2NDI. The LINK DRIVER line in the NET.CFG file should

be as follows:

LINK DRIVER name

"name" being the name of the NDIS MAC Driver that is

specified in the BINDINGS statement for ODI2NDI in the

PROTOCOL.INI file.

There are three supported NET.CFG keywords

o    NODE ADDRESS 4000xxxxxxxx

ODI2NDI reqiures that the NODE ADDRESS be configured

to the same address in all NDIS protocol stacks that

will be binding to the NDIS MAC Driver.

Node address can also be configured via the

PROTOCOL.INI file. Step 6 covers modifying the

PROTOCOL.INI file. ODI2NDI defaults to the uni-

versally administered address of the adapter.

o   FRAME

ODI2NDI supports two frame types. These can be

specified with a FRAME keyword entry in the

NET.CFG file. The two frame types are:

FRAME TOKEN-RING

FRAME TOKEN-RING_SNAP

Frame type can also be configured via the

PROTOCOL.INI file. Step 6 covers modifying the

PROTOCOL.INI file. ODI2NDI defaults to frame type

Token-Ring.

o   PROTOCOL IPX

This keyword allows ODI2NDI to be configured to

support various protocols.  ODI2NDI default protocol

is IPX.

Step 8

Install any IBM products that coexist with the NetWare

Requester (i.e. IBM LAN Support Program)

Step 9

Restart the workstation to activate DOS ODI2NDI NetWare

Requester support.