LNE390A/B DIAGNOSTICS

A diagnostic utility is provided to aid in installation and

configuration of your Mylex LAN adapter.

For the LNE390A the program file is TESTLNE.EXE

For the LNE390B the program file is TESTLNEB.EXE

LNE390A :

Configure the board according to the users manual for a thin

ethernet ( also known as 10 Base 2 ) . Make up a thin

ethernet cable such as Belden RG-58/U (at least 15 feet or

5 meters in length) with a BNC tee at each end and a 50 ohm

terminator on one port of each tee. Attach one tee to the LAN

adapter to be tested, leaving the other end free and run the

above program . This diagnostic will check the adapter base

address and interrupt request line against the configuration,

test the dual ported RAM and run a loop-back test with the

cable and terminators attached. The test may also be conducted

by selecting the AUI or the thick  ethernet port. In case of

thick ethernet port , a thick ethernet transceiver needs to be

attached to the AUI port on the card. Also the trasceiver needs

to be cabled with a thick ethernet cable and terminated at

both ends of the cable for the test. Now run the test.

LNE390B :

Configure the board using the EISA configuration for thin

ethernet ( also known as 10 Base 2 ) . Make up a thin

ethernet cable such as Belden RG-58/U (at least 15 feet or

5 meters in length) with a BNC tee at each end and a 50 ohm

terminator on one port of each tee. Attach one tee to the LAN

adapter to be tested, leaving the other end free and run the

program above. This diagnostic will check the adapter base

address and interrupt request line against the configuration,

test the dual ported RAM and run a loop-back test with the

cable and terminators attached. The test may also be conducted

by selecting the AUI or the thick  ethernet port. Select

Thick Ethernet port through EISA configuration . Connect a

thick ethernet transceiver to the AUI port on the card. Also

the trasceiver needs to be cabled with a thick ethernet cable

and terminated at both ends of the cable for the test. Now run

the test . In order to test out the 10 base T port , select the

10 Base T port in EISA configuraton. Connect the 10 Base T port

to a 10 Base T concentrator and run the test.

#################################################################################################