FILE NAME:  2XUPGRDE.DOC

DATE:       7 June 1993

PROGRAM:    2XUPGRDE.NLM v1.10

NETWIRE:    NOVLIB Library 6, file 2XTO3X.EXE

==========================================================================

Chapter 5

Choose Migration or In-Place Upgrade

Options for Upgrading Your Existing File Server

-----------------------------------------------

To upgrade an existing NetWare v2.1x or v3.x server, use either the

NetWare Migration utility or the NetWare In-Place Upgrade NLM. This

section provides you with information to help you decide which utility

is best for you.

Table 5-1, "NetWare Upgrade Options," below gives you a brief

explanation of upgrade options and the advantages of one option over

another.

The NetWare Migration utility may be used using one of two methods:

"Across-the-Wire Migration" and "Same-Server Migration." Table 5-1

lists both methods.

For a complete discussion of the NetWare Migration utility, see "What a

Migration Does".

For a complete discussion of the NetWare In-Place Upgrade NLM, see

"What an In-Place Upgrade Does".

Table 5-1

NetWare Upgrade Options

-----------------------------------------------------------------------

Upgrade        Options Available    Advantages        Disadvantages

-----------------------------------------------------------------------

NetWare v2.x   Across-the-Wire to   No risk of data   Need a v3.11

to v3.11       an installed v3.11   loss.             server installed.

server

Can migrate many

servers to one

server.

Can choose what

data you want to

migrate.

Can direct data to

specific volume or

directory.

--------------------------------------------------------

Same-Server          Can choose what   Some risk of data

migration            data you want to  lost.

migrate.

Need a

workstation (or

tape backup) with

enough disk space

for data files.

--------------------------------------------------------

In-Place Upgrade     Just need one     Cannot upgrade

from v2.1x or v2.2   v2.1x server.     from v2.0a.

to v3.11

Need an 80386 or

higher processor.

If upgrade fails,

you may have to

restore your

v2.1x data from

backup.

-----------------------------------------------------------------------

NetWare v3.x   Across-the-Wire      No risk of data   Need a v3.11

to v3.11       migration to an      loss.             server installed.

installed v3.11

server               Can migrate many

servers to one

server.

Can choose what

data you want to

migrate.

Can direct data to

specific volume or

directory.

--------------------------------------------------------

Same-Server          Can choose what   Some risk of data

migration            data you want to  loss.

migrate.

Need a

workstation (or

tape backup) with

enough disk space

for data files.

-----------------------------------------------------------------------

NetWare v3.x   In-Place Upgrade     Just need one     Cannot upgrade

to v3.11       from v3.x to v3.11   v3.x server       from v3.0.

--no additional

hardware needed.  If upgrade fails,

may have to

restore original

data from backup.

-----------------------------------------------------------------------

LAN Server or  Across-the-Wire      No risk of data   Additional

PCLP to        migration procedures loss.             hardware needed.

NetWare v3.11

-----------------------------------------------------------------------

LAN Server or  Same-Server          No additional     Some risk of data

PCLP to        migration procedures hardware needed.  loss.

NetWare v3.11

Need a

workstation (or

tape backup) with

enough disk space

for data files.

-----------------------------------------------------------------------

What a Migration Does

---------------------

(See either MIGRAT.EXE or MIGRAT.ZIP also on NetWire in NOVLIB Library 6.)

What an In-Place Upgrade Does

-----------------------------

An in-place upgrade allows you to upgrade a NetWare 2.1x server to

v3.11 using SERVER.EXE with the 2XUPGRDE.NLM.

Upgrading a NetWare v2.1x or v2.2 server includes two parts:

1. The file system is upgraded.

2. The new operating system is installed.

Upgrading a NetWare v2.1x file system has four phases:

1. The file system is analyzed/inventoried.

2. The disks are analyzed.

3. The disks are modified.

4. A NetWare v3.11 bindery is created to replace the v2.1x bindery.

Figure 5-3 illustrates the four phases.

Figure 5-3

Phases in the

Upgrade

v2.1x                                                              v3.11

+------------------\-----------------\---------------------\------------+

|  System Analysis  >  Disk Analysis  >  Disk Modification  >  Bindery  |

+------------------/-----------------/---------------------/------------+

Phase #1            Phase #2            Phase #3          Phase #4

Following is a discussion of what happens during each phase of the

upgrade.  When you run the upgrade, status messages refer to each of the

actions discussed here.

v2.1x

+------------------\

|  System Analysis  >

+------------------/

Phase #1

Phase 1. The NetWare v2.1x file system is analyzed.

o  Each disk is inventoried.

o  The system displays what volumes are on each disk.

o  This phase approximates the memory needed to successfully complete

an upgrade.

\-----------------\

>  Disk Analysis  >

/-----------------/

Phase #2

Phase 2. Each NetWare v2.1x disk is analyzed.

o  The locations of the Hot Fix area, the system and volume areas, the

Directory Entry Tables (DETs) and the File Allocation Tables (FATs)

are determined.  A layout of the disk for v3.11 is created from this

information.

o  A list of disk blocks that must be moved is created.

For example, if a block on VOL1: is on the disk in a location that

is now assigned to VOL0:, it must be moved to a location that was

assigned to VOL1: when the disk layout for v3.11 was created.

o  The DETs and FATs for each volume are translated.

o  Directory and file attributes are upgraded automatically.

Macintosh folders and files are also upgraded if you are running

NetWare for Macintosh.  The Macintosh name space module should also

be loaded (MAC.NAM).

o  This phase ensures the system has enough memory and free disk space

to complete the upgrade.

If the v2.1x disks do not have enough space for the upgrade to

occur, a warning appears and the upgrade stops.  (You must create

more room on the disk, and then restart the upgrade.)

\---------------------\

>  Disk Modification  >

/---------------------/

Phase #3

Phase 3. Each NetWare v2.1x disk is modified.

o  The partition table on track 0 is updated and the v3.11 system

tables are written to the system area.  The DETs and FATs for each

of the v3.11 volumes are duplicated.

o  Blocks are moved to their new locations on the disk. The v2.1x

system is not functional after this phase begins.

If a power failure or similar event occurs during this phase, you

must restore your v2.1x system from your backup.

v3.11

\------------+

>  Bindery  |

/------------+

Phase #4

Phase 4. The NetWare v2.1x bindery is upgraded to v3.11.

o  Most v2.1x bindery information is upgraded to v3.11. Other

information that is upgraded includes account restrictions and print

queues.

o  NetWare v2.1x passwords are not retained in the v3.11 bindery.  If

you chose to assign new passwords to users, those are created and

put into a file called NEW.PWD (in the SYS: SYSTEM directory).

o  VAPs do not get upgraded. They are unique to NetWare v2.x and do not

exist in NetWare v3.x.

Third-party VAP functionality is lost during this upgrade. After you

upgrade to v3.11, delete VAP files.

o  Core printing services are not upgraded. After you finish upgrading

to v3.11, delete print services and re-create them.

o  Volume and disk restrictions for users are not upgraded because of

differences between the v2.1x and v3.x file systems.

In v2.1x the restrictions were server-wide, whereas in v3.x the

restrictions are only per volume.

==========================================================================

Chapter 7

Upgrade Using In-Place Upgrade

Overview

--------

This chapter covers the procedures for upgrading an existing NetWare

server using the In-Place Upgrade NLM.

Use the In-Place Upgrade NLM for upgrading from an existing NetWare

v2.1x or v2.2 to a v3.11 or v4.0 server.

NetWare v2.1x or v2.2 Upgrade

=============================

NOTE: References in this section to NetWare v2.1x include NetWare v2.2.

A NetWare v2.1x server must have an 80386 or higher microprocessor to

be upgraded to a v3.11 server.

If your NetWare 2.1x server uses an 80286 microprocessor, see Chapter

6, "Upgrade Using Migration".

NOTE: NetWare v2.0 and v2.0a are not supported by the In-Place Upgrade

NLM.

Necessary Resources

-------------------

o  A working copy of the NetWare system diskettes.

o  A NetWare server running versions 2.10, 2.11, 2.12, 2.15 (revisions

a, b, and c), or 2.2.

o  A computer with an 80386 or higher microprocessor with a minimum of

4 MB of RAM. NetWare v3.x does not run on an 80286 microprocessor.

Servers with large disks and a large number of directories may need

more memory to complete the upgrade than they would normally need to

run the server after the upgrade.

o  A backup device, such as a tape drive or a DOS hard drive.

o  Enough disk space to accommodate the v3.11 file system. Each volume

should have at least 10% free disk space to accommodate the enlarged

directory and file allocation tables.

To store the v3.11 operating system files, we recommend you reserve

20-30 MB on volume SYS:.

We also recommend that you create at least a 5MB DOS partition on

the disk containing volume SYS:.

Ensure that no disk has more than eight volumes on it.

o  The NetWare System_1 diskette which contains the 2XUPGRDE.NLM.

o  Sufficient time to upgrade your server. This depends on the

following conditions:

o  The number of hard disks, not including mirrored disks

o  The amount of disk space in use

If you have several disks, and a lot of disk space in use, this

upgrade could take several hours.  See the "Memory and Timing Case

Study" in the README file.

o  (Optional) NetWare for Macintosh (to support Macintosh files on your

network).

Prerequisites

-------------

o  If you are running NetWare v2.1x on an 80286 microprocessor, upgrade

your computer to an 80386 or higher microprocessor.

o  Back up your v2.1x system twice to ensure that you have a good

backup.

A backup is essential if you must restore your v2.1x system.

o  Make sure that all users are logged out and that all files (except

bindery files) are closed during the upgrade.

o  Make sure all print servers are logged out.

o  You may need updated disk and LAN drivers to run v3.11. If you have

third-party drivers, check with the manufacturer before you upgrade

to see if you have compatible versions.

WARNING: If you are using the IDE.DSK disk driver, you cannot add a

DOS partition to an upgraded v3.11 disk.

Claiming a DOS partition with FDISK on an upgraded IDE disk causes

the IDE.DSK driver to get its parameters for heads, sectors, and

cylinders from the CMOS tables instead of from the disk drive.

The different parameters cause the DOS and NetWare partitions to

overlap, resulting in data loss or data corruption.

The NetWare partition will also be read from and written to

erroneously with the wrong drive parameters.

This problem occurs since the NetWare 2.1x IDE disk driver did not

conform to the same specifications as DOS.

The IDE driver is designed to check for the existence of a DOS

partition. If a DOS partition exists, then the driver uses the drive

parameters from the CMOS tables so that they will match what DOS is

using. Otherwise, the driver will use the drive parameters from the

NetWare 2.1x partition on the IDE disk drive.

Boot NetWare v3.11

------------------

Procedure

---------

IMPORTANT: Keep a record of your hardware configuration information to

be used at a later stage of the upgrade, as well as for future

reference.

NOTE: When instructed to insert a particular diskette, the diskette may

be different depending upon which version of NetWare you are using,

(ie: 3.11, 4.0).

1. At the NetWare v2.1x server system console prompt (:), type

CONFIG <Enter>

2. Copy the following information from the server screen onto the

NetWare v3.11 Server Worksheet provided at the back of this manual.

o  Server name

o  LAN configuration information (including the network address)

o  Disk channel configuration information

3. From a workstation, run BINDFIX.

NOTE: You must have rights to the SYS:SYSTEM directory to run

BINDFIX.

BINDFIX deletes mail subdirectories and trustee rights of users who

no longer exist on the network and fixes the incorrect records in

the bindery.

To delete mail subdirectories and trustee rights, answer "Y" to the

questions presented during BINDFIX.

SUGGESTION: You may want to copy the NET$BIND.OLD and NET$BVAL.OLD

files to diskette to recover the bindery later if needed.

For more information on "BINDFIX", see Utilities Reference.

4. Down the NetWare v2.1x server.

At the system console prompt (:), type

DOWN <Enter>

IMPORTANT: Do not down the server simply by turning its power switch

to OFF. This creates errors when upgrading to a NetWare v3.x file

system.

After the server responds that it is down, wait two to three minutes

to be sure that all transactions to system files are complete before

turning the power off.

5. Run VREPAIR.

From the downed server, run the v2.1x VREPAIR utility on each

volume.

See the v2.1x VREPAIR documentation for further instructions.

6. Insert the bootable INSTALL diskette into drive A:.

7. Turn the server off and then on again.

The computer now boots with DR DOS 6.0.

NOTE: If you are creating a DOS partition on the hard drive, record

the information you will enter in 8 through 13 on the worksheet at

the end of this manual. You will use this information again later.

8. With the System_1 diskette in drive A:, load the v3.11 operating

system by typing

SERVER <Enter>

You receive a message similar to the following:

Loading...

Novell NetWare v3.11

Processor speed: 193

(Type SPEED at the command prompt for an explanation of the speed

rating)

File server name:

NOTE: Don't type "SPEED" at this prompt (unless you want to name the

server "SPEED").

For an explanation of the speed rating (which is computer-specific),

wait until after you name the server and give it an internal network

number before you type SPEED at the console prompt.

9. Name the server by typing

servername <Enter>

SUGGESTION: Use your NetWare v2.1x servername so that you do not

have to change any login files, map statements, or batch files.

10. Assign an IPX internal network number to the server.

This number does not exist on a NetWare v2.1x network. In versions

v3.0, v3.1, v3.11, and v4.0, it is a logical network number that

identifies the individual file server.

This number must be different from other network numbers (for

cabling systems) or internal network numbers (for NetWare v3.0,

v3.1, v3.11, and v4.0 file servers).

The internal network number for each server must be

o  Unique

o  Hexadecimal (base 16: using numbers 0 through 9 and letters A

through F)

o  One to eight digits

11. At the cursor prompt, type the IPX internal network number and

press <Enter>.

The v3.11 console prompt (:) appears. The remaining steps are

executed from the console prompt.

12. Load the appropriate disk driver by typing

LOAD A:disk_driver <Enter>

Replace disk_driver with one of the v3.11 drivers.

Table 1-1

NetWare v3.11 disk drivers

-----------------------------------------------------------------------

Computer             Disk controller type    NetWare v3.11 disk driver

Architecture                                 (*.DSK)

-----------------------------------------------------------------------

Industry Standard    AT, MFM, RLL, ARLL,     ISADISK

Architecture (ISA)

ESDI                    ISADISK /b

IDE                     ISADISK /b /l

[lowercase "L"]

IDE (You cannot add a DOS

partition to an upgraded

v3.11 IDE disk.)

Novell disk             DCB

coprocessor board

-----------------------------------------------------------------------

Microchannel         ESDI                    PS2ESDI

MFM                     PS2MFM

IBM SCSI                PS2SCSI

-----------------------------------------------------------------------

Extended Industry    AT class                ISADISK

Standard

Architecture (EISA)

EISA vendor             See vendor

proprietary

Other devices may be

supported.

Contact the device

manufacturer to find out

if it has a v3.11

supported driver.

-----------------------------------------------------------------------

To decide which disk drivers to load, look at the disk channel

configuration information you recorded in Step 2. Load the

equivalent v3.11 disk driver.

Many more third-party disk drivers are available from the disk drive

manufacturers. Third-party disk driver manufacturers include

Adaptec

ADIC

Always Technology

Bus Logic

Data Technology Corp.

DPT

Future Domain

Quantum

UltraStor

IMPORTANT: Load the disk drivers in the order of the controller

boards. Load the driver for the internal controller first, the

driver for the first disk controller board second, etc.

If you do not follow the correct order, system messages about your

hard disks will be incorrect.

13. Answer the disk driver configuration information prompts by

entering the information you recorded in Step 2.

In NetWare v2.x, the interrupt number was decimal. In NetWare v3.x,

the interrupt number is hexadecimal.

14. (Optional) To test third-party LAN drivers for compatibility, load

them by typing

LOAD A: LAN_driver <Enter>

Replace LAN_driver with the third-party v3.11 LAN driver.

Table 7-2 lists all LAN drivers supported by Novell, Inc.

WARNING: Do not upgrade if LAN drivers are not compatible. Call the

manufacturer to get an updated version of the driver.

IMPORTANT: If you experience problems with third-party drivers,

contact the manufacturer listed in the driver description that

appears when you load the driver.

Table 7-2

NetWare v3.11 LAN drivers

-----------------------------------------------------------------------

Cabling System    Network Board              NetWare v3.11 LAN Driver

(*.LAN)

-----------------------------------------------------------------------

ARCnet            RX-Net                     TRXNET

RX-Net II

RX-Net/2

-----------------------------------------------------------------------

Ethernet          Etherlink II - ASSY 2227   3C503

(3C503)

EtherLink/MC               3C523

EtherLink Plus - ASSY 2012 3C505

NE/2                       NE2

NE/2-32                    NE2-32

NE1000 - ASSY 950-054401   NE1000

NE1000 - ASSY 810-160-001

NE2000 - ASSY 810-149      NE2000

NE3200                     NE3200

-----------------------------------------------------------------------

Token-Ring        16/4 Adapter               TOKEN

16/4 Adapter /A

PC Adapter II

PC Adapter /A

-----------------------------------------------------------------------

IBM PC Network    PCN baseband               PCN2

PCNA baseband

PCN2 broadband

PCN2/A broadband

-----------------------------------------------------------------------

Many more third-party LAN drivers are available from the NIC card

manufacturers. Third-party LAN driver manufacturers include

3COM

Cabletron

Hewlett-Packard

IBM

Intel

Madge

NCR

Proteon

Racal Datacom

Standard Microsystems

Thomas-Conrad

Ungermann-Bass

15. (Optional) Answer the LAN driver configuration information prompts

by entering the information you recorded in Step 2.

If the LAN driver loads, it can communicate with the network board

and is compatible with NetWare v3.11.

If the LAN driver does not load, it is either incompatible with

v3.11 or the configuration is wrong.

The following message appears:

Module xxx.LAN not loaded.

Check the configuration and try again. If you're sure the

configuration is correct, find out if you have the correct driver

version.

16. (Optional) Unload the third-party LAN drivers to free the memory

for the upgrade by typing

UNLOAD LAN_driver <Enter>

17. With the System_1 diskette in drive A:, load Macintosh name space

support by typing

LOAD A:MAC <Enter>

Loading the Macintosh name space module ensures that all volumes are

mounted properly at the phase 4 bindery step.

18. Continue the upgrade by determining which upgrade option to use.

See "Choose In-Place Upgrade Options".

Choose In-Place Upgrade Options

-------------------------------

Before doing an In-Place Upgrade, decide the following:

o  Run the upgrade interactively?

o  Run the upgrade as a batch process?

The following sections discuss both options to help you make your

decisions and then give you instructions for completing each task.

Run the Upgrade Interactively?

------------------------------

When you run the upgrade interactively, you are prompted whether to

continue from one phase of the upgrade to the next, whether to create

space for a DOS partition, and whether to have the NLM create and

assign new passwords.

Status messages and error messages are displayed.

If you are a first time installer, use the interactive method so you

can follow each phase of the upgrade.

Run the Upgrade as a Batch Process?

-----------------------------------

When you run the upgrade as a batch process, only error and status

messages are displayed. The upgrade automatically proceeds through the

phases without requiring input from the user.

Use the optional parameters described below to create space for a DOS

partition and to assign random passwords to users. If no options are

specified, the defaults are used.

If you are an experienced installer and have run several upgrades, use

the batch process method, especially if you are upgrading several

servers.

Run the In-Place Upgrade

------------------------

NOTE: The optional parameters can be entered whether the upgrade is run

interactively or as a batch process.

Procedure

---------

1. Insert the System_1 diskette into drive A:.

2. (Conditional) To run the upgrade interactively, type

LOAD A:2XUPGRDE <Enter>

A warning screen appears. You are asked the following question:

Do you have a recent backup of your server? y

Skip to Step 4.

3. (Conditional) To run the upgrade as a batch process, type

LOAD A:2XUPGRDE [optional parameters] <Enter>

Replace [optional parameters] with any of the following:

Table 7-3

In-Place Upgrade

optional parameters

---------------------------------------------------------------------

Parameter      Explanation

---------------------------------------------------------------------

B or BATCH  Runs the upgrade in batch mode.

BATCH2      Runs the upgrade in batch mode, not pausing for

non-critical errors.

BINDERY     Skips to phase 4 to upgrade the 2.1x bindery only.

F or FAST   Skips the memory and free disk space check.

H, ? or     Shows a list of parameters.

HELP

P0          Does not create space for a DOS partition.

Px          Creates space for a DOS partition on volume SYS:

(x = partition size in megabytes).

Size range: 0 to 32 MB.

Default: 5 MB.

R or R+     Assigns random passwords.

R-          Does not assign random passwords (default).

---------------------------------------------------------------------

You can enter the parameters in any order. Parameters can be

separated by either a slash (/), a minus sign (-), or a space.

For example, to run the upgrade as a batch process, create a 5MB DOS

partition, and assign random passwords, type

LOAD A:2XUPGRDE /B /P5 /R+ <Enter>

When the "Final Status" screen appears (Figure 7-7), continue with

Step 9.

4. To proceed with the upgrade, type "Y".

WARNING: If the procedure runs correctly (that is, if there are no

power failures or power surges, etc.), you do not need to restore

your disk from the backup.

However, if a failure occurs, you may need to restore your disk to

its previous condition.

Table 7-4 below explains the In-Place Upgrade phases.

Table 7-4

In-Place Upgrade Phases

---------------------------------------------------------------------

Phase                Description

---------------------------------------------------------------------

1 System Analysis    Inventories each disk and each volume.

Also ensures that the system has enough memory to

successfully complete the upgrade.

2 Disk Analysis      Analyzes each disk.

An image of the new NetWare v3.11 disk is built

and stored into memory.

This phase is non-destructive and does not damage

the existing NetWare v2.1x or v2.2 file system.

3 Disk Modification  Writes the new NetWare v3.11 file system to the

disk, destroying the NetWare v2.1x or v2.2 file

system.

A hardware failure during this phase will result

in the irrecoverable loss of data on the server.

4 Bindery            Updates bindery objects, properties, and values

such as user accounts and passwords. In NetWare

v2.1x, these were included in two files in

SYS:SYSTEM:

NET$BIND.SYS (for objects and properties)

NET$BVAL.SYS (for property data sets)

Random user passwords are assigned if requested.

---------------------------------------------------------------------

The DOS partition screen (Figure 7-1) appears.

Figure 7-1

DOS partition

screen

������������������������������������������������������������������������Ŀ

�����������������������������������������������������������������������ͻ�

��    In-Place Upgrade from NetWare v2.1x and v2.2 to NetWare v3.1x     ��

�����������������������������������������������������������������������ͼ�

�                                                                        �

�                    >>>>> OPTIONAL DOS PARTITION <<<<<                  �

�                                                                        �

�This utility can create space for a DOS partition at the beginning of   �

�the disk containing the SYS: volume.                                    �

�                                                                        �

�NetWare is much easier to load from a hard disk than from floppy disks. �

�In addition, NetWare 4.0 requires a minimum of a 5 MB DOS partition.    �

�                                                                        �

�If you do NOT want a DOS partition on this hard disk, enter 0.          �

�If you do want a DOS partition, enter the number of megabytes.          �

�                                                                        �

�Enter a number between 0 and 32: 5                                      �

��������������������������������������������������������������������������

5. At the prompt, enter the number of megabytes you want for the DOS

partition and press <Enter>.

NOTE: The In-Place Upgrade can create the space (between 0 and 32

MB) for a DOS partition on the disk that contains volume SYS:. We

recommend a DOS partition of at least 5 MB.

Booting the server from a DOS partition is significantly faster than

booting from a diskette.

The "0" option is available for servers that will boot from a

diskette rather than from a DOS partition.

If there is not enough space on your disk to create a DOS partition,

the upgrade stops and displays the following:.

"This NetWare server has insufficient free hard disk space to

complete the upgrade. The In-Place Upgrade process is now

being aborted."

NOTE: Reboot the v2.1x file server, and then restart the upgrade.

Delete unnecessary files to free more space on the disk.

After you create the DOS partition, screens similar to the following

appear.

Figure 7-2, "System Analysis screen", details the System Analysis

phase where each disk is scanned and inventoried.

Figure 7-2

System Analysis

screen

������������������������������������������������������������������������Ŀ

�����������������������������������������������������������������������ͻ�

��     In-Place Upgrade from NetWare v2.1x and v2.2 to NetWare v3.1x    ��

�����������������������������������������������������������������������ͼ�

�                                                                        �

�                   >>>>> PHASE #1: SYSTEM ANALYSIS <<<<<                �

�                                                                        �

�Scanning all disks for NetWare v2.1x and v2.2 volumes:                  �

�                                                                        �

�  Disk #0  (21000)     (CDC WrenII/Embedded SCSI)                       �

�     Volume SYS:                                                        �

�     Volume VOL1:                                                       �

�                                                                        �

�This server has 12.7 MB of memory.                                      �

�This upgrade will require approximately 10.2 MB of server memory.       �

�                                                                        �

�<Press ESC to terminate or any key to continue>                         �

��������������������������������������������������������������������������

Figure 7-3, "Disk Analysis screen 1", details the first portion of

the Disk Analysis phase.

This screen only appears if multiple disks are being upgraded.

These extra phases are run to determine how much server memory and

free disk space is required per volume to successfully complete the

upgrade.

Figure 7-3

Disk Analysis

screen 1

������������������������������������������������������������������������Ŀ

�����������������������������������������������������������������������ͻ�

��     In-Place Upgrade from NetWare v2.1x and v2.2 to NetWare v3.1x    ��

�����������������������������������������������������������������������ͼ�

�                                                                        �

�                    >>>>> PHASE #2: DISK ANALYSIS <<<<<                 �

�                                                                        �

�The system is now being analyzed to ensure that it has enough memory    �

�and free disk space to successfully complete the upgrade process.  High �

�capacity disks and disks with lots of directories may require more      �

�server memory to upgrade.                                               �

�                                                                        �

�Analyzing Disk #0                                                       �

�Creating list of disk blocks to be moved...COMPLETE                     �

�Creating list of Hot Fix Redirection Area blocks to be moved...COMPLETE �

�Translating the Directory Entry Table (DET) for volume SYS:...COMPLETE  �

�Translating the File Allocation Table (FAT) for volume SYS:...COMPLETE  �

�Translating the Directory Entry Table (DET) for volume VOL1:...COMPLETE �

�Translating the File Allocation Table (FAT) for volume VOL1:...COMPLETE �

�Creating the Volume Segment Table...COMPLETE                            �

��������������������������������������������������������������������������

Servers with only one hard disk proceed directly to the second part

of the Disk Analysis phase. See Figure 7-4.

The calculations made during the first part of the Disk Analysis

phase are discarded to conserve server memory.

As shown in Figure 7-4, "Disk Analysis screen 2", the Disk Analysis

phase analyzes each disk. An image of the new NetWare v3.11 disk is

built and stored in memory.

This phase is nondestructive and will not damage the existing

NetWare v2.1x file system.

Figure 7-4

Disk Analysis

screen 2

������������������������������������������������������������������������Ŀ

�����������������������������������������������������������������������ͻ�

��     In-Place Upgrade from NetWare v2.1x and v2.2 to NetWare v3.1x    ��

�����������������������������������������������������������������������ͼ�

�                                                                        �

�                    >>>>> PHASE #2: DISK ANALYSIS <<<<<                 �

�                                                                        �

�Analyzing Disk #0                                                       �

�Creating list of disk blocks to be moved . . . COMPLETE                 �

�Creating list of Hot Fix Redirection Area blocks to be moved...COMPLETE �

�Translating the Directory Entry Table (DET) for volume SYS:...COMPLETE  �

�Translating the File Allocation Table (FAT) for volume SYS:...COMPLETE  �

�Translating the Directory Entry Table (DET) for volume VOL1:...COMPLETE �

�Translating the File Allocation Table (FAT) for volume VOL1:...COMPLETE �

�Creating the Volume Segment Table...COMPLETE                            �

��������������������������������������������������������������������������

Figure 7-5, "Disk Modification screen", details the Disk

Modification phase.  In this phase, the v2.1x file system is

overwritten by the v3.11 file system.

The Disk Modification phase is the only destructive phase of the

upgrade. This phase must be completed before NetWare v3.11 can use

the disk.

Figure 7-5

Disk Modification

screen

������������������������������������������������������������������������Ŀ

�����������������������������������������������������������������������ͻ�

��     In-Place Upgrade from NetWare v2.1x and v2.2 to NetWare v3.1x    ��

�����������������������������������������������������������������������ͼ�

�                                                                        �

�                  >>>>> PHASE #3: DISK MODIFICATION <<<<<               �

�                                                                        �

�During the next phase of the In-Place Upgrade, the disks are modified.  �

�The v2.1x or v2.2 file system is overwritten by the v3.1x file system.  �

�After you press 'Y', you cannot reconstruct the v2.1x or v2.2 file      �

�system.                                                                 �

�                                                                        �

�Proceed with the Disk Modification Phase? y                             �

�                                                                        �

�Modifying Disk #0                                                       �

�Moving disk blocks to their new locations.  Percent Moved: 100%         �

�Mirroring each volume's DET and FAT tables . . . COMPLETE               �

��������������������������������������������������������������������������

6. At the Disk Modification screen, type "Y" to continue with the

upgrade or type "N" to quit.

NOTE: If you have multiple disks, the Disk Analysis and Disk

Modification process repeats itself.

After all disks have been upgraded, volume SYS: is mounted and the

bindery is upgraded. The v2.1x bindery is merged into the newly

created v3.11 bindery.

The Bindery phase updates bindery objects, properties, and values

such as user accounts and passwords.

Figure 7-6

Bindery screen

������������������������������������������������������������������������Ŀ

�����������������������������������������������������������������������ͻ�

��     In-Place Upgrade from NetWare v2.1x and v2.2 to NetWare v3.1x    ��

�����������������������������������������������������������������������ͼ�

�                                                                        �

�                       >>>>> PHASE #4: BINDERY <<<<<                    �

�                                                                        �

�Mounting volume SYS                                                     �

�Initializing Transaction Tracking System                                �

�                                                                        �

�The In-Place Upgrade does not transfer user passwords to the v3.1x      �

�bindery.  You can choose to either leave users without passwords, or to �

�assign a random password to each user.  The SUPERVISOR will not be given�

�a random password.  Random passwords are recorded in the file NEW.PWD in�

�the SYS:SYSTEM directory.                                               �

�                                                                        �

�Assign random passwords to users? y                                     �

�                                                                        �

�Upgrading the bindery information . . . COMPLETE                        �

�                                                                        �

�Dismounting volume SYS                                                  �

��������������������������������������������������������������������������

7. (Conditional) In the process of assigning passwords, if you did not

use the [R+] or [R-] or [B] parameters when you started the upgrade,

you are prompted to assign random passwords.

See Step 8 for password options.

8. Choose one of the following password options:

---------------------------------------------------------------------

Password Option   Description

---------------------------------------------------------------------

Y (Yes)           Assign random passwords.  A random password for each

user who originally had a password is created by the

utility and stored in a file called NEW.PWD.

User SUPERVISOR is not assigned a password.

Only the network supervisor has access to this file.

Users cannot log in until the supervisor gives them

their password from this list.

N (No)            Assign no passwords.  Users are not prompted for a

password when they first log in.

However, after they have logged in once, and if their

accounts are set to require passwords, they are

prompted to type a new password.

---------------------------------------------------------------------

The Final Status screen displays how your disks and volumes were

affected by the upgrade. See Figure 7-7, "Final Status screen", for

an example.

Figure 7-7

Final Status screen

������������������������������������������������������������������������Ŀ

�����������������������������������������������������������������������ͻ�

��     In-Place Upgrade from NetWare v2.1x and v2.2 to NetWare v3.1x    ��

�����������������������������������������������������������������������ͼ�

�                                                                        �

�����������������������������������������������������������������������Ŀ�

�� *** The In-Place Upgrade to the v3.1x File System is now complete ***��

�����������������������������������������������������������������������Ĵ�

��  DISKS    VOLUMES        �  STATUS MESSAGES                          ��

�����������������������������������������������������������������������Ĵ�

��  Disk 0   SYS:           �  Disk successfully upgraded.              ��

��           VOL1:          �                                           ��

�����������������������������������������������������������������������Ĵ�

��  Optional DOS Partition Size:  5 MB                                  ��

�����������������������������������������������������������������������Ĵ�

��  Random Passwords Assigned?  YES  (stored in SYS:SYSTEM\NEW.PWD)     ��

������������������������������������������������������������������������ٳ

�                                                                        �

�<Press ESC to terminate or any key to continue>                         �

��������������������������������������������������������������������������

The NetWare v2.1x file system has been upgraded to a v3.11 file

system.

9. Press any key to receive further instructions and then return to the

console prompt (:).

Mount Volumes

-------------

Procedure

---------

1. Mount volume SYS: and other volumes by typing

MOUNT ALL <Enter>

2. Temporarily prevent users from logging in to the server by typing

DISABLE LOGIN <Enter>

This prevents users from logging in as SUPERVISOR.

To allow users to log in to the server when the upgrade is

completed, type

ENABLE LOGIN <Enter>

Load LAN Drivers

----------------

Procedure

---------

1. Insert the System_2 diskette in drive A:.

2. Load the appropriate LAN driver(s) by typing

LOAD A:LAN_driver <Enter>

See Table 7-2 for a list of Novell LAN drivers.

The default frame type for NetWare 4.0 is Ethernet 802.2. To user

the Ethernet 802.3 frame type, used by default in NetWare v3.11 and

earlier, add "frame=Ethernet_802.3" when loading the LAN driver.

For example, type

LOAD A:NE2000 frame=Ethernet_802.3 <Enter>

3. Bind LAN drivers to the server's registered protocol.

For example, to bind IPX to the NE2000 LAN driver, type

BIND IPX TO NE2000

Enter the network number for your cabling system. To bind the

Ethernet 802.3 frame type to the LAN_driver, add

"frame=Ethernet_802.3" when binding the protocol.

For example, type

BIND IPX to NE2000 frame=Ethernet_802.3 <Enter>

Copy SYSTEM and PUBLIC Files

----------------------------

Procedure

---------

1. Inser the System_1 diskette in drive A:.

2. Load INSTALL.NLM by typing

LOAD A:INSTALL <Enter>

3. Choose "System Options" and press <Enter>.

4. From the "Available System Options" menu, choose "Copy System and

Public Files" and press <Enter>.

5. Insert diskettes as prompted.

Create a STARTUP.NCF File

-------------------------

A STARTUP.NCF file contains commands to load the disk drivers and name

space support for your file server.

This file is executed after SERVER.EXE and is stored on the disk you

boot from (hard disk or diskette).

Once this file loads the disk drivers and name space support, and

mounts volume SYS:, it turns control over to AUTOEXEC.NCF to complete

the boot process.

Procedure

---------

1. Insert the writable System_1 diskette into drive A:.

2. Select "Create STARTUP.NCF File" from the "Available System Options"

menu.

3. Enter the proper drive letter on the "Path For STARTUP.NCF File"

screen.

Use <Backspace> to make sure the drive is set to

o  C: to boot from hard disk.

o  A: to boot from drive A:. (This file can be copied to drive C:

later.)

4. Press <Enter>.

A screen similar to the following appears:

Figure 7-8

A sample

STARTUP.NCF file

�����������������������������������������������������Ŀ

�LOAD ISADISK PORT=1F0 INT=E                          �

�LOAD DCB PORT=340 INT=B                              �

�LOAD DCB PORT=348 INT=C                              �

�LOAD MAC                                             �

�������������������������������������������������������

The operating system reads the information you previously entered

for disk drivers and name space support.

5. From the following table, determine your system configuration and

decide which additional commands you would like added to the

STARTUP.NCF file.

---------------------------------------------------------------------

To                Add these or other commands

---------------------------------------------------------------------

Load name spaces  LOAD MAC.NAM

for Macintosh,    LOAD OS2.NAM

OS/2, UNIX, or    LOAD NFS.NAM

FTAM              LOAD FTAM.NAM

(These commands must precede the command to mount the

volume that stores the files using the name space.)

Set server        (You can add the following eight commands to the

parameters        STARTUP.NCF file only. You can add other SET commands

to the STARTUP.NCF and to the AUTOEXEC.NCF file.)

SET Maximum Physical Receive Packet Size

SET Cache Buffer Size

SET Reserved Buffers Below 16 Meg

SET Maximum Subdirectory Tree Depth

SET Concurrent Remirror Requests

SET Auto TTS Backout Flag

SET Minimum Packet Receive Buffers

(For more on these and other SET parameters, see

"SET" in Utilities Reference, or type "SET" at the

server console.)

Pause after each  PAUSE

command

---------------------------------------------------------------------

6. Type one command per line.

7. Press <Esc> to save the file.

Create an AUTOEXEC.NCF File

---------------------------

An AUTOEXEC.NCF file contains commands to complete the boot process

after SERVER.EXE and STARTUP.NCF executes.

Because AUTOEXEC.NCF is saved in and runs from SYS:SYSTEM, place most

boot commands (except the disk driver and name space support commands)

in AUTOEXEC.NCF.

Commands you can use in AUTOEXEC.NCF do the following:

o  Name the file server.

o  Assign the internal network number for the file server.

o  Load LAN drivers for the file server and assign the network number.

o  Bind LAN drivers to the file server's registered protocol.

o  Load other modules you want loaded when the file server boots. (See

"LOAD" in System Administration.)

o  Execute other console commands (such as TRACK ON or VOLUMES) during

the boot process.

o  Set other parameters for the file server. (See "SET" in System

Administration.)

o  Mount volumes.

Procedure

---------

1. From the "Available System Options" menu, select "Create

AUTOEXEC.NCF File".

The operating system gathers information you have previously entered

at the console prompt for this file.

A screen similar to the following appears, showing the commands that

are automatically placed in AUTOEXEC.NCF.

Figure 7-9

A sample

AUTOEXEC.NCF file

�����������������������������������������������������Ŀ

�file server name SPEEDY                              �

�ipx internal net 1993ABCD                            �

�LOAD NE1000 port=300 int=3                           �

�bind IPX to NE1000 net=DEADBEAF                      �

�mount all                                            �

�������������������������������������������������������

NOTE: When using the 802.3 frame type, rather than the 802.2

default, the screen shown above displays the frame type number.

If you load multiple LAN drivers, your AUTOEXEC.NCF file is

different. See "INSTALL" in System Administration.

2. (Optional) From the table below, decide which loadable modules to

load with AUTOEXEC.NCF.

---------------------------------------------------------------------

To                                     Add these modules

---------------------------------------------------------------------

Create disk partitions, create         LOAD INSTALL

volumes, format a hard disk, etc.

Lock the file server console, view     LOAD MONITOR

network operation information.

Load the print server on the file      LOAD PSERVER finance_print

server and establish print services.

Correct volume problems or remove      LOAD VREPAIR

name space entries from File

Allocation and Directory tables.

---------------------------------------------------------------------

To delete or modify commands, backspace to erase the command.

3. (Optional) From your system configuration, determine additional

loadable modules to include in AUTOEXEC.NCF.

Additional module types are shown in the table below.

---------------------------------------------------------------------

Module Type                      Module Name

---------------------------------------------------------------------

Equipment                        UPS.NLM

Remote Console                   REMOTE.NLM

RSPX.NLM

Remote Boot for token ring       TOKENRPL.NLM

Remote boot for IBM and Western  ETHERRPL.NLM

Digital on Ethernet

Third-party loadable modules     (See third-party documentation)

---------------------------------------------------------------------

NOTE: If you use an uninterruptible power supply, see "UPS" in

System Administration for troubleshooting tips.

Some loadable modules need other loadable modules to function. If

the prerequisite module is not loaded first, the operating system

will look at your default drive and then at SYS:SYSTEM and

automatically load the necessary module.

4. From the table below, decide if you want any of the following

console commands to execute with the AUTOEXEC.NCF file.

---------------------------------------------------------------------

To                      Add these console commands

---------------------------------------------------------------------

Implement security      SECURE CONSOLE

measures

Display the processor   SPEED

speed

---------------------------------------------------------------------

5. Decide from the table below, additional commands you would like

added to AUTOEXEC.NCF.

---------------------------------------------------------------------

To                      Add these or other commands

---------------------------------------------------------------------

Mount Volumes.          MOUNT VOL1 (or other volume name)

MOUNT ALL

Note: Volume SYS: is mounted automatically when

its corresponding disk driver is loaded during

the STARTUP.NCF file's execution.

Bind protocols other    BIND IP to...

than IPX to the LAN

drivers.

Set server parameters.  SET commands (see "SET" in Utilities

Reference.)

Pause after each        PAUSE

command.

---------------------------------------------------------------------

6. (Optional) Allow unencrypted passwords.

Use the table below to help you decide whether to use unencrypted

passwords on a network using encrypted passwords.

---------------------------------------------------------------------

For                                 Use

---------------------------------------------------------------------

A new installation with one file    Encrypted passwords.

server on the network.

Upgrading from NetWare 2.x          Encrypted passwords temporarily.

operating system.

A netWare v3.11 file server         Encrypted passwords permanently.

running on the same network

as NetWare v2.11 or earlier.

A NetWare v3.11 file server         Encrypted passwords. Use SET in

running on the same network         AUTOEXEC.NCF until utilities on

as NetWare v2.11 or v2.15.          other file servers are upgraded.

Copy NetWare v3.11 utilities to all

NetWare v2.12 or v2.15 file servers

to allow all servers to use

encrypted passwords. (See "Appendix

A" of Utilities Reference.)

---------------------------------------------------------------------

Type the following command at the console prompt to allow

unencrypted passwords:

SET ALLOW UNENCRYPTED PASSWORDS=ON <Enter>

For more information on AUTOEXEC.NCF files or on the commands listed

above, see System Administration. See also "Boot Files" in Concepts.

7. Press <Esc> to save the file.

8. Press <Esc> twice to exit INSTALL.

Format the DOS Partition

------------------------

Procedure

---------

1. At the console prompt (:), down the server by typing

DOWN <Enter>

2. Insert the bootable INSTALL diskette in drive A:.

3. Return to the DOS prompt by typing

EXIT <Enter>

4. From the downed server, create a DOS primary partition and make it

the active partition by typing

FDISK <Enter>

NOTE: The two utilities used to partition and format the DOS

partition are found on the bootable INSTALL diskette. These DR DOS

files, FDISK.COM and FORMAT.COM, also ship with NetWare v3.11.

FDISK is a DOS utility that creates or deletes DOS partitions on

your hard disk. It can also select which partition is active. The

DOS partition will be placed on the first few cylinders of the hard

disk. The server will reboot DOS as part of the FDISK process.

5. Format the new DOS partition.

If you are using DR DOS, the format command is as follows:

FORMAT C:/s/x <Enter>

The /s parameter makes the partition bootable.

The /x parameter tells DR DOS that it is formatting a hard drive.

NOTE: If you are using DR DOS to format your hard drive, use

FORMAT.COM on the INSTALL diskette or a later version.

If you are not using DR DOS, type the format command as follows:

FORMAT C:/s <Enter>

6. Reboot the server to make sure that DOS boots from the hard disk.

Copy the New Boot Files to a Boot Directory

-------------------------------------------

Procedure

---------

1. Create a boot directory.

For example:

MD C:\SERVER.311

The boot files are copied to this directory on drive C:.

2. Copy the newly created STARTUP.NCF file from the System_1 diskette

to the boot directory by typing

COPY A:\STARTUP.NCF C:\Boot_directory <Enter>

3. Copy the v3.11 server boot files to the boot directory.

Copy the following files to the newly-created boot directory on

drive C: (or another directory on C: that you specify.)

o  SERVER.EXE (the NetWare operating system).

o  NUT.NLM (an interface utility needed for certain NetWare Loadable

Modules).

o  NWSNUT.NLM (an interface utility needed for certain NetWare

Loadable Modules).

o  Disk drivers (*.DSK).

o  Name space modules (*.NAM).

o  CLIB.NLM (C NLM Run-time Library).

o  Diagnostic NLMs (*.NLM).

o  INSTALL.NLM (the installation utility).

o  LAN drivers (*.LAN).

o  Message and help files.

The upgrade is complete.

4. From the boot directory, load the server operating system by typing

SERVER <Enter>

5. Complete the steps under "What to Do After the Upgrade" below.

What to Do After the Upgrade

----------------------------

Complete the following steps before allowing users to log in.

Procedure

---------

1. Change SUPERVISOR password.

Log in from a workstation as SUPERVISOR. Change the password using

either SETPASS or SYSCON.

2. Change user passwords.

If you chose to assign random passwords, print the NEW.PWD file and

distribute the password information to each user. Users should

change their passwords immediately, using either SETPASS or SYSCON.

3. Check applications to see if they run properly.

Some DOS applications don't work when installed on volumes that have

more than 32 MB of disk space. Some of these applications can be

made to work by doing the following:

o  Restrict the application's directory on the destination server

with DSPACE.

o  Make the directory path a fake root using the MAP command.

4. Check directory security.

Check directory security if you used any of the following items for

NetWare v2.1x security:

o  Maximum Rights Mask

o  Private attribute

5. Set any new directory and file attributes using FLAG or FILER.

6. Check directories for unnecessary NetWare files.

NetWare v2.1x

-------------

The following NetWare v2.1x files are not compatible with NetWare

v3.11 and should be deleted.

VAP files (*.VAP, *.HLD, *.VP?)

LARCHIVE.EXE

LRESTORE.EXE

MACBACK.EXE

NARCHIVE.EXE

NRESTORE.EXE

NET$BIND.SYS

NET$BVAL.SYS

NET$BIND.OLD

NET$BVAL.OLD

NET$ERR.SYS

NET$OS.EXE

NET$DOS.SYS

NET$MSG.SYS

NET$REC.DAT

HIDEFILE.EXE and SHOWFILE.EXE still work but have not been included

in NetWare v3.x.

These files were copied into SYS:SYSTEM. You can either delete them

(FLAG allows users to add and remove System and Hidden attributes

from files) or leave them.

NetWare v2.0a

-------------

The following NetWare v2.0a files are not compatible with NetWare

v3.11. They should be deleted.

ENDSPOOL.EXE

Q.EXE

QUEUE.EXE

SPOOL.EXE

The utilities listed above are replaced by ENDCAP, PCONSOLE, and

CAPTURE in NetWare v3.x. See Utilities Reference for more

information.

7. Modify the system and user login scripts.

Update as necessary any references to the server in the system and

user login scripts if you changed the server name.

Although user login scripts are upgraded, they are not modified and

server names are not changed to match your new environment. Use

SYSCON to modify the login scripts.

With NetWare 2.x, you could give users drive mappings to directories

in which they had not been granted rights. NetWare 3.x will not

allow users to have drive mappings to directories in which they have

not been assigned rights.

If you have set up your login scripts so that users have drive

mappings to directories in which they have no rights, these users

will receive the following message when they log in:

"Attempt to map drive to invalid path in MAP command."

Complete one of the following:

o  Delete the drive mapping from the login script.

o  Delete the drive mapping from the system login script and insert

the drive mapping in the user login scripts only when the user

has been granted rights to the directory.

o  Create a group, grant the group the trustee assignment, assign

the appropriate users to the group, and then use an IF. . . THEN

command in the system login script before the drive mapping (IF

member of "groupname", THEN map p:=volume:directory).

8. Create new boot diskettes for each workstation.

All users on the NetWare v3.x file server need to boot with the new

shell and IPX files before they log in.

8a.   If the server was renamed, change the file server's name in

the users' AUTOEXEC.BAT files to the new v3.11 file server

name.

8b.   If you have other NetWare v2.x file servers on your network,

upgrade the workstation files for those users even if you

aren't upgrading the file server.

The new workstation files shipped with NetWare v3.11 are

compatible with NetWare v2.x. All users on the internetwork

should use the latest version of the workstation files to

ensure that there are not conflicts.

9. Copy the NetWare v3.11 utilities to other file servers on the

network.

The NetWare v3.x public utilities will run on file servers running

NetWare v2.x. Complete one of the following:

o  If you have file servers running v2.0a, v2.10, or v2.11, do not

copy the NetWare 3.x public utilities to them. However, do copy

the NetWare v3.x LOGIN.EXE file to the SYS:LOGIN and SYS:PUBLIC

directories.

o  If you have NetWare v2.12 or v2.15 file servers on your network,

replace the NetWare public utilities with the NetWare v3.x public

utilities to allow the v2.12 and v2.15 file servers to work with

encrypted passwords.

To copy utilities, complete the following steps:

9a.   Log in to the NetWare v2.x file server as SUPERVISOR.

9b.   Flag the NetWare v2.x utilities Normal. In the SYS:LOGIN and

SYS:PUBLIC directories, type

FLAG *.* N <Enter>

9c.   Protect the system login script. In the SYS:PUBLIC directory,

type

FLAG NET$LOG.DAT SRO <Enter>

9d.   Map a drive to the SYS:PUBLIC directory on the NetWare v3.11

file server by typing

MAP drive:=fileserver/SYS:PUBLIC <Enter>

Replace drive with a drive letter not currently being used and

fileserver with the name of the NetWare v3.11 file server.

9e.   Enter your username and password for the NetWare v3.11 file

server.

9f.   Copy the NetWare v3.11 PUBLIC files to the NetWare v2.x file

server.

For example, if drive Q is mapped to SYS:PUBLIC on the NetWare

v3.11 server and drive F is mapped the SYS:PUBLIC on the

NetWare v2.x file server, change to drive F and type

NCOPY Q:*.* <Enter>

9g.   Flag the NetWare v3.11 utilities on the NetWare v2.x file

server Shareable and Read Only. Type

FLAG *.* SRO <Enter>

9h.   Repeat Steps 9e and 9f for the SYS:LOGIN directory.

IMPORTANT: Login scripts on the NetWare v3.11 file server do

not execute properly unless you copy the NetWare v3.11 login

programs to all NetWare LOGIN directories on your

internetwork.

9i.   When you have copied the NetWare v3.11 utilities to all

NetWare v2.1x file servers and you do not have any NetWare

v2.0a file servers, you can change the setting for unencrypted

passwords. At the console prompt of the NetWare v3.11 file

server, type

SET ALLOW UNENCRYPTED PASSWORDS=OFF <Enter>

If you still have NetWare v2.0a file servers on your

internetwork, the setting at the NetWare v3.11 file server for

unencrypted passwords must be ON.

10.   If your v2.x server had mirrored disks, remirror them using

INSTALL.NLM.

11.   Update as necessary upgraded print queues using PCONSOLE.

12.   Use NBACKUP to make a backup copy of all data on the NetWare

v3.11 server.

See Utilities Reference for NBACKUP instructions.

13.   Check user restrictions and accounting charge rates to make sure

your system is configured the way you want it.

14.   (Optional) If your network includes workstations that use an

operating system that supports long filenames, make sure you have

loaded the name space module, then use the ADD NAME SPACE command

to add name space to the volume. Type

ADD NAME SPACE name-support TO VOLUME volume_name <Enter>

For example, if you have a volume named MAC for Macintosh files,

type the following at the console prompt:

ADD NAME SPACE MACINTOSH TO VOLUME MAC <Enter>

Use this command once for every volume that needs to store files

with long names.

To ensure name space support each time the file server is brought

up, load the appropriate name space module in the STARTUP.NCF

file.

15.   (Optional) If you want to store Macintosh files and folders on a

NetWare v3.11 server, install NetWare for Macintosh (a separate

Novell product) on the v3.11 server.

This product provides support for Macintosh workstations that

connect to the v3.11 server.

16.   Allow users to log in to the NetWare v3.11 file server.

User can login if

o  The SYS: volume is mounted

o  The user exists in the bindery

o  The LAN driver is loaded and bound to a protocol

o  Login in enabled

To ensure that login is enabled, type

ENABLE LOGIN <Enter>

==========================================================================

************************************************************************

DISCLAIMER

Novell, Inc. makes no representations or warranties with respect to any

NetWare software, and specifically disclaims any express or implied

warranties of merchantability, title, or fitness for a particular

purpose.

Distribution of any NetWare software is forbidden without the express

written consent of Novell, Inc.  Further, Novell reserves the right to

discontinue distribution of any NetWare software.

Novell is not responsible for lost profits or revenue, loss of use of the

software, loss of data, costs of re-creating lost data, the cost of any

substitute equipment or program, or claims by any party other than you.

Novell strongly recommends a backup be made before any software is

installed.  Technical support for this software may be provided at the

discretion of Novell.

************************************************************************