STANDARD MICROSYSTEMS CORPORATION

Novell Netware 2.15 or 2.2 Server with Appletalk Phase II

For Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All right reserved.

This installation guide describes:

o  How to use EZStart or a manual procedure to copy the Novell

NetWare 2.15 and 2.2 server driver files from the SuperDisk

to your PC.

o  How to build the Novell NetWare 2.2 server.

o  How to install the NetWare 2.2 server.

o  Error messages.

Note: This driver supports all 8216 (Ultra) and 80xx (PLUS) Series

ISA and Micro Channel Ethernet adapters.

COPYING THE DRIVER FILES

EZStart Method:

From the Driver Installation screen, select 'Copy Files' to copy

all the files used by this driver from the SuperDisk to the

Destination Directory recommended by EZStart, or to a different

directory that you specify.

Manual Method:

To generate NetWare 2.2 software from a floppy drive, the

generation software prompts you to insert the driver diskette at

the appropriate time. NetWare installation procedures require

the driver files to reside in the root directory of a floppy disk

whose volume label is LAN_DRV_200. If you copy these files to

another floppy disk, be sure to set the volume label of the new

disk to LAN_DRV_200.

To generate the driver from a hard disk:

1. Copy the Netware 2.2 floppy disks to your PC's hard disk, as

described in the Netware 2.2 manual.

2. Create a directory named LAN_DRV_.200. This directory must be

at the same level as the LAN_DRV_.001 directory. Copy all .OBJ

files to the LAN_DRV_.200 directory.

3. If you have a server, copy the SMCNW286.LAN file to the SYSTEM-1

file. If you have a router, copy this file to the ROUTEGEN

directory.

4. If you have a server, run INSTALL. If you have a router,

run ROUTEGEN. For more information, refer to the NetWare 2.2

Installation manual.

BUILDING THE NetWare 2.2 SERVER

To build the NetWare 2.2 server:

1. Determine the LAN adapter configuration. For information on

RAM, Base I/O and IRQ, see the supplied SYSMAP.DOC file.

2. If you are running Appletalk on your server and want to use

these server drivers with Appletalk Phase II, download the

NetWare for Macintosh VAPs to the server's hard disk during the

installation process. This is accomplished by the INSTALL

program, which asks if you want to load the Macintosh VAPs.

Answer "Yes" to load them.

Follow the instructions in the NetWare for Macintosh Installation

and Maintenance manual in order to initialize these VAPs and to

bring up your server using Appletalk Phase II.

3. Run the Novell server generation program, INSTALL. Follow the

instructions in the Netware Installation manual.

4. Select the SMC Ethernet Adapter Novell 286 Server driver.

5. Select the Software Configuration.

For all SMC ISA adapters: Select 'First Software Configured

Adapter' for the first software-configured LAN adapter. Select

'Second Software Configured Adapter' for the second software-

configured LAN adapter, and so on.

When one software-configured LAN adapter card is installed in

the server, select the default configuration 'First Software

Configured Adapter'. The server driver will read the specific

configuration information directly from the adapter.

If the server contains more than one software-configured

LAN adapter, select 'Second Software Configured Adapter'

for the second software configured LAN adapter, etc. You

can select up to four adapters per server driver.

First, Second, etc., are determined by the Base I/O

address of the adapter; for example, if there are two

adapters, one with Base I/O address 240 and another with

an address of 280, 240 will be the 'First Software

Configured Adapter'.

For Western Digital adapters, which are not software configurable:

Configuration entries must specify a base I/O, IRQ, and RAM

size that match those selected on the adapter. Ifnone of the

choices in the table matches the hardware configuration

desired, select the "Ethernet LAN Adapter Configurable by

Jumpers Utility" entry and run Novell's JUMPERS.EXE version

2.00 configuration program. This utility lets you configure

the LAN software to use any supported hardware configuration

parameters. For more information on JUMPERS.EXE, refer to the

documentation supplied with the software. The JUMPERS utility

is available from Novell or CompuServ.

When selecting RAM size, note that the RAM location for

jumper-configured adapters is set by the driver to the

location specified in the configuration selected.

There are configuration selections designating RAM sizes

of 8, 16, and 32 Kbytes of RAM. Adapters with 8 Kbytes

of RAM can be used with configuration selections that

specify 8, 16 or 32 Kbytes of RAM. Adapters with 32 Kbytes

of RAM must be used with 32 Kbyte RAM configuration

selections.

For Micro Channel adapters: Select 'First Micro Channel Adapter'

for the first LAN adapter, 'Second Micro Channel Adapter' for

the second LAN adapter, and so on. First, Second, etc., are

determined by the slot number of the adapter; for example, if

two adapters are in slots 2 and 4 of the computer, the adapter

in slot 2 is the 'First'.

When there is more than one adapter in a computer, each adapter

must be configured so that interrupts are not shared. Do NOT

use the automatic configuration option when initializing the

adapter from the Reference Diskette. Automatic configuration

assumes that interrupts are shared. Instead, initialize the

adapter by selecting the Set Configuration option from the

main menu. Set the interrupt vector to an interrupt that is

not being used by another adapter.

INSTALLING THE NetWare 2.2 SERVER

1. Follow the instructions in the Netware Installation manual

if building and installing a 2.2 server on a hard disk for the

first time.

2. To update a current 2.2 Server, answer "No" to the question

"Will this machine be the server" during the INSTALL process.

This will create a NET$OS.EXE file that may be loaded to the

\SYS\SYSTEM subdirectory of the server's hard disk via a

workstation.

3. Down the server, then reboot the server to run the new driver.

4. If installing an AppleTalk driver in a NetWare 2.2 server,

refer to the NetWare for Macintosh Installation Supplement.

Note: When installing a NetWare External Bridge to be used with

AppleTalk Phase 2, the minimum requirements for system memory

are 5 Mbytes.

ERROR MESSAGES

"Could not find an adapter at I/O base address XXX"

The driver was unable to find an SMC Ethernet LAN adapter

at the specified base I/O address. Check for adapter

conflicts and retry the installation. Check that the adapter

is seated properly in adapter slot.

"Could not find an adapter at slot X (Micro Channel only)"

The driver was unable to find an SMC Ethernet LAN adapter

at the specified slot number. Check for adapter conflicts

and retry the installation. Check that the adapter is seated

properly in adapter slot.

"Adapter hardware failure during initialization"

The adapter did not pass the initialization process.

Check the adapter for configuration conflicts.

"Adapter hardware failure detected (self-test)"

The adapter did not pass the initialization self-test

process due to a hardware problem. Check the adapter

for configuration conflicts.

"Cannot access adapter RAM properly"

The adapter could not read from or write to the adapter RAM.

Verify that another adapter does not share the same memory

space as the LAN adapter.

"Other system RAM already occupies adapter RAM"

The driver has detected that the adapter RAM conflicts with

other RAM in the system. Reconfigure the adapter to use

a different RAM address and retry the installation.

"Other system ROM already occupies adapter RAM"

The driver has detected that the adapter RAM conflicts with

ROM in the system. Reconfigure the adapter to use a

different RAM address and retry the installation.

"IRQ higher than 7 not valid in 8-bit slot"

The IRQ number specified cannot be used if the adapter

is inserted into an 8-bit slot. Select a different IRQ

number.

"Adapter unable to process a multicast address"

A node address was given on the command line or the board is

configured for a node address that is reserved for multicast

address. Broadcast and multicast addresses are not allowed

as valid node addresses. Use the node address over-ride

with caution as this can cause problems on your network.

"Configuration corrupted; run software setup"

The driver failed to initialize the adapter due to invalid

data in configuration EEROM. Run the software setup utility

and reconfigure the adapter.

"I/O base address chosen for driver not valid"

Reconfigure the adapter with another I/O base address,

then reinstall.

"I/O base address for driver out of range"

The selected I/O base address is not within the valid

range of I/O base addresses for this adapter. Select

another I/O base address and reinstall.

"IRQ higher than 7 not valid in 8-bit slot"

When using an 8-bit slot, the IRQ value selected must be

3 - 7. Change the IRQ value or move the adapter to a 16-bit

slot.

"IRQ chosen for driver not valid (out of range)"

The IRQ value read from the software-configurable adapter is

not one of the IRQs supported by the adapter.

"Adapter RAM base address on illegal boundary"

The RAM base value is invalid for this adapter. Reconfigure

the RAM base address and reinstall.

"RAM base address for driver out range"

The memory base address cannot be used for the configuration

or the installed adapter.

"RAM size chosen for driver out of range"

The memory size cannot be used for this configuration or

adapter.

"Invalid error message"

The error return code was not valid. Reinstall or run the

adapter diagnostics.

"IRQ chosen for driver does not match adapter configuration"

The driver configuration does not match the jumper

configuration on the adapter. Change the jumpers on the

adapter to match the RAM size configuration.

"RAM size chosen for driver does not match adapter

configuration"

The driver configuration does not match the RAM size

selected by jumpers on the adapter. Change the jumpers

on the adapter to match the RAM size configuration.

"Adapter hardware failure detected (self test)"

The adapter did not pass the initialization self-test

procedure due to a hardware problem. Check the adapter

for configuration conflicts.

"Cannot access adapter RAM properly"

The adapter could not read from or write to the adapter

RAM. Verify that another adapter does not share the same

memory space as the LAN adapter.

"Adapter RAM conflicts with other system RAM"

Adapter RAM must be reconfigured into another area of

the System Memory Map. Then reinstall.

"Adapter ROM conflicts with other system ROM"

Adapter ROM must be reconfigured into another area of

the System Memory Map. Then reinstall.

"Adapter hardware failure during initialization"

The adapter has not passed the initialization procedure.

Run the adapter diagnostics and check for proper

configuration.

"Configured RAM size invalid"

Configured RAM size does not match the physical RAM size.

EtherCard PLUS and SuperDisk are trademarks, and SMC and Standard

Microsystems are registered trademarks, of Standard Microsystems

Corporation. Other trademarks mentioned herein belong to their

respective companies.