IPXWATCH : A Protocol Analyzer for Novell Networks

--------------------------------------------------

1. Introduction

---------------

Ipxwatch is a protocol analyzer for Novell's IPX protocol.

It  was  developed  at the Indian Institue of Technology, Bombay.

It allows  for  identification  of  all  routers,  servers  (file

servers, print servers etc.) and nodes across an IPX internet. It

also provides limited diagnostic facilities.

It  runs  on  packet  drivers  (you  can  get  them  from

crynwr.com).   It  supports  only  class  1  packet drivers (i.e.

ethernet class drivers). It has  been  tested  on  the  following

packet  drivers: NE1000, NE2000, WD8003E, and ARCETHER.  IPXWATCH

can analyze the following protocols : IPX, SPX, NCP, SAP and RIP.

1.1 Copying

-----------

See COPYING.DOC  for details.

No Warranty: see  COPYING.DOC for details.

2. Installing IPXWATCH

----------------------

This section describes how to install IPXWATCH on your machine.

IPXWATCH can be run on any IBM PC or its compatible  running  MS-

DOS  ver 3.0 or greater. The PC should be attached to the network

and must have an ethernet class packet driver. By ethernet  class

we  mean  that  although the media may not be ethernet the packet

driver  should  be  able  to  fake  ethernet   packets   to   the

application.  A  good  example is the Arcether packet driver from

crynwr.com which runs on arcnet but fakes  ethernet  packets   to

the applications.

Usage

-----

IPXWATCH <a|b> <pkt_int_no>

Before running IPXWATCH the packet driver has to  be  loaded

(ipx  should  not  be loaded). IPXWATCH requires two command line

parameters.  The first one is the protocol. You  may  be  running

either  ethernet  or  arcnet.  On an ethernet network the minimum

packet size is 60 bytes.  Smaller packets are padded up.  This is

not  the case for arcnet.  The command line parameter a|b is used

to decide whether to pad smaller packets are not. However on  the

arcnet  network  you  can  still  use  the  specifies  that it is

ethernet.  The second parameter is the  packet  driver  interrupt

vector.

IPXWATCH displays the addresses of the nodes  in  hexadecimal

format.    However   for  diagnostic  purposes  the  node/network

addresses  can  be  specified  in  symbolic  form.   Create   two

configuration  files in the directory from which IPXWATCH is run.

The configuration files  are  node.ipx  and  net.ipx.  The  files

node.ipx  and  net.ipx contain the numeric and symbolic addresses

of the nodes and networks.  A sample node.ipx and  net.ipx  files

are given.

Examples

--------

a) For a network running IEEE 802.3, and a node having NE1000

card a sample batch file would look like

rem   example1.bat

rem   the -n switch of the packet driver converts packets of

rem   type 8137 to IEEE 802.3 and vice versa

ne1000 -n 0x60 3 0x300

ipxwatch b 0x60

b) For a network running  blue  book  ethernet,  and  a  node

having NE1000 card the batch file would look like

rem    example2.bat

ne1000 0x60 3 0x300

ipxwatch b 0x60

c) For a network running arcnet, and a node having an  arcnet

card the batch file would look like

rem    example3.bat

arcether 0x60 2 0x2e0 0xd000

ipxwatch a 0x60

4. Using IPXWATCH

-----------------

By   default IPXWATCH   catches  all   IPX  packets.  It  has   a

command   prompt. Typing  h at the command prompt gives a summary

of the  commands. The commands are explained below.

Command     Function

-------     --------

A           Match All Packets

This is the default command when ipxwatch is invoked.

This  catches  all  ipx packets on the network.  Note

that in arcnet it is not possible to  catch  all  the

packets.    Typing  A  at  the  command  prompt  puts

IPXWATCH in the default command mode.

B           Display packet type Statistics

This command displays the packet type count  and  the

byte  count  for  various packets (IPX, RIP, SAP, NCP

etc.)

C           Clear Screen

Clears the display window.

D           Match on Destination Address

This is filter which displays/logs only  the  packets

going  to a particular destination node.  To use this

type d at the command prompt and enter the 6 byte  or

12  digit  address in hex, do not precede the address

with the 0x symbol.

F           Close Log File

This command closes the previously opened log file.

H           This Help Screen

Displays a summary listing of all the commands

I           Node Information

To use this command  first  use  the  Select  Network

command (N).  This command displays the status of all

nodes on the selected network.  For nodes  which  are

active and have ipx loaded the status is displayed as

ACTIVE.  If a node goes down after IPXWATCH has  been

invoked  its status is displayed as DOWN and the time

at which it went down is also displayed.   For  nodes

which   have   their   address   specified   in   the

configuration file node.ipx, if the  node  has  never

come  up  since the invocation of IPXWATCH its status

is displayed as UNKNOWN. Also the type of the node is

displayed.  IPXWATCH can determine the following node

types : WORKSTATION, FILE SERVER, and ROUTER.

L           List Servers

Lists all the  known  servers  on  the  network.  The

server  name, type and the clock ticks are displayed.

A server type 0004 means it's a  file  server.   This

command   displays  all  known  File  Servers,  Print

Servers, Btrieve Servers and so on.  The clock  ticks

tell  the  maximum  number  of  seconds  before which

IPXWATCH should receive another  SAP  broadcast.   If

IPXWATCH doesn't receive any SAP broadcast within the

remaining clock ticks, the server  entry  is  removed

from the table and will be no longer displayed.

N          Select Network

Displays all known networks. To select  a  particular

network  for  diagnostics use the cursor keys to move

to  the  appropriate   network   and   press   enter.

Subsequently  use the Node Information command (I) to

find out about the nodes  on  the  selected  network.

Note  that IPXWATCH can monitor only one network at a

time.

O          Open Log File

This command logs data on to a file.   All  the  data

displayed  on the screen (only the packets) is logged

into a user-specified file. To close the log file use

the  Close  Log  file command (F).  Note that no data

compression is done, the log file may chew up  a  lot

of disk space.

P          Match on Destination Source Pair

This is a filter which displays/logs only the packets

going between a source-destination pair.  To use this

command type P at the command prompt and  then  enter

the  destination  address  (12  digits),  immediately

followed by the  source  address  (12  digits).   The

address should be entered in hex.

Q          Quit

Exit from IPXWATCH and return to DOS prompt.

R           Routing Information

This command displays the routing table maintained by

ipxwatch.   It  displays  the  network  address,  the

number of hops, and number  of  ticks  to  reach  the

network. The address of the router is also displayed.

For the current network the router address  is  shown

as 00000000.

S           Match on Source Address

This is a filter which displays/logs only the packets

coming  from a particular node. To use this type S at

the command prompt and enter the 6 byte or  12  digit

address  in  hex, do not precede the address with the

0x symbol.

T           Match on Packet Type (NCP/IPX/RIP/SAP/DIAG/NB)

This is a filter which displays/logs only the packets

of  a  selected  type.   To  use  this  type T at the

command   prompt   and   enter   the   packet    type

NCP/IPX/RIP/SAP/DIAG/NB.    Only   one  type  can  be

selected at a time.

IPXWATCH  periodically  sends  diagnostic  configuration  request

packets  to  any selected network.  All the nodes in the selected

network respond to this request with a  diagnostic  configuration

response   packet.    As  IPXWATCH  implements  Novell's  routing

information protocol, it maintains its own routing table. It  can

thus  learn  about all nodes even across routers.  However at any

point of time IPXWATCH  sends  diagnostic  configuration  request

packets only to the selected network.  In order to know about the

active nodes on another network it is necessary  to  select  that

network.   It  is  also  possible to have information such as the

nodes shell version, connection number, default file server etc.,

but  due  to  lack of information on SPX this was not implemented

(Hope somebody/Novell will provide us the details !!).  Similarly

the details of the NCP packets are also not displayed for lack of

information on the packet structure.

5.  Troubleshooting using IPXWATCH

----------------------------------

IPXWATCH  can be used to identify many problems in a network.

Login Process

Using IPXWATCH one can observe a node trying to get attached  and

hence  logging  in. If a node is not attached to any file server,

during the process of attach the  following  events  take  place.

Below each event a sample output of IPXWATCH is displayed.

i) The node sends a Get Nearest Server broadcast packet.

000000000003 FFFFFFFFFFFF  SAP  110  Get Nearest Server

ii) The servers/routers respond with a Give Nearest  Server

Packet

0000000000F1  000000000003  SAP  110  Give  Nearest  Server IIT/01/02

This packet can only be observed on ethernet networks.  Here  the

server/router   responds  with  the  server  name,  address  etc.

However if a server/router does not reply, the node  repeats  the

request. If the servers or routers do not reply after a number of

repeated requests the node backs out by giving the  message  FILE

SERVER COULD NOT BE FOUND.

iii)  After a server/router replies to the  node,  it  tries

to  find   the   route   to  the  server  by  making a request to

the network on which the server is located. It sends a  broadcast

packet.

000000000003 FFFFFFFFFFFF  RIP   54  Route  request 000000A3

iv) The server/router responds to this request. If there  is

no  response  the  node repeatedly broadcasts the request.  If no

router responds then the node backs out by giving a message  like

ROUTER OR GATEWAY NOT RESPONDING.

0000000000F1  000000000003  RIP   54  Route  Info  000000A3

01/02

After this the node sends NCP packets to the server and the

node gets attached.  However on arcnet only the broadcast packets

can be observed.

5.1     Booting of Diskless Nodes

---------------------------------

A diskless node boots from a file server. When a diskless

node  is  powered  on  it sends  NCP  broadcast  packets  on  the

net.  Then  it  follows the same  procedure  as  described  above

to get attached to a server.

If a server/router does not respond to the NCP  broadcast

the  node   repeats  the NCP broadcasts. If  no  server  responds

within  a  specified  time  the  node  backs  out  by  giving   a

message  FILE  SERVER   COULD NOT BE FOUND  or  YOUR  STATION  IS

NOT CONNECTED PROPERLY.

We  were able to locate a faulty diskless node  (it's  boot

prom  was  bad)  by  using IPXWATCH. The  node  was  sending  NCP

broadcasts   repeatedly  but  the  server  never  responded   and

the   node   was   unable   to   boot.   The   node   information

command can  be  used   to  identify  all  active  nodes   on   a

network.  The nodes respond with DIAG packets as shown below.

000000000003 000000000094 DIAG   52 01110 -> 01110

The last two fields (01110 -> 01110)  indicate  the  source

and  destination  socket  numbers.  The routing  information  and

the server  information  can  be  used to  monitor  the   routers

and servers.

6. Files

----------------

COPYING  DOC     12375 /* GNU's General Public Licence  */

DIAG     C       16880 /* Diagnostics */

IBUFF    C        1693 /* Buffer management */

IPX      C        3675 /* IPX protocol code */

IPXWATCH DOC     15432 /* This file you are reading */

IPXWATCH H        1957 /* IPX protocol data structures */

PARSE    C        3585 /* Parser for command prompt */

TIMER    C        4156 /* The timer handler routine */

DIAG     H        7365 /* The header files */

SAP      H         936 /* " " */

IPXRIP   H        1070 /* " " */

HELP     H        1030 /* " " */

YAPCBR   H        2337 /* " " */

IPXRIP   C        7736 /* IPX routing information protocol */

SAP      C        5688 /* SAP protocol */

NEW      C        5623 /* Packet driver interface */

SCREEN   C        4972 /* Screen management routines */

MISC     C        1276 /* Miscellaneous functions */

MAIN     C       14864 /* The main program */

SUP1     ASM      4794 /* Packet driver <-> assembly interface */

NODE     IPX       470 /* Sample Configuration file : node addresses */

NET      IPX       543 /* Sample Configuration file : net addresses */

IPXWATCH EXE     74563 /* The actual executable */

IPXWATCH PRJ     40608 /* TC++ prj file */

7. Acknowledgments

___________________

We would  like  to  thank  all  our  colleagues  especially

Gadekal  Gopal  Reddy  and  K  V  Ramani  for  their comments and

suggestions and code.

8.  Future Work ---------------

Currently  IPXWATCH  only  supports  a  limited  diagnostic

facility.   It  would  be  very  useful  if  all   the   features

available  in  ipx  diagnostic services  were  implemented.  This

was  not  implemented  in  this version as  details  of  the  SPX

protocol were not  available.  Since  ODI  drivers  are  used  by

a number of users it would be a useful if this was ported  on ODI

drivers. All that is required to be done is to change the  packet

driver   interface   in   sup1.asm  and  main.c    to    the  ODI

driver  interface.   The   data   logging facility   is   limited

to   the   packets,   the   routing/server information  table  is

not  captured  in  a   log   file.   Also   the  data   is    not

compressed   and  it  consumes a lot  of   disk space.  As we are

leaving the institute, we may  not  be  able  to  support/enhance

IPXWATCH. We would be glad if some one could make enhancements to

IPXWATCH.

Email address :

srinivas@cse.iitb.ernet.in                            and

vishwas@cse.iitb.ernet.in

Postal address :

V. Srinivas,         Department of Computer  Science  and

Engg,           Indian  Institute  of  Technolgy,          Powai,

Bombay, 400 076, India

Thanks and enjoy :-)