NodeID 1.04 is Copyright (c) 1992, 1993 by John L. Villalovos of

Seriously Sound.  This program and previous versions of this program are

freeware.  Future versions may become shareware.  This program may be freely

distributed with the restriction that this program may not be uploaded or

be on Compuserve Information Services (commonly known as Compuserve).

If you want you can send me email, money or a postcard.  I would appreciate it

but it is not required or assumed that you will.

NodeID          Get network address and place into environment variables.

Description:    Sets four environment variables based on your machines

configuration.  The four environment variables are as

follows.

VIDEO   This variable is set depending on the display adapter

installed in your computer.  It will be set to one

of the following.  Excuse any errors in translating the

acronyms.

MDA     Monochrome Display Adapter

CGA     Color Graphics Adapter

EGA     Enhanced Graphics Adapter

MCGA    Multi-Color Graphics Adapter

VGA     Video Graphics Adapter

NETNUM  This variable is set to an 8 byte value for the

network number.  The network number is the number

assigned to the section of network your station is

connected.  You may have a number of computers on

the same network number.  In a single file server

installation with only one NIC (Network Interface

Card) then all the stations will have the same

network number.

NODE1 & NODE2 compromise the 12 byte node address of the

workstation.  The reason they are split up is so that it is

easy to use the variables to create file names or

subdirectories.

NODE1   This variable is set to the first 4 bytes of the node

address of the workstation.  In a Arcnet installation

this will always be 0000 while in Ethernet and other

topologies this will vary by the card.  Usually cards

from the same manufacturer will have the same value

for all their cards.

NODE2   This variable is set to the last 8 bytes of the node

address of the workstation.  This is the variable that

will probably be used the most to uniquely identify a

workstation.

Command Line Commands:  /Q      Quiet output.  This will not display any

messages unless an error occurs during the

program.

/N      Will not display a warning message if NETX is

not loaded.  Use this option with /Q if you

want no output and you are only loading IPX.

Requirements:   This program requires at a minimum that IPX be loaded.

DOS 2.0+.

History of changes.

1.04

3-Nov-93

Converted to EXE instead of COM, no other changes.  Removed beta label.

Has been out for a while and no bug reports.

1.03 beta

4-Aug-93

Removed windowing.  Added /Q & /N options.  Actually put a little

documentation into the file.

1.02 beta

Added a window look to display.

1.01 beta

Added VIDEO environment variable.

1.00 beta

Initial version of NodeID.  Written entirely in assembly language using

the excellent Spontaneous Assembly library.  This version sets the

NETNUM, NODE1, & NODE2 environment variables.  Also tests for DOS 2.0

or above.