ROUTE.COM - NetWare Source Routing Driver for DOS.  All command line

parameters (for AUTOEXEC.BAT) are described below:

ROUTE U, BOARD=dd, CLEAR, DEF, GBR, HOPS=dd, MBR, NODES=dd, TIME=ss,

TRA=dd, TRO=dd, XTX=nn, LSB

?        Gives a help display of all ROUTE.COM syntax if ROUTE.COM has not

been loaded.  If ROUTE.COM has already been loaded, the source routing

table is dumped to the screen or can be redirected to a file.

U        Unload a previously installed Source Router (ROUTE.COM) from

memory.

BOARD=dd A DECIMAL board number as assigned by ODI.  This parameter is only

for use with ODI drivers.  Do not use this parameter if you are using

dedicated IPX Drivers.

CLEAR    CLEAR ALL Nodes from a previously loaded Source Router.  This

clears an existing source routing table of all entries.

DEF      Send DEFault (Unknown) Node Addresses ALL ROUTES Broadcast.  If

this parameter is not entered, SINGLE ROUTE Broadcast is assumed.

GBR      Send Broadcast (FFFF FFFF FFFF) Addresses ALL  ROUTES Broadcast.

If this parameter is not entered, SINGLE ROUTE Broadcast is assumed.

HOPS=dd  A DECIMAL number specifying the number of bridge hops to configure.

The DEFAULT is 07, and the MAXIMUM is 13; the minimum is 01.  This parameter

adjusts the size of the source routing table to allow up to 13 source routing

bridge hops per table entry.  If "0" is specified, the default of 07 is used.

To conserve memory, this number should be set as low as possible while still

allowing for the maximum number of bridge hops possible on the network.

MBR      Send MultiCast (C000 xxxx xxxx) Addresses ALL ROUTES Broadcast.  If

this parameter is not entered, SINGLE ROUTE Broadcast is assumed.

NODES=dd A DECIMAL number of NODE Addresses to support.  The DEFAULT is 16,

and the MAXIMUM is 1000.  If the value is less than 08, then 08 is used.

TIME=ss  A DECIMAL number specifying the number of seconds to wait before a

known route is timed out.  The  DEFAULT is 10, and the MAXIMUM is 3640.  On

receives, a timed out route will be updated.  On transmits, a timed out route

will not be used; instead, the frame will be all-routes or single-routes

broadcast according to the "DEF" parameter.  The full range of ss = 0-3640.

If ss=0, time-outs are disabled and routes will NEVER be timed out.  ss=0 is

not recommended where alternate bridge routes have been incorporated into the

design of the network.

XTX=nn   A DECIMAL number specifying the number of extra transmits on a

route that has been timed-out by the TIME= setting.  This allows the route

to be timed-back-in before SR or AR broadcasts are sent out (per the "DEF"

parameter).  The default is XTX=1, and the full range is XTX = 0 - 255.

TRA=dd   A DECIMAL number specifying a THIS RING ALTERNATE count for

Broadcasts.  The Source Router will alternate between SOURCE ROUTED and

NON-SOURCE ROUTED broadcasts.  The DEFAULT is 00, and the MAXIMUM is 255.

Example: If TRA=2 is used, and the IPX Retry Count is 20, then the first two

retries would not be source routed.  If no response to these broadcasts is

received, the next two retries would be source routed.  Each pair of retries

alternates back and forth from non-source-routed to source routed until all

IPX Retries are exhausted.  Note:  The TRA value must be less than the IPX

retry count.

TRO=dd   A DECIMAL number specifying a THIS RING ONLY count for Broadcasts.

The DEFAULT is 00, and the MAXIMUM is 255.

LSB      Use LSB (Least Significant Bit) mode to evaluate MAC addresses.

MSB (Most Significant Bit) mode is the default.  This should match the mode

the LAN adapter is using.  Usually Token-Ring uses MSB mode and FDDI uses

LSB mode.

All of the above ROUTE.COM parameters are optional, are not case sensitive,

and may be entered in any order.  They may be used to SET the source router

(ROUTE.COM) that is being loaded, or CHANGE the configuration of a previously

loaded source router (ROUTE.COM).  All of these parameter defaults may be

changed by SRCONFIG.COM.

***************************************************************************

ROUTE.VP0 - Novell 286 OS Source Routing VAP.  Use SRCONFIG.COM to configure

the source routing VAP.  Each route table entry supports up to 13 source

routing bridge hops by default.  There is a maximum of 250 table entries.

NetWare console commands for ROUTE.VP0 appear below:

ROUTE     BOARD=n, CLEAR, DEF, GBR, MBR, REMOVE=xxxxxxxxxxxx, RSP=rr,

TIME=ss

BOARD=n  n is the LAN Driver Board: A, B, C, or D.  If NOT entered, BOARD=A

is ASSUMED.

CLEAR    CLEAR ALL Nodes from a previously loaded Source Router.

DEF      Send DEFault (Unknown) Node Addresses ALL ROUTES Broadcast.  If

this parameter is not entered, SINGLE ROUTE Broadcast is assumed.

GBR      Send Broadcast (FFFF FFFF FFFF) Addresses ALL ROUTES Broadcast.  If

this parameter is not entered, SINGLE ROUTE Broadcast is assumed.

MBR      Send MultiCast (C000 xxxx xxxx) Addresses ALL ROUTES Broadcast.  If

this parameter is not entered, SINGLE ROUTE Broadcast is assumed.

REMOVE=xxxx is A HEXADECIMAL NODE Address to REMOVE from a previously loaded

Source Router.  If less than 09 hexadecimal digits are entered, then

4000 xxxx xxxx is assumed.  Example: ROUTE REMOVE=10005A123456 will delete

node 10005A123456 from the source routing table maintained by ROUTE.VP0.

RSP=AR   Respond to Broadcast Requests through ALL ROUTES Broadcast.

RSP=SR   Respond to Broadcast Requests through SINGLE ROUTE Broadcast.

RSP=NR   Respond DIRECTLY to a Broadcast Request.  This is the DEFAULT.

TIME=ss  ss is the number of SECONDS that the Source Router should wait

before updating its Source Routing Table.  The DEFAULT is 03 seconds.

All parameters are separated by commas or blanks, are optional, are not case

sensitive, and may be entered in any order at the 286 console prompt.  All

parameter defaults may be changed by SRCONFIG.COM.

***************************************************************************

SRCONFIG.COM - Novell Source Routing Configuration Utility has been updated

to configure the new features in ROUTE.COM.  SRCONFIG.COM will change the

default parameters of ROUTE.VP0 and ROUTE.COM only.

***************************************************************************

ROUTE.NLM - Novell Source Routing NLM.  It works with NetWare 3.x and 4.x

file servers and routers and bridges.  It has been updated to comply with

the new SRT Bridging standard where up to 13 SRT bridge hops can take place

on a network.  The source routing table built and maintained by route.nlm is

now increased from 18 bytes to 30 bytes per table entry.  This is not

configurable. There is no limit to the number of route table entries that

can be maintained by ROUTE.NLM.  The number of table entries is limited

only by the amount of RAM installed in the NetWare 386 file server.

Valid parameters for ROUTE.NLM are the following:

LOAD     [d:]ROUTE BOARD=dd, NAME=yy, CLEAR, DEF, GBR, MBR,

REMOVE=xxxxxxxxxxxx, RSP=rr, TIME=ss, XTX=nn, LSB

BOARD=dd dd is a DECIMAL board number as assigned by the System.  If this

parameter is not entered, BOARD=1 is ASSUMED.

NAME=yy  Cyy is an alphanumeric name that was given to the lan board when

the lan driver was loaded.  The NAME= parameter allows the user to load

route by board name instead of BOARD=.

CLEAR    CLEAR ALL Nodes from a previously loaded Source Router.

DEF      Send DEFault (Unknown) Node Addresses ALL ROUTES Broadcast.  If

this parameter is not entered, SINGLE ROUTE Broadcast is assumed.

GBR      Send Broadcast (FFFF FFFF FFFF) Addresses ALL ROUTES Broadcast.  If

this parameter is not entered, SINGLE ROUTE Broadcast is assumed.

MBR      Send MultiCast (C000 xxxx xxxx) Addresses ALL ROUTE Broadcast.  If

this parameter is not entered, SINGLE ROUTE Broadcast is assumed.

REMOVE=xxxx is a HEXADECIMAL NODE Address to REMOVE from a previously loaded

Source Router.  If less than 09 hexadecimal digits are entered, then

4000 xxxx xxxx is assumed.

RSP=AR   Respond to Broadcast Requests through ALL ROUTES Broadcast.  This

is the DEFAULT for the RSP parameter.

RSP=SR   Respond to Broadcast Requests through SINGLE ROUTE Broadcast.

RSP=NR   Respond DIRECTLY to a Broadcast Request.

TIME=ss  A DECIMAL number specifying the number of seconds to wait before a

known route is timed out.  The  DEFAULT is 10, and the MAXIMUM is 65535.  On

receives, a timed out route will be updated.  On transmits, the timed out

route will not be used; however, the frame will be all-routes or

single-routes broadcast according to the "DEF" parameter. The full range of

ss = 0-65535.  If ss=0, time-outs are disabled and routes will NEVER be timed

out.  ss=0 is not recommended for where alternate bridge routes have been

incorporated into the design of the network.

XTX=xx   A DECIMAL number specifying the number of extra transmits on a

timed-out route (see TIME=ss parameter). This allows the route to be

timed-back-in before any single-routes or all-routes broadcasts are used

according to the "DEF" parameter.  The default is XTX=1, and the full range

is:   XTX = 0-255.

All ROUTE.NLM parameters are optional, are not case sensitive, and may be

entered in any order. They may be used to SET the Source Router that is being

loaded, or CHANGE the configuration of a previously loaded source router.

LSB      Use LSB (Least Significant Bit) mode to evaluate MAC addresses.

MSB (Most Significant Bit) mode is the default.  This should match the mode

the LAN adapter is using.  Usually Token-Ring uses MSB mode and FDDI uses

LSB mode.

***************************************************************************

ROUTE.SYS - NetWare for OS/2 Source Routing Support driver.  The syntax for

ROUTE.SYS must appear in the NET.CFG file.  The syntax is as follows:

protocol route

source route def gbr mbr nodes n board n xtx n

def      When def is specified, frames with an unknown destination node

address are sent "all routes."  If def is not specified, the default is

"single routes."

gbr      When gbr is specified, broadcast frames are sent "all routes."  If

gbr is not specified, then broadcast frames are sent "single routes," which

is the default.

mbr      When mbr is specified, multicast (or functional) frames are sent

"all routes."  If mbr is not specified, then multicast (functional) frames

are sent "single routes."

nodes n  n specifies the number of route table entries allowed in the route

table maintained by ROUTE.SYS.  n is a decimal number from 8 to 255.  The

default is 16.

board n  n is the logical board number of the board performing the source

routing.

xtx n    n is a DECIMAL number specifying the number of extra transmits

on a route that has been timed-out.  This allows the route to be

timed-back-in before SR or AR broadcasts are sent out (per the "def"

parameter).  The default is 2, and the range is 0 to 6.

-----------------------------------------------------------------

Any trademarks referenced in this document are the property of their

respective owners.  Consult your product manuals for complete trademark

information.

-----------------------------------------------------------------