NAME

bootpd - Internet Boot Protocol server NLM for the Novell

NetWare Operating System 3.11

SYNOPSIS

bootpd.nlm -i addr[/netmask] [-t timeout] [-d] [configfile]

DESCRIPTION

Bootpd implements an Internet Boot Protocol server as

defined in RFC951 and RFC1048. It is normally loaded

in the NetWare AUTOEXEC.NCF script at system startup.

OPTIONS

-i addr[/netmask | :host_bits]

Sets the IP address of the network adapter the server

can use for BOOTP replies. More than one adapter can be used

by using the -i option once for each adapter. If no netmask

is specified, BOOTPD uses the network class netmask as default.

The format for addr and netmask is standard INTERNET style e.g.

"aaa.bbb.ccc.ddd". BOOTPD can not reply to clients which reside

on a network not specified by the -i option. Instead of

setting a netmask, the number of host bits are specified

by using the :host_bits option, :4 will set a

subnet mask of 255.255.255.240, :6 will set 255.255.255.192.

-t timeout

If bootpd does not receive another boot request within

fifteen minutes of the last one it received, it will

exit to conserve system resources. The -t switch may

be used to specify a different timeout value in minutes

(e.g. -t20). A timeout value of zero means forever.

-d

Each instance of the -d switch increases the level of

debugging output. Since most of the error diagnostics

is given at level 2 it's advisable to start bootpd with

at least two -d options. The maximum level of debugging

is 4 e.g. "-d -d -d -d".

configfile

As default bootpd uses the configuration file

SYS:\ETC\BOOTPTAB. This can be overwritten by adding a

filename when using the NetWare load command.

FILE FORMAT

Upon startup, bootpd first reads its configuration file,

SYS:\ETC\BOOTPTAB and then begins listening for BOOTREQUEST

packets. The configuration file has a format in which

two-character case-sensitive tag symbols are used to represent

host parameters. These parameter declarations are separated

by colons (:). The general format is:

hostname:tg=value. . . :tg=value. . . :tg=value. . . .

where hostname is the actual name of a bootp client and tg

is a two-character tag symbol. Most tags must be followed

by an equals-sign and a value as above. Some may also

appear in a boolean form with no value (i.e. :tg:). The

currently recognized tags are:

bf   Bootfile

bs   Bootfile size in 512-octet blocks

cs   Cookie server address list

ds   Domain name server address list

gw   Gateway address list

ha   Host hardware address

hd   Bootfile home directory

hn   Send hostname

ht   Host hardware type (see Assigned Numbers RFC)

im   Impress server address list

ip   Host IP address

lg   Log server address list

lp   LPR server address list

ns   IEN-116 name server address list

rl   Resource location protocol server address list

sm   Host subnet mask

tc   Table continuation (points to similar "template" host entry)

to   Time offset in seconds from UTC

ts   Time server address list

vm   Vendor magic cookie selector

There is also a generic tag, Tn, where n is an RFC1048 ven-

dor field tag number. Thus it is possible to immediately

take advantage of future extensions to RFC1048 without being

forced to modify bootpd first. Generic data may be

represented as either a stream of hexadecimal numbers or as

a quoted string of ASCII characters. The length of the gen-

eric data is automatically determined and inserted into the

proper field(s) of the RFC1048-style bootp reply.

The following tags take a whitespace-separated list of IP

addresses: cs, ds, gw, im, lg, lp, ns, rl, and ts. The ip

and sm tags each take a single IP address. All IP addresses

are specified in standard Internet "dot" notation and may

use decimal, octal, or hexadecimal numbers (octal numbers

begin with 0, hexadecimal numbers begin with '0x' or '0X').

The ht tag specifies the hardware type code as either an

unsigned decimal, octal, or hexadecimal integer or one of

the following symbolic names: ethernet or ether for 10Mb

Ethernet, ethernet3 or ether3 for 3Mb experimental Ethernet,

ieee802, tr, or token-ring for IEEE 802 networks, pronet for

Proteon ProNET Token Ring, or chaos, arcnet, or ax.25 for

Chaos, ARCNET, and AX.25 Amateur Radio networks, respec-

tively. The ha tag takes a hardware address which must be

specified in hexadecimal; optional periods and/or a leading

'0x' may be included for readability. The ha tag must be

preceded by the ht tag (either explicitly or implicitly; see

tc below).

The hostname, home directory, and bootfile are ASCII strings

which may be optionally surrounded by double quotes (").

The client's request and the values of the hd and bf symbols

determine how the server fills in the bootfile field of the

bootp reply packet.

If the client specifies an absolute pathname and that file

exists on the server machine, that pathname is returned in

the reply packet. If the file cannot be found, the request

is discarded; no reply is sent. If the client specifies a

relative pathname, a full pathname is formed by prepending

the value of the hd tag and testing for existence of the

file. If the hd tag is not supplied in the configuration

file or if the resulting boot file cannot be found, then the

request is discarded.

Clients which specify null boot files will always elicit a

reply from the server. The exact reply will again depend

upon the hd and bf tags. If the bf tag gives an absolute

pathname and the file exists, that pathname is returned in

the reply packet. Otherwise, if the hd and bf tags together

specify an accessible file, that filename is returned in the

reply. If a complete filename cannot be determined or the

file does not exist, the reply will contain a zeroed-out

bootfile field.

In all these cases, existence of the file means that, in

addition to actually being present, the file must have its

public read access bit set, since this is required by

tftpd to permit the file transfer. Also, all filenames

are first tried as filename.hostname and then simply as

filename, thus providing for individual per-host bootfiles.

The time offset to may be either a signed decimal integer

specifying the client's time zone offset in seconds from

UTC, or the keyword auto which uses the server's time zone

offset. Specifying the to symbol as a boolean has the same

effect as specifying auto as its value.

The bootfile size bs may be either a decimal, octal, or hex-

adecimal integer specifying the size of the bootfile in

512-octet blocks, or the keyword auto which causes the

server to automatically calculate the bootfile size at each

request. As with the time offset, specifying the bs symbol

as a boolean has the same effect as specifying auto as its

value.

The vendor magic cookie selector (the vm tag) may take one

of the following keywords: auto (indicating that vendor

information is determined by the client's request), rfc1048

(which always forces an RFC1048-style reply), or cmu (which

always forces a CMU-style reply).

The hn tag is strictly a boolean tag; it does not take the

usual equals-sign and value. It's presence indicates that

the hostname should be sent to RFC1048 clients. Bootpd

attempts to send the entire hostname as it is specified in

the configuration file; if this will not fit into the reply

packet, the name is shortened to just the host field (up to

the first period, if present) and then tried. In no case is

an arbitrarily-truncated hostname sent (if nothing reason-

able will fit, nothing is sent).

Often, many host entries share common values for certain

tags (such as name servers, etc.). Rather than repeatedly

specifying these tags, a full specification can be listed

for one host entry and shared by others via the tc (table

continuation) mechanism. Often, the template entry is a

dummy host which doesn't actually exist and never sends

bootp requests. This feature is similar to the tc feature

of termcap for similar terminals. Note that bootpd

allows the tc tag symbol to appear anywhere in the host

entry, unlike termcap which requires it to be the last tag.

Information explicitly specified for a host always overrides

information implied by a tc tag symbol, regardless of its

location within the entry. The value of the tc tag may be

the hostname or IP address of any host entry previously

listed in the configuration file.

Sometimes it is necessary to delete a specific tag after it

has been inferred via tc. This can be done using the con-

struction tag@ which removes the effect of tag as in

termcap. For example, to completely undo an IEN-116 name

server specification, use ":ns@:" at an appropriate place in

the configuration entry. After removal with @, a tag is

eligible to be set again through the tc mechanism.

Blank lines and lines beginning with "#" are ignored in the

configuration file. Host entries are separated from one

another by newlines; a single host entry may be extended

over multiple lines if the lines end with a backslash (\).

It is also acceptable for lines to be longer than 80

characters. Tags may appear in any order, with the follow-

ing exceptions:  the hostname must be the very first field

in an entry, and the hardware type must precede the hardware

address.

An example SYS:\ETC\BOOTPTAB file follows:

# Sample bootptab file

default1:\

:hd=/usr/boot:bf=null:\

:ds=128.2.35.50 128.2.13.21:\

:ns=0x80020b4d 0x80020ffd:\

:ts=0x80020b4d 0x80020ffd:\

:sm=255.255.0.0:gw=0x8002fe24:\

:hn:vm=auto:to=-18000:\

:T37=0x12345927AD3BCF:T99="Special ASCII string":

carnegie:ht=6:ha=7FF8100000AF:ip=128.2.11.1:tc=default1:

baldwin:ht=1:ha=0800200159C3:ip=128.2.11.10:tc=default1:

wylie:ht=1:ha=00DD00CADF00:ip=128.2.11.100:tc=default1:

arnold:ht=1:ha=0800200102AD:ip=128.2.11.102:tc=default1:

bairdford:ht=1:ha=08002B02A2F9:ip=128.2.11.103:tc=default1:

bakerstown:ht=1:ha=08002B0287C8:ip=128.2.11.104:tc=default1:

# Special domain name server for next host

butlerjct:ht=1:ha=08002001560D:ip=128.2.11.108:\

ds=128.2.13.42:tc=default1:

gastonville:ht=6:ha=7FFF81000A47:ip=128.2.11.115:tc=default1:

hahntown:ht=6:ha=7FFF81000434:ip=128.2.11.117:tc=default1:

hickman:ht=6:ha=7FFF810001BA:ip=128.2.11.118:tc=default1:

lowber:ht=1:ha=00DD00CAF000:ip=128.2.11.121:tc=default1:

mtoliver:ht=1:ha=00DD00FE1600:ip=128.2.11.122:tc=default1:

NOTES

When using paths with Netware volume name like SYS:FOO take

care that the path is placed between quotes e.g. fn="SYS:FOO".

The colon is used as a field seperator in the configuration

file. Directories must be either specified using the / e.g.

fn="SYS:/FOO/BAR" or escaped twice when using the \ e.g.

fn="SYS:\\\\FOO\\\\BAR".

Bootpd looks in SYS:\ETC\SERVICES to find the port numbers it

should use. Two entries are extracted: bootps -- the bootp

server listening port, and bootpc -- the destination port

used to reply to clients. If the port numbers cannot be

determined this way, they are assumed to be 67 for the

server and 68 for the client.

Bootpd rereads its configuration file when it receives a bootp

request packet and detects that the file has been updated. Hosts

may be added, deleted or modified when the configuration file is

reread.

The bootp forwarding feature is not supported on the Netware OS

version.

Only the interface specified by the -i option is used for

bootp request and replies.

Bootp replies are broadcastet to the network.

Timezone GMT is assumed if nothing is  specified in the

configuration file.

INSTALLATION

1. Copy the file BOOTPD.NLM into your SYS:\SYSTEM directory.

2. Create a file SYS:\ETC\BOOTPTAB that includes your bootp

information.

3. If there is no file SYS:\ETC\SERVICES copy the template

from SYS:\ETC\SAMPLES\SERVICES to SYS:\ETC\SERVICES.

4. Verify that SYS:\ETC\SERVICES includes the following lines:

bootps          67/udp          bootp server

bootpc          68/udp          bootp client

5. Load the bootpd NLM. This can also be done by editing the

SYS:\SYSTEM\AUTOEXEC.NCF file. The following lines give

a sample AUTOEXEC.NCF:

file server name NW

ipx internal net DEADBEEF

load 3c509 port=360 name=en0 frame=ETHERNET_II

load exp16 port=300 int=a name=en1 frame=ETHERNET_II

load smc8100 port=280 name=tk0 frame=TOKEN-RING_SNAP

load tcpip

bind ipx to en0 net=23

bind ip  to en0 addr=192.109.23.195 mask=255.255.255.240

bind ipx to en1 net=24

bind ip  to en1 addr=192.109.24.195

bind ipx to tk0 net=25

bind ip  to tk0 addr=192.109.25.195

load bootpd -i 192.109.23.195/255.255.255.240 -i 192.109.24.195

-i 192.109.25.195 -t 0 -d -d -d -d

KNOWN PROBLEMS

Individual host entries must not exceed 1024 characters.

Netware INSTALL.NLM and EDIT.NLM cut of long lines which could appear

when using multiple -i options.

BOOTPD handels up to 16 network interfaces.

FILES

SYS:\ETC\BOOTPTAB          default file for bootp database

SYS:\ETC\SERVICES          service name database

COPYRIGHT

Copyright (c) 1992,93 Dirk Koeppen EDV-Beratungs GmbH

D-63073 Offenbach, Germany

CREDITS

The bootpd NLMS is based on the Carnegie Mellon bootpd version 2.1a.

Original CMU bootpd source is distributed with every copy of the

bootpd NLM without fee.

REVISION VERSION 1.50

- BOOTPD did not bind to the server port (bootps/udp) if the port was

specified in the SYS:\ETC\BOOTPTAB file.

- The daemon is now able to handle multiple network adapters and

subnet masks.

SEE ALSO

RFC951, RFC1048            the BOOTP protocol

RFC1084                    assigned Numbers