�����������������������������������������������������������Ŀ

�  HOW TO INSTALL YOUR LCS-8834P ON WINDOWS FOR WORKGROUPS  �

�������������������������������������������������������������

This subdirectory includes an OEMSETUP.INF file for WINDOWS FOR

WORKGROUPS .Using the OEMSETUP.INF file, the user can select ShineNet series

LAN cards on Windows dialog menus.

Installation procedures:

1. Proceed to install Windows for Workgroups.  If you already have an

installed Windows for Workgroups, open the Control Panel Window to

change the network settings.

2. When Windows asks for the type of network adapter, it will show a

list of all network adapters supported by Windows.  You must select

'Unlisted or Updated Network Adapter'.

3. Windows will ask for an OEMSETUP.INF file. Since the ShineNet OEMSETUP.INF

file is included in the subdirectory WFW311 of this distributition driver

diskette, insert the driver diskette in drive A:

and input the full path name where the OEMSETUP.INF file is located.

( i.e. A:\WFW311 )

4. You will see a Windows dialog menu listing the ShineNet network adapters

in the OEMSETUP.INF file. Select the network adapter that you will be using.

Now, you should select the LCS-8834P.

5. Windows will copy the NDIS driver file and update the Windows network

system automatically. Or you can select the ODI driver ODIPKT.COM to

work with NetWare system.

6. The network settings are not nessary, because the LCS-8834P is

auto-detected.

At this point, you can finish the installion or continue the process

according to your preferences.

7. After installation is completely finished, restart your system for the

updates to take effect.