NDIS Driver Installation Guide for IBM LAN Server

=======================================================

Introduction

============

This document describes the procedures to install NDIS driver on IBM

OS/2 LAN Server for AE-200PNP Ethernet Adapter.

These drivers conform to the Network Driver Interface Specification

(NDIS 2.01) defined by Microsoft/3Com.

IBM LAN Server network OS does not support Plug&Play feature and if

your computer does not support PnP function also,  you must set the

network card in "Jumperless" mode through our PNPSETUP.EXE program;

Otherwise, it can not work correctly.

Installation Procedures

=======================

A. Installing driver procedures for IBM LAN Server 2.0 or 3.0 :

Before you start with the installation  process,  make sure that

OS/2 LAN Server  is properly installed.  Similarly, your adapter

should also be properly installed in your server.

1. Copy PNPND.OS2 and RTL8019.NIF files from your Driver diskette

(\LANSRVR\OS2) to C:\IBMCOM\MACS.

2. Click the OS/2 LAN Service icon.

3. Select "Installation  and  configuration".  The IBM logo should

appear onscreen.

4. Click <OK>.

5. Select <Advance> and press <Enter>.

6. Select "Install or Configuration this Workstation."

7. Select server type, adding a server or domain controller. The

"install or remove" selections should appear.

8. Select "Configure a component."

10. Click the "Configure" button.

11. Select "Configure workstation."

12. Select "AE-200PNP Ethernet Adapter" when prompted to identify

you Network Adapter. Then click the <Add> button.

13. Select IBM NETBIOS from the list and click the <Add> button.

14. Select <OK> to confirm all selections.

15. Follow screen instructions to complete the process, selecting

<OK> to accept default selections.

16. Installation is complete now. Reset your PC by turning it off

and on.

B. Installing driver procedures for IBM OS/2 LAN Server 4.0 :

Before you start with the installation  process,  make sure that

OS/2 LAN Server  is properly installed.  Similarly, your adapter

should also be properly installed in your server.

1. In OS/2 window, Click the "MPTS" icon.

2. The screen will display the IBM Log, press "OK".

3. Select "Install" button.

4. Insert the driver disk, specify the driver path A:\LANSRVR\OS2

and press <Enter>.

5. When complete above step to load  PNPND.OS2  into OS/2 system,

then configure your system.

6. The same step as step 3, select "Configure" button.

7. The screen will display "Configure" dialog box and select "LAN

adapters and protocols" then press "configure" button.

8. The screen will display "LAPS Configuration" dialog box, move

highlight to  "AE-200PNP Ethernet Adapter" item in "Network

Adapter" window, and select "NetBios" protocol. Press "OK".

9. Installation is complete now.  Reset your PC by turning it off

and on.