Driver Installation Guide for LANtastic 5.0

=================================================

Introduction

============

This document describes the procedures to setup LANtastic 5.0 driver

for AE-200PNP Ethernet Adapter.

LANtastic v5.0 network OS does not support Plug&Play feature and if

your computer does not support PnP function also,  you must set the

network card in "Jumperless" mode through our PNPSETUP.EXE program;

Otherwise, it can not work correctly.

Installation Procedures

=======================

1. Before you start with the installation  process, make sure that

the LANtastic operation system (server/workstation) is properly

installed.  Similarly,  your  adapter  should  also be properly

installed in your workstation.

2. Run LANtastic "install" program, and move highlight to  "Network

Adapter Install" item, then ENTER and selec "Novell NE2000 16-bit

ISA bus or Compatible" item.

3. When complete installation, then copy "PNPLAN5.EXE" from driver

diskette A:\LANTASTI\5.X to your LANtastic directory C:\LANTASTI.

4. Modify STARTNET.BAT file as sample configuration files.

--- Sample STARTNET.BAT file ---

A. STARTNET (for LANtastic server)

:

@ECHO OFF

SET LAN_DIR=C:\LANTASTI.NET

SET LAN_CFG=C:\LANTASTI

PATH C:\LANTASTI;%PATH%

SHARE /L:200

REM NEX000  IRQ=3 IOBASE=300 16-BIT VERBOSE  ---> Mask this line

PNPLAN5 VERBOSE 			     ---> Added for adapter

AILANBIO

REDIR NAME LOGINS=3

SERVER

NET LOGIN/WAIT \\SERVERNAME NAME

NET LPT TIMEOUT 10

NET POSTBOX

:

B. STARTNET.BAT  (for LANtastic workstation)

:

@ECHO OFF

SET LAN_CFG=C:\LANTASTI

PATH C:\LANTASTI;%PATH%

REM NEX000  IRQ=3 IOBASE=300 16-BIT VERBOSE  ---> Mask this line

PNPLAN5 VERBOSE 			     ---> Added for adapter

AILANBIO

REDIR WORKSTATION_NAME LOGINS=2

NET LPT TIMEOUT 10

NET POSTBOX

: