Driver Installation Guide for LANtastic 6.0

=================================================

Introduction

============

This document describes the procedures to setup LANtastic 6.0 driver

for AE-200PNP Ethernet Adapter.

LANtastic v6.0 network OS does not support Plug&Play feature and if

your computer does not support PnP function also,  you must set the

network card in "Jumperless" mode through our PNPSETUP.EXE program;

Otherwise, it can not work correctly.

Installation Procedures

=======================

You can install LANtastic on computers that are running DOS only, or on

computers that are using both DOS and Windows. LANtastic v6.0 includes

both DOS- and Windows-cased interfaces.

If you run the command "INSTALL" under Windows, both the DOS and Windows

interface will be installed on the computer. If Windows isn't running,

only the DOS interface will be installed.

1. Run your existing DOS (or Windows) system as you usually do.

2. Place the first LANtastic diskette in floppy drive A: or B:

3. Run the LANtastic INSTALL programe.

a. If you are installing from the DOS prompt, type:

A: INSTALL <Enter> or B: INSTALL <Enter>

b. If you are starting from Windows. Form Windows Program Manager

or the File Manager windows, select the File menu and choose

RUN. In the RUN window, type:

A: INSTALL <Enter> or B: INSTALL <Enter>

then choose OK.

4. Proceed throught the installation screens, following the instruction

on each. Choose OK or press Enter to continue from each screen.

5. In the Select Network Adapter window select NE2000 adapter as your

Ethernet Hardware Adapter.

6. Follow all the operations as you are prompted on the screen.

7. When the installation is complete, INSATLL automatic reboot your

computer.