Driver installation Guide for NetWare DOS ODI client

==========================================================

Introduction

============

This document describes the procedures to install  Netware DOS ODI

driver for AE-200PNP Ethernet Adapter.

NetWare DOS ODI client network OS does not support Plug&Play feature

and if your computer doesn't support PnP function also, you must set

the LAN card in  "Jumperless" mode through our PNPSETUP.EXE program;

Otherwise, it can not work correctly.

Installation Procedures

=======================

1. Copy the following files to a user specified directory:

copy a:\NETWARE\DOSODI\*.*

2. If using VLM, also copy the following files:

copy a:\NETWARE\DOSODI\VLM\*.*

3. Edit the NET.CFG file if necessary to select protocol and

media type options.

4. To connect to a server, use the following loading sequence:

LSL

PNPODI

IPXODI

NETX     or VLM

LOGIN