Driver installation Guide for NetWare OS2 ODI client

==========================================================

Introduction

============

This document describes the procedures to install  Netware OS2 ODI

driver for AE-200PNP Ethernet Adapter.

NetWare OS2 ODI client network OS does not support Plug&Play feature

and if your computer doesn't support PnP function also, you must set

the LAN card in  "Jumperless" mode through our PNPSETUP.EXE program;

Otherwise, it can not work correctly.

Installation Procedures

=======================

Before you start with the installation procedure, make sure that the

adapter is properly installed and configured.  Similarly, Make sure

that your NetWare Requester for OS/2 is properly installed.

Please refer to "Novell NetWare Requester for OS/2" documentation for

a detailed	Requester and Utilities installation guide.

1. Copy the PNPOS2.SYS  file into the NETWARE subdirectory of your

hard disk from the Driver Diskette A:\NETWARE\OS2ODI.

2. Add or replace the PNPOS2.SYS in the CONFIG.SYS file.

3. Edit the NET.CFG file that you copied over. Edit the file per your

requirements (see examples above).  If there are two frame types

listed, whichever frame type is listed first, that is the one that

the driver will load.  See above for examples.

4. Reboot your system to get service of network.

--- Sample	CONFIG.SYS file ---

DEVICE=C:\NETWARE\LSL.SYS	    ; OS/2 Requester supported

DEVICE=C:\NETWARE\DDAEMON.SYS	    ; OS/2 Requester supported

DEVICE=C:\NETWARE\PNPOS2.SYS	    ; Manufacturer supported

DEVICE=C:\NETWARE\IPX.SYS	    ; OS/2 Requester supported

--- Sample NET.CFG file ---

LINK DRIVER PNPOS2

FRAME ETHERNET_802.3

FRAME ETHERNET_802.2