Packet Driver Installation Guide for FTP PC/TCP

=====================================================

Introduction

============

This document describes the procedure to install Packet Driver for

AE-200PNP Ethernet Adapter.

The PNPPD.COM program is complies with the FTP PC/TCP Packet driver

Specifications.  This packet driver allow you to use The  AE-200PNP

Ethernet Adapter with FTP PC/TCP software.

FTP PC/TCP network OS doesn't support Plug&Play feature and if your

computer does not support  PnP function also,  you must set the LAN

card in "Jumperless" mode through our PNPSETUP program;  Otherwise,

it can not work correctly.

Installation Procedures

=======================

1. Execute the PNPPD.COM program by follows command line :

> PNPPD [options] [<SOFTWARE_INT>]

OPTIONS:

--------

-i - Force driver to report itself as 802.3 instead of Ethernet II.

-d - Delayed initialization.  Used for diskless booting

-n - NetWare conversion.  Converts 802.3 packets into 8137 packets

-w - Windows hack, obsoleted by winpkt

-p - Promiscuous mode disable

-h - Help

-u - Uninstall

SOFTWARE_INT:

-------------

Interrupt vector number, which can be decimal, hexadecimal or

octal format.

if:

decimal format -- from 96 to 128

hexadecimal ----- from 0x60 to 0x80

octal format ---- from 0140 to 0200

Note: user can use default settings by only type "PNPPD". The

program will automatic search the Ethernet Adapter,

and uses its internal setting.

2. Execute the \ETHDRV.EXE device program, which is provided by FTP

with PC/TCP software package together.

Now, your AE-200PNP Ethernet Adapter is work with FTP PC/TCP.

Note: Except the FTP software's PC/TCP package, most of the DOS

version's TCP/IP software also can be run with packet driver.

Please refer to the software package installation for packet

driver install procedure.