Driver Installation Guide for SCO UNIX 5.x

================================================

Introduction

============

This document describes the procedures to install the SCO UNIX driver

for AE-200PNP Ethernet Adapter.

SCO UNIX OS does not support Plug&Play feature and	if your computer

doesn't support PnP function also,  you must set the network card in

"Jumperless" mode through our PNPSETUP.EXE program; Otherwise, it can

not work correctly.

The following files will be used in this installation:

Files	      Contents

-----------    -----------------------------------------

Driver.o       MDI Driver Program

setup	      Transfer dos format to unix format setup program

Master	      Used by netconfig

System	      Used by netconfig

Node	      Used by netconfig

Space.c	      Used by netconfig

space.h	      Used by netconfig

lkcfg	      Used by netconfig

AOF/r2e	      Used by netconfig

Installation Procedures

=======================

Before you start with the installation  process,  make sure that the

SCO UNIX system is properly installed. Similarly,your adapter should

also be properly installed in your machine.

1. Login as root user. (On maintenance mode)

2. Installation Diskette in DOS format :

Put SCO UNIX Driver diskette to floppy A: and use doscp command

to copy the file "setup" for installing SCO UNIX PnP Driver into

UNIX's directories as following :

# cd /

# doscp A:/sco_unix/5.x/setup /setup

!! if you don't have the directory ,namely, "a:/sco_unix/5.x"

in the disk, you should modify contents of the setup file

in the floppy. Please check contents of the setup file.

# chmod +x /setup

# ./setup

3. Use netconfig utility, Avaiable options :

# netconfig

4. To add adapters, select "Add New Lan Adapter" from "Hardware".

5. Add Protocol to select SCO TCP/IP.

6. Enter the internet Address of this interface.

for example : 192.9.100.1

7. Enter the netmask for this interface (default 255.255.255.0).

8. Enter the boardcast address for this interface.

default : xxx.xxx.xxx.255			   press ENTER

9. Do you want to relink to kernel now ?		   TYPE 'y'

10. Do you want this kernel to boot by default (y/n)    TYPE 'y'

11. Do you want the kernel environment rebuilt (y/n)    TYPE 'y'

12. Reboot unix

# shutdown -y -g0