Driver Installation Guide for Windows For Workgroups 3.11

===============================================================

Introduction

============

This document describes the procedures to install Microsoft Windows

for Workgroups 3.11 driver for AE-200PNP Ethernet Adapter.

MS Windows for Workgroups 3.11 OS doesn't support Plug&Play feature

and if your computer does not support PnP function also,  you must

set the LAN card in "Jumperless" mode through our PNPSETUP program;

Otherwise, it can not work correctly.

Installation Procedures

=======================

1. Turn your computer off, then Install the AE-200PNP Ethernet Adapter

to your computer.

2. After the adapter installation, turn your computer on.

3. Start your Windows for Workgroups 3.11.

4. After Windows Start OK, Select "Network" icon and then Select

"Network  Setup" icon.

5. Click on the "Drivers..." button.

6. If you are replacing a previous LAN adapter select the Remove

button to delete that adapter

7. Click on the "Add Adapter" button.

8. Select "Unlisted or Updated Network Adapter" item bar on the Adapter

list.

9. Insert the driver diskette into drive A: or B: and enter A:\WFW311

or B:\WFW311.

10. "AE-200PNP Ethernet Adapter" appears in the list and select it, then

Select "OK" bar.

11. "AE-200PNP Ethernet Adapter [NDIS3/NDIS2]" bar appears. It indicates

the NDIS driver has been copied into your Windows system.

12. Select "Close" bar and then Select "OK" bar.

13. Windows prompts you to insert several WFW system diskettes.

14. When message "Windows Setup has modified your SYSTEM.INI and

PROTOCOL.INI...	  ....." appears, select "OK" bar.

15. Select "Restart Computer" bar.

16. Now, the Windows for Workgroup 3.11 is ready for your Ethernet

Adapter.