Mar. 1995

SCO UNIX V.3 (TCP/IP V1.2X) Driver

Installation

for INFOTEL IN-220P SERIES Ethernet Card

==========================================

* Installation for multi-cards *

This installation is for IN-220P SERIES multi-cards driver of SCO UNIX

(Runtime v3.2.4x) with TCP/IP v.1.2 .

1). It is allowed to install 4 NICs. The following is as an example:

IN-220PT *4 or

IN-220PCT *4

2). Support gateway function. ( routing function )

3). Statistic function is added when the following command is executed:

# netatat -i

Name   Mtu   Network   Address   Ipkts  Ierrs Opkts   Oerrs Collis

d10e0  1500  192.9.200 unix2     423     0     423     0     0

d22e0  1500  15        unix3     423     0     423     0     0

d40e0  1500  170.23    unix1     15322   0     552     0     0

d20e0  1500  168.8     unix      556     0     890     0     0

lo0    2048  170.23    localhost 180     0     180     0     0

*********************

* Install Procedure *

*********************

The following files will be used in this installation:

Files          Contents

-----------    -----------------------------------------

driver.o       IN-220P SERIES Driver Program

setupin        IN-220P SERIES software setup program

d22e           Used by netconfig

info           Used by netconfig

init           Used by netconfig

master         Used by netconfig

node           Used by netconfig

reconf         Used by netconfig

remove         Used by netconfig

space.c        Used by netconfig

system         Used by netconfig

0.  In DOS environment prepared Diskette for setup22x :

Insert INFOTEL IN-220P SERIES Series Driver Programs in A: drive

Insert a blank diskette in B: drive ( for INFOTEL SCO UNIX driver)

Copy INFOTEL SCO UNIX Driver into root directory of blank diskette.

C> copy A:\scounix\*.*  B:\

1. Login as root user. (On maintenance mode)

2. Install TCP/IP software through the custom utility.

# custom

Select Install

Select A New Product

Select entire to install all of the packages

3.  Installation Diskette should be in DOS format.

Put INFOTEL SCO UNIX Driver diskette to floppy A: and use doscp

command to copy INFOTEL SCO UNIX Driver into UNIX's directories.

# cd /

# doscp A:/setup /setup

# chmod +x /setup

# ./setup

4. #netconfig

Available options:

1) Add a chain

2) Remove a chain

3) Reconfigure an element in a chain

4) Quit                                          Select 1. Add a chain

5. Select top level of chain to Add or q to quit:       1. sco_tcp

(SCO TCP/IP for UNIX)

6. Select next level of chain to Add or q to quit:      Select INFOTEL d22e

Ethernet Driver unite 0

7. Add chain sco_tcp -> d22e (y/n)                      type 'y'

***************************************************

*Note:                                            *

*   Add chain sco_tcp  -> d22e0 unit 0 (y/n)      *

*                     :                           *

*   Add chain sco_tcp  -> d22e3 unit 3 (y/n)      *

*   It depends on card number.                    *

***************************************************

.Enter IRQ (3,4,5,9,10,11,15) [5] or 'q' to quit     value

.Enter I/O base address (300..360) or 'q' to quit    value

8. Enter the internet Address of this interface         n.n.n.n

9. Enter the netmask for this interface                 (default 255.255.255.0)

10. Does the interface use a broadcast address of all 1's (y/n)  default : y

11. Enter the boardcast address for this interface       default: n.n.n.n

12. Are these values correct ? (y/n)                    y

13. XX Pseudo ttys are currently configured, do you want to:

1) Add Pseudo ttys

2) Remove Pseudo ttys

Select an option or enter q to quit [q]          q

14.Available options:

1) Add a chain

2) Remove a chain

3) Reconfigure an element in a chain

q. Quit                                          Select option: q

15. Do you want to relink to kernel now ?               y

16. Do you want this kernel to boot by default (y/n)    y

17. Do you want the kernel environment rebuilt (y/n)    y

18. Reboot unix

# shutdown -y -g0